PURPOSE

This instruction describes how garment measurements are checked at EMS against the customer measurement chart, so that out-of-tolerance garments are found early and corrected.

SCOPE

This instruction applies to measurement checks of samples, first pieces, in-line production, finished garments after ironing and final inspection.

RESPONSIBILITIES

• Quality Controller: Measures the garments and records the results.

• Line QC Supervisor: Reviews the results and stops the line or informs production when deviations are found.

• Quality Manager: Ensures that the latest approved measurement chart is used.

INSTRUCTIONS

1. BEFORE STARTING

1.1. Use the latest approved measurement chart and the sealed sample for the style.

1.2. Use a calibrated, undamaged measuring tape in centimetres or inches as given in the chart.

1.3. Measure on a flat, clean table; garments that came from ironing should rest before measuring.

2. MEASUREMENT STEPS

2.1. Lay the garment flat without stretching; close buttons and zips and smooth out wrinkles by hand.

2.2. Measure each point exactly as described in the measurement chart (point of measure).

2.3. Measure critical points (body length, chest, waist, hip, sleeve length) on every checked garment.

2.4. Check per size and colour: at least 5 pieces at first pieces and in-line checks, or the quantity required by the customer.

2.5. Write the measured value and the difference (+/-) on the measurement report and mark values outside tolerance.

3. RULES / PROHIBITIONS

- Do not stretch or pull the garment while measuring.

- Do not use an old or unapproved measurement chart.

- Do not use a damaged or stretched measuring tape.

4. IN CASE OF PROBLEM

4.1. If a critical point is out of tolerance, inform the supervisor immediately; the cause (pattern, cutting, sewing, shrinkage) is found and corrected before production continues.

4.2. Out-of-tolerance garments are separated and labelled, and are not packed until a decision is made.

5. RELATED DOCUMENTS AND RECORDS

- Customer Measurement Chart and Measurement Report

- EMS.QA.PRO.02.12. Pre-Production Sample and Approval Procedure

- EMS.LB.PRO.09.02. Calibration of Measuring Equipment Procedure

6. FEEDBACK AND MONITORING

The Quality Manager reviews measurement defects monthly and this instruction every year, and revises it when necessary.

الغرض

تصف هذه التعليمات كيفية فحص مقاسات الملابس في EMS مقارنة بجدول مقاسات العميل، بحيث تُكتشف الملابس الخارجة عن التفاوت المسموح به مبكرًا وتُصحح.

النطاق

تُطبق هذه التعليمات على فحص مقاسات العينات والقطع الأولى والإنتاج أثناء التشغيل والملابس الجاهزة بعد الكي والفحص النهائي.

المسؤوليات

• مراقب الجودة: يقيس الملابس ويسجل النتائج.

• مشرف الجودة على الخط: يراجع النتائج ويوقف الخط أو يبلغ الإنتاج عند اكتشاف انحرافات.

• مدير الجودة: يضمن استخدام أحدث جدول مقاسات معتمد.

التعليمات

1. قبل البدء

1.1. استخدم أحدث جدول مقاسات معتمد والعينة المختومة للموديل.

1.2. استخدم شريط قياس معايرًا وسليمًا بالسنتيمتر أو البوصة حسب ما ورد في الجدول.

1.3. قِس على طاولة مستوية ونظيفة؛ ويجب ترك الملابس القادمة من الكي لترتاح قبل القياس.

2. خطوات القياس

2.1. افرد القطعة دون شدها؛ أغلق الأزرار والسحابات وأزل التجاعيد باليد.

2.2. قِس كل نقطة تمامًا كما هو موضح في جدول المقاسات (نقطة القياس).

2.3. قِس النقاط الحرجة (طول الجسم والصدر والخصر والورك وطول الكم) في كل قطعة يتم فحصها.

2.4. افحص لكل مقاس ولون: 5 قطع على الأقل عند فحص القطع الأولى وأثناء الإنتاج، أو الكمية التي يطلبها العميل.

2.5. اكتب القيمة المقاسة والفرق (+/-) في تقرير المقاسات، وحدد القيم الخارجة عن التفاوت المسموح به.

3. القواعد / المحظورات

- لا تشد القطعة أو تسحبها أثناء القياس.

- لا تستخدم جدول مقاسات قديمًا أو غير معتمد.

- لا تستخدم شريط قياس تالفًا أو ممدودًا.

4. في حالة وجود مشكلة

4.1. إذا كانت نقطة حرجة خارج التفاوت المسموح به، أبلغ المشرف فورًا؛ ويُحدد السبب (الباترون أو القص أو الخياطة أو الانكماش) ويُصحح قبل استمرار الإنتاج.

4.2. تُفصل الملابس الخارجة عن التفاوت المسموح به وتوسم، ولا تُعبأ حتى يُتخذ قرار بشأنها.

5. الوثائق والسجلات ذات الصلة

- جدول مقاسات العميل وتقرير المقاسات

- EMS.QA.PRO.02.12. إجراء عينة ما قبل الإنتاج واعتمادها

- EMS.LB.PRO.09.02. إجراء معايرة أجهزة القياس

6. التغذية الراجعة والمتابعة

يراجع مدير الجودة عيوب المقاسات شهريًا وهذه التعليمات سنويًا، ويعدلها عند الحاجة.

ONAYLAYANLAR:

| Strateji ve Süreç Geliştirme Müdürü BAHAR YILDIRIM |  | İnsan Kaynakları Grup Müdürü ALİ KAAN GENÇ |  | Kalite Güvence Müdürü ABDULLAH BAYDOĞAN |  | EMS Genel Müdürü FATİH SALBARS |
| --- | --- | --- | --- | --- | --- | --- |