- PURPOSE

The purpose of this procedure is to establish a structured system for the identification, segregation, handling, storage, transportation, recycling, disposal and documentations of all type of waste generated within the facility.

This procedure ensures the following:

- Prevention of environmental pollution and minimization of environmental impacts.

- Compliance with applicable Egyptian environmental legislation, including the Egypyian Law (4/ 1994) and its amendments, as well as the (WMRA) Waste Management Regulation Law 202/2020.

- Compliance with relevant international environmental programs and frameworks, including:

- Zero Discharge of Hazardous Chemicals Foundation (ZDHC).

- Higg Facility Environmental Module (Higg FEM).

- ISO 14001.

- SCOPE

This procedure applies to all types of waste ( Hazard & Non Hazard) generated from production and operational activities within the facility, including but not limited to:

- Spinning operations

- Knitting and weaving processes

- Dyeing and finishing operations

- Cutting operations

- Printing and embroidery

- Sewing operations

- Quality control activities

- Packaging and shipping operations

  - The procedure also applies to supporting areas, including:

- Chemical storage areas

- Maintenance workshops

- Warehouses

- Laboratories

- Effluent Treatment Plant (ETP)

  - This procedure applies to all employees, contractors, and personnel involved in waste management activities within the facility.

- DEFINITIONS

- Waste
Any material that is discarded or intended for disposal as a result of industrial or operational activities.

- Hazardous Waste
Waste that poses a potential risk to human health or the environment due to its chemical, toxic, flammable, or reactive characteristics.

- Non-Hazardous Waste
Waste that does not pose significant risk to human health or the environment when properly managed.

- Waste Segregation
The process of separating waste according to type or characteristics to facilitate safe handling, recycling, or disposal.

- Roles and Responsibilities

| Position | Responsibilities |
| --- | --- |
| SUSTAINABILITY MANAGER | Overseeing the implementation of the facility’s waste management program, ensuring compliance with environmental regulations and standards, maintaining waste records and environmental reports and coordinating with licensed waste management contractors and regulatory authorities |
| Department Managers | Ensuring proper waste segregation practices within their departments, monitoring waste handling practices and reporting any environmental incidents or non-compliance |
| Waste Management Personnel | Collecting and transporting waste within the facility, properly labeling waste containers and maintaining cleanliness and organization within waste storage areas. |
| Maintenance Department | Managing used oils and lubricants, maintaining waste storage areas and related equipment and supporting safe waste management practices. |
| Procurement Department | Identifying and contracting licensed waste management companies for waste transportation and disposal. |
| Legal Affairs Department | Reviewing and approving contracts with licensed waste management companies. |
| Administrative Affairs Department | Monitoring waste segregation and all internal waste transportation activities, coordinating with licensed waste transportation companies the times and how is the transportations |
| Logistics Department | Managing customs clearance procedures for waste materials requiring special permits and Maintaining documentation related to waste traceability and transport. |
| Warehouse Manager | Appointing and assigning qualified Persons to manage waste storage facilities |
| Warehouse responsible | Conducting alla documents and records for in and out quantity of wastes and follow up the segregation and classification of wastes on the warehouse |

      - Waste type and classification

| Waste Category | Type / Case | Classification | Typical Examples |
| --- | --- | --- | --- |
| Chemical Waste | Solid | Hazardous | Used Dyeing, solvents, expired chemicals, contaminated containers, any other materials contaminated with chemicals and any sharp equipment using in production processes |
| Sludge | Solid | Hazardous / Non-hazardous (based on analysis) | ETP sludge |
| General Waste | Solid | Non-hazardous | Paper, food waste, packaging, rubber, plastic |
| Textile Waste | Solid | Non-hazardous | Fabric scraps, cutting off-cuts |
| All sharp equipment | Solid | Hazardous | All needles and cutting equipments |
| Electronic Waste | Solid | Hazardous | Damaged electronic devices, fluorescent lamps |
| Used Oil & Filters | Solid | Hazardous | Lubrication oil, machinery filters |
| Batteries | Solid | Hazardous | Used or damaged batteries, forklift batteries |
| Domestic water | liquid | Non-hazardous | Disposal human biological waste |
| Industrial wastewater before treatment | liquid | Hazardous | All liquid produce from production processing |

- WASTE SEGREGATION

Waste shall be segregated at the source to prevent contamination and facilitate recycling.

- Segregation Principles

- Separate hazardous and non-hazardous waste.

- Use color-coded containers where applicable or clearly label all waste containers with  poly ethylene colored bag for each type of waste.

- Avoid mixing incompatible waste types.

- Hazardous waste must be placed in sealed, leak-proof, and compatible containers.

- All containers must display:

  - Waste name.

  - Hazard symbols (where applicable).

- Example Waste Segregation System

| Waste Type | Container Label |
| --- | --- |
| Textile waste | Textile waste bin |
| Paper & cardboard | Paper recycling bin |
| Plastic waste | Plastic recycling bin |
| Chemical waste | Hazardous waste container |
| Used oil | Used oil container |
| Broken needles and sharp objects waste | Hazardous waste container |
| Lint waste | Lint waste container |
| Hazard Medical waste | Hazardous medical waste container |
| Canteen and restaurant waste | Non-hazardous waste |

- temporary storage

- Designated waste storage areas shall be:

  - Clearly identified

  - Well-ventilated

  - Secured and access-controlled

- Hazardous waste storage areas shall include secondary containment (bunding).

- Waste shall not exceed 90 days of on-site storage unless officially authorized.

- Incompatible waste shall be stored separately.

- Waste collections and internal transporting

Waste shall be collected and transported within the facility according to the following procedures:

- Use designated carts or containers for waste transportation.

- Follow designated waste transport routes within the facility.

- Maintain proper housekeeping during waste transport activities.

- Appropriate PPE

- Each transfer shall be recorded in an Internal Waste Transfer Log.

- Any leakage or spillage shall be managed according to the Chemical Spill Response Procedure.

- RECYCLING AND WASTE RECOVERY

The facility is committed to minimizing waste generation through recycling initiatives and partnerships with specialized recycling companies.

Examples include:

- Recycling textile scraps

- Recycling cardboard and paper packaging

- Recycling plastic materials

All recycling contractors must be licensed and approved by the Egyptian Environmental Affairs Agency and the Waste Management Regulatory Authority.

- HAZARDOUS WASTE DISPOSAL

Hazardous waste must be disposed of through licensed waste management contractors in accordance with regulatory requirements set by:

- Egyptian Waste Management Regulatory Authority

- Egyptian Environmental Affairs Agency

Procedures include:

- Maintaining hazardous waste transport manifests.

- Tracking waste quantities.

- Documenting final disposal through an official waste tracking document.

- ETP SLUDGE MANAGEMENT

Sludge generated from the Effluent Treatment Plant (ETP) must be managed as follows:

- Dewater sludge prior to storage.

- Store sludge in designated containers.

- Transport sludge to authorized disposal facilities.

- WASTEWATER MANAGEMENT

Wastewater generated from production and operational activities shall be properly collected, treated, monitored, and controlled to prevent environmental contamination and ensure compliance with applicable environmental standards.

All wastewater streams shall be managed through the facility’s Effluent Treatment Plant (ETP) prior to discharge or reuse.

13.1 Sources of Wastewater

Wastewater may originate from:

- Dyeing house and washing processes

- Fabric finishing operations

- Printing processes

- Chemical preparation and equipment cleaning

- Utility systems such as boilers and cooling systems

- Domestic sanitary wastewater

All wastewater streams must be directed to the centralized wastewater collection system connected to ETP.

13.2 Effluent Treatment Plant (ETP) Operation

The Effluent Treatment Plant shall be operated according to approved operational procedures to ensure effective removal of pollutants prior to discharge or reuse.

Typical treatment stages include:

- Equalization Tank

- pH Neutralization

- Coagulation and Flocculation

- Primary Sedimentation

- Biological Treatment

- Filtration and Final Polishing

The plant must be operated and monitored by qualified personnel.

13.3 Wastewater Monitoring and Testing

Wastewater quality shall be monitored regularly to ensure compliance with environmental standards.

| Parameter | Description |
| --- | --- |
| pH | Acidity or alkalinity level |
| COD | Chemical Oxygen Demand |
| BOD | Biological Oxygen Demand |
| TSS | Total Suspended Solids |
| Color | Dye concentration |

Monitoring shall be conducted through:

- The facility’s internal laboratory.

- Accredited external laboratories when required.

- Treated Water Reuse

Where technically feasible, treated wastewater may be reused within the facility to improve water efficiency and reduce freshwater consumption.

Reuse applications may include:

- Certain production processes

- Cleaning operations.

- Cooling systems.

- Landscape irrigation.

Notes: in case of the treatment plant (ETP) is not operational, the contaminated water will be transferred by an accredited company to a specialized treatment company that is also accredited by the Egyptian Environmental Affairs Agency (EEAA)and the Waste Management Regulatory Authority (WMRA).

- Documentation and records

| Record | Responsible | Retention / Frequency |
| --- | --- | --- |
| Waste Disposal Record | Storekeeper / EHS | Per disposal |
| Waste Inventory Log | EHS / Compliance | Monthly |
| Waste Disposal Log | EHS / Compliance | Per dispatch |
| Waste Manifest & Certificate | Transporter / EHS | Per dispatch |
| Internal Audit Report | Compliance Officer | Quarterly |

- TRAINING AND AWARENESS

Employees shall receive training on:

- Waste segregation procedures

- Hazardous waste handling

- Spill response procedures

- Environmental awareness

- Training shall be conducted:

- During employee induction

- At least annually thereafter.

- Emergency response

- Any spill or leakage shall be handled according to the Chemical Spill Response SOP.

- Spill kits and emergency PPE shall be available at all hazardous waste locations.

- The EHS Department must be informed immediately.

- Monitoring and audit

- Monthly inspections of waste storage areas shall be conducted.

- Periodic internal audits shall verify waste tracking and compliance.

- Waste data shall be reported through:

  - Higg FEM Assessments

  - ZDHC Performance Reporting

- Compliance References

  - ZDHC Wastewater Guidelines – Version 2.1

  - Higg FEM 4.0 – Section 5.3

  - National Environmental Laws & EPR Requirements

  - ISO 14001: Environmental Management Systems

    - الغرض

الغرض من هذا الإجراء هو إنشاء نظام منظم لتحديد وفصل والتعامل مع والتخزين والنقل وإعادة التدوير والتخلص والتوثيق لجميع أنواع المخلفات الناتجة داخل المنشأة.

تضمن هذه العملية ما يلي:

- الوقاية من التلوث البيئي وتقليل الآثار البيئية.

- الامتثال للتشريعات البيئية المصرية المعمول بها، بما في ذلك قانون إجبيا (4/1994) وتعديلاته، بالإضافة إلى قانون تنظيم إدارة المخلفات المصري 2020/202 (WMRA).

- الامتثال للبرامج والأطر البيئية الدولية ذات الصلة، بما في ذلك:

- مؤسسة عدم تصريف المواد الكيميائية الخطرة (ZDHC).

- وحدة البيئة في منشأة هيغ (Higg FEM).

- ISO 14001.

    - النطاق

ينطبق هذا الإجراء على جميع أنواع المخلفات (الخطرة والغير خطرة) الناتجة عن أنشطة الإنتاج والتشغيل داخل المنشأة، بما في ذلك على سبيل المثال لا الحصر:

- عمليات الغزل

- عمليات الحياكة والنسيج

- عمليات الصبغ والتشطيب

- عمليات القطع

- الطباعة والتطريز

- عمليات الطباعة

- عمليات الخياطة

- أنشطة مراقبة الجودة

- عمليات التغليف والشحن

  - تنطبق الإجراء أيضا على المجالات الداعمة، بما في ذلك:

- مناطق تخزين المواد الكيميائية

- ورش الصيانة

- المستودعات

- المختبرات

- محطة معالجة الصرف الصحي (ETP)

  - ينطبق هذا الإجراء على جميع الموظفين والمقاولين والموظفين المشاركين في أنشطة إدارة المخلفات داخل المنشأة.

    - التعريفات

- المخلفات
أي مواد يتم التخلص منها أو التخلص منها نتيجة الأنشطة الصناعية أو التشغيلية.

- مخلفات خطرة
تشكل خطرا محتملا على صحة الإنسان أو البيئة بسبب خصائصها الكيميائية، أو السامة أو القابلة للاشتعال أو التفاعلية.

- مخلفات غير خطرة
لا تشكل خطرا كبيرا على صحة الإنسان أو البيئة عند إدارتها بشكل صحيح.

- فصل المخلفات
عملية فصل المخلفات حسب النوع أو الخصائص لتسهيل التعامل الآمن مع النفايات أو إعادة التدوير أو التخلص منها.

    - الأدوار والمسؤوليات

| الموقع | المسؤوليات |
| --- | --- |
| مدير EHS | الإشراف على تنفيذ برنامج إدارة النفايات في المنشأة، وضمان الامتثال للوائح والمعايير البيئية، والحفاظ على سجلات النفايات والتقارير البيئية، والتنسيق مع المقاولين المرخصين لإدارة النفايات والجهات التنظيمية |
| مديرو الأقسام | ضمان ممارسات فصل النفايات بشكل صحيح داخل أقسامها، مراقبة ممارسات مناولة النفايات، والإبلاغ عن أي حوادث بيئية أو عدم الامتثال |
| موظفو إدارة النفايات | جمع ونقل النفايات داخل المنشأة، ووضع علامات صحيحة على حاويات النفايات، والحفاظ على النظافة والتنظيم داخل مناطق تخزين النفايات. |
| قسم الصيانة | إدارة الزيوت والزيوت المستخدمة، وصيانة مناطق تخزين النفايات والمعدات ذات الصلة، ودعم ممارسات إدارة النفايات الآمنة. |
| قسم المشتريات | تحديد وتعاقد شركات إدارة النفايات المرخصة لنقل النفايات والتخلص منها. |
| قسم الشؤون القانونية | مراجعة والموافقة على العقود مع شركات إدارة النفايات المرخصة. |
| قسم الشؤون الإدارية | مراقبة فصل النفايات وجميع أنشطة نقل النفايات الداخلية، والتنسيق مع شركات نقل النفايات المرخصة حول أوقات النقل وكيفية الانتقال |
| قسم اللوجستيات | إدارة إجراءات التخليص الجمركي للنفايات التي تتطلب تصاريح خاصة والحفاظ على الوثائق المتعلقة بتتبع النفايات ونقلها. |
| مدير المستودع | تعيين وتعيين أشخاص مؤهلين لإدارة مرافق تخزين النفايات |
| مسؤول المستودع | إجراء مستندات وسجلات ALLA لكمية النفايات الداخلية والخارجة ومتابعة فصل وتصنيف النفايات في المستودع |

    - نوع المخلفات والتصنيف

| فئة النفايات | النوع / العلبة | التصنيف | أمثلة نموذجية |
| --- | --- | --- | --- |
| النفايات الكيميائية | صلبة | خطرة | الصباغة المستخدمة، المذيبات، المواد الكيميائية المنتهية الصلاحية، الحاويات الملوثة، أي مواد أخرى ملوثة بالمواد الكيميائية وأي معدات حادة تستخدم في عمليات الإنتاج |
| الطين | صلبة | خطر / غير خطير (بناء على التحليل) | طين ETP |
| النفايات العامة | صلبة | غير خطير | الورق، نفايات الطعام، التغليف، المطاط، البلاستيك |
| نفايات النسيج | صلبة | غير خطير | قطع أقمشة، قصاصات |
| جميع المعدات الحادة | صلبة | خطرة | جميع الإبر ومعدات القطع |
| النفايات الإلكترونية | صلبة | خطرة | الأجهزة الإلكترونية التالفة، المصابيح الفلورية |
| الزيت والفلاتر المستعملة | صلبة | خطرة | زيت التزييت، فلاتر الآلات |
| البطاريات | صلبة | خطرة | البطاريات المستخدمة أو التالفة، بطاريات الرافعة الشوكية |
| المياه المنزلية | سائل | غير خطير | التخلص من النفايات البيولوجية البشرية |
| مياه الصرف الصناعي قبل المعالجة | سائل | خطرة | جميع المنتجات السائلة الناتجة عن معالجة الإنتاج |

    - فصل المخلفات

يجب فصل النفايات عند المصدر لمنع التلوث وتسهيل إعادة التدوير.

- مبادئ الفصل النوعي

- افصل بين المخلفات الخطرة وغير الخطرة.

- استخدم حاويات ملونة عند الحاجة أو حاويات يوجد عليها ملصقات توضح موع المخلف داخلها ، وحدد بوضوح جميع حاويات النفايات.

- تجنب خلط أنواع المخلفات غير المتوافقة.

- يجب وضع المخلفات الخطرة في حاويات محكمة الإغلاق ومقاومة للتسرب ومتوافقة.

- يجب أن تعرض جميع الحاويات:

  - اسم المخلف.

  - رموز المخاطر (عند الاقتضاء).

    - مثال على نظام فصل المخلفات

| نوع النفايات | ملصق الحاوية |
| --- | --- |
| نفايات النسيج | حاوية نفايات النسيج |
| ورق وكرتون | حاوية إعادة تدوير الورق |
| النفايات البلاستيكية | حاوية إعادة تدوير البلاستيك |
| النفايات الكيميائية والزيوت | حاوية النفايات الخطرة |
| مخلفات الابر المكسورة والأسلحة الحادة | حاوية نفايات خطرة |
| مخلفات الوبرة | حاوية نفايات الوبرة |
| مخلفات طبية | حاوية نفايات طبية خطرة |
| مخلفات المطعم والكانتين | مخلفات غير خطرة |

    - التخزين المؤقت

- يجب أن تكون مناطق تخزين النفايات المخصصة:

  - تم التعرف عليه بوضوح

  - مهوية جيدا

  - مؤمن وتحت تحكم الوصول

- تشمل مناطق تخزين النفايات الخطرة احتواء ثانوي (تجميع).

- يجب ألا تتجاوز النفايات 90 يوما من التخزين في الموقع ما لم يكن مصرحا به رسميا.

- يجب تخزين النفايات غير المتوافقة بشكل منفصل.

    - جمع المخلفات والنقل الداخلي

يجب جمع ونقل النفايات داخل المنشأة وفقا للإجراءات التالية:

- استخدم عربات أو حاويات مخصصة لنقل النفايات.

- اتبع طرق نقل المخلفات المحددة داخل المنشأة.

- حافظ على التنظيف المناسب أثناء أنشطة نقل النفايات.

- معدات الحماية الشخصية المناسبة

- يجب تسجيل كل نقل في سجل نقل المخلفات الداخلي.

- يجب إدارة أي تسرب أو انسكاب وفقا لإجراء الاستجابة للانسكاب.

    - إعادة التدوير واسترداد المخلفات

تلتزم المنشأة بتقليل توليد النفايات من خلال مبادرات إعادة التدوير والشراكات مع شركات إعادة التدوير المتخصصة.

تشمل الأمثلة:

- إعادة تدوير بقايا النسيج

- إعادة تدوير تغليف الكرتون والورق

- إعادة تدوير المواد البلاستيكية

يجب أن يكون جميع مقاولي إعادة التدوير مرخصين ومعتمدين من قبل هيئة شؤون البيئة المصرية وهيئة تنظيم إدارة النفايات.

    - التخلص من النفايات الخطرة

يجب التخلص من المخلفات الخطرة من خلال مقاولين مرخصين لإدارة  المخلفات وفقا للمتطلبات التنظيمية التي تحددها من:

- هيئة تنظيم إدارة المخلفات المصرية

- الوكالة المصرية للشؤون البيئية

تشمل الإجراءات:

- الحفاظ على قوائم نقل المخلفات الخطرة.

- تتبع كميات المخلفات.

- توثيق التخلص النهائي من خلال وثيقة تتبع المخلفات الرسمية.

    - إدارة الحمأة في ETP

يجب إدارة الحمأة الناتجة عن محطة معالجة الصرف الوظيفي (ETP) كما يلي:

- طين إزالة الماء قبل التخزين.

- خزن الحمأة في حاويات مخصصة.

- نقل الحمأة إلى مرافق التخلص المصرح بها.

    - إدارة مياه الصرف الصناعي

يجب جمع ومعالجة ومراقبة ومراقبة مياه الصرف الصناعي الناتجة عن أنشطة الإنتاج والتشغيل، بشكل صحيح لمنع التلوث البيئي وضمان الامتثال للمعايير البيئية المعمول بها.

يجب إدارة جميع مجاري مياه الصرف الصناعي من خلال محطة معالجة مياه الصرف (ETP) في المنشأة  قبل التصريف أو إعادة الاستخدام.

13.1 مصادر مياه الصرف الصناعي

قد تنشأ مياه الصرف من:

- عملية صبغ القماش

- عمليات تشطيب الأقمشة

- عمليات الطباعة

- التحضير الكيميائي وتنظيف المعدات

- أنظمة المرافق مثل الغلايات وأنظمة التبريد

- مياه الصرف الصحي المنزلية

يجب توجيه جميع تيارات مياه الصرف إلى نظام جمع مياه الصرف المركزي المتصل ب ETP.

13.2 تشغيل محطة معالجة الصرف ا الصناعي (ETP)

يجب تشغيل محطة معالجة الصرف الصناعي وفقا لإجراءات التشغيل المعتمدة لضمان إزالة فعالة للملوثات قبل التصريف أو إعادة الاستخدام.

تشمل مراحل العلاج النموذجية:

- خزان المعادلة

- تحييد الرقم الهيدروجيني

- التخثر والتجمع

- الترسيب الأولي

- العلاج البيولوجي

- الترشيح والتلميع النهائي

يجب تشغيل المحطة ومراقبتها من قبل موظفين مؤهلين.

13.3 مراقبة واختبار مياه الصرف الصحي

يجب مراقبة جودة مياه الصرف الصحي بانتظام لضمان الامتثال للمعايير البيئية.

| المعلمة | الوصف |
| --- | --- |
| PH | الحموضة أو مستوى القلوية |
| COD | الطلب الكيميائي على الأكسجين |
| BOD | الطلب البيولوجي على الأكسجين |
| TSS | إجمالي المواد الصلبة المعلقة |
| COLOUR | تركيز الصبغة |

يجب إجراء المراقبة من خلال:

- المختبر الداخلي للمنشأة.

- مختبرات خارجية معتمدة عند الحاجة.

    - إعادة استخدام المياه المعالجة

حيثما كان ذلك ممكنا تقنيا، يمكن إعادة استخدام مياه الصرف المعالجة داخل المنشأة لتحسين كفاءة المياه وتقليل استهلاك المياه العذبة.

قد تشمل تطبيقات إعادة الاستخدام:

- بعض عمليات الإنتاج

- عمليات التنظيف.

- أنظمة التبريد.

- الري بالمناظر الطبيعية.

ملاحظة: في حال عدم تشغيل محطة المعالجة (ETP)، سيتم نقل المياه الملوثة من قبل شركة معتمدة إلى شركة معالجة متخصصة معتمدة أيضا من قبل الهيئة المصرية للشؤون البيئية (EEAA) وهيئة تنظيم إدارة النفايات (WMRA).

    - الوثائق والسجلات

| السجل | المسؤول | الاحتفاظ / التكرار |
| --- | --- | --- |
| سجل التخلص من النفايات | صاحب المتجر / خدمات الصحة والسلامة والصحة | لكل عملية تصرف |
| سجل جرد النفايات | الصحة والسلامة والصحة / الامتثال | شهريا |
| سجل التخلص من النفايات | الصحة والسلامة والصحة / الامتثال | حسب التقارير |
| بيان النفايات وشهادة | ناقل / خدمات الصحة والسلامة والصحة والسلامة | حسب التقارير |
| تقرير التدقيق الداخلي | ضابط الامتثال | الربع سنوي |

    - التدريب والوعي

يجب أن يتلقى الموظفون تدريبا حول:

- إجراءات فصل المخلفات

- التعامل مع المخلفات الخطرة

- إجراءات الاستجابة للانسكابات

- الوعي البيئي

- سيتم إجراء التدريب:

- خلال فترة استقطاب الموظفين

- على الأقل سنويا بعد ذلك.

    - الاستجابة للطوارئ

- يجب التعامل مع أي تسرب أو تسرب وفقا لبيان الاستجابة للانسكابات الكيميائية.

- يجب أن تتوفر مجموعات التسرب ومعدات الحماية الشخصية للطوارئ في جميع مواقع النفايات الخطرة.

- يجب إبلاغ قسم الصحة والسلامة والصحة فورا.

    - المراقبة والتدقيق

- تجرى عمليات تفتيش شهرية لمناطق تخزين النفايات.

- يجب على التدقيقات الداخلية الدورية التحقق من تتبع النفايات والامتثال.

- يجب الإبلاغ عن بيانات النفايات من خلال:

  - تقييمات هيغ FEM

  - تقارير الأداء في ZDHC

    - مراجع الامتثال

  - إرشادات مياه الصرف الصحي لمركز ZDHC – الإصدار 2.1

  - FEM 4.0 Higg

  - القوانين البيئية الوطنية ومتطلبات EPR

  - ISO 14001: أنظمة إدارة البيئة

التوقيع والاعتماد

الاسم /

التوقيع/