1. PURPOSE

To ensure a structured and immediate response to environmental test non-conformities in final products, in order to:

- Prevent shipment of non-compliant products

- Identify root causes of failures

- Implement effective corrective and preventive actions

- Ensure compliance with customer requirements and international standards

2. SCOPE

This procedure applies to:

- All final products (T-shirts / fabrics)

- All environmental testing requirements (RSL – Restricted Substances List)

- All departments (knitting, dyeing, printing, finishing, quality, laboratory)

3. DEFINITIONS

- Non-Conformance: Any test result exceeding permissible limits

- RSL: Restricted Substances List applicable to final products

- CAP: Corrective Action Plan

4. RESPONSIBILITIES

| Department | Responsibility |
| --- | --- |
| Quality (QA) | Receive and evaluate test results |
| Laboratory | Re-testing and verification |
| EHS / Chemical | Chemical root cause analysis |
| Production | Implement corrective actions |
| Management | Final decision-making |

5. PROCEDURE

5.1 Immediate Notification

- Upon receiving a non-compliant test result:

  - Notify:

    - Quality (QA)

    - EHS

    - Production

- Record the case in:

  - Non-Conformance Log

5.2 Product Containment

Immediate actions:

- Stop product shipment

- Isolate affected quantity in a designated quarantine area

- Identify:

  - LOT number

  - Batch

  - Production records

5.3 Verification & Re-Testing

- Re-test the same sample

- Test another sample from the same batch

✔ Objective: confirm the result

5.4 Root Cause Analysis (RCA)

Investigate by reviewing:

- Chemicals used (ZDHC status)

- Chemical LOT numbers

- Recipes

- Process conditions

5.5 Corrective Actions

Depending on root cause:

- Replace non-compliant chemicals

- Adjust recipes

- Improve washing/treatment processes

- Retrain operators

5.6 Preventive Actions

- Update Approved Chemical List

- Strengthen LOT tracking system

- Enhance chemical approval procedures

5.7 Re-Testing

- Re-test after implementing corrective actions

✔ Product release only after compliant results

5.8 Final Decision

| Status | Action |
| --- | --- |
| Compliant | Release product |
| Non-compliant | Reprocess / downgrade / dispose |

5.9 Customer Communication (if required)

- Inform customer

- Provide:

  - CAP report

  - Re-test results

6. RECORDS

- Non-Conformance Reports

- Test Reports

- RCA Reports

- CAP Records

7. KEY PERFORMANCE INDICATORS (KPIs)

| KPI | Target |
| --- | --- |
| Number of non-conformities | Reduction |
| Closure time | ≤ 7 days |
| Recurrence rate | 0 |

8. AUDIT & REVIEW

- Periodic review of cases

- Trend analysis

- Continuous system improvement