1. PURPOSE

To establish a structured and data-driven system for monitoring, controlling, and reducing water consumption across all facility operations, ensuring:

- Reduction of environmental impact and operational costs

- Compliance with ISO 14001, Higg FEM, ZDHC requirements, and brand expectations

- Continuous improvement in water efficiency and sustainability performance

2. SCOPE

This procedure applies to:

Industrial Water Use:

- Dyeing operations

- Printing processes

- Washing and finishing

- Utilities and support systems

- Wastewater Treatment Plant (ETP)

Domestic Water Use:

- Restrooms and sanitary facilities

- Canteen and kitchen areas

- Handwashing and general use areas

3. DEFINITIONS

- Water Intensity: Water consumption per unit of production (e.g., m³/kg of fabric)

- Reuse Ratio: Percentage of treated or recovered water reused within operations

- Baseline: Average water consumption over the previous 12 months used for comparison

4. ROLES & RESPONSIBILITIES

| Role | Responsibility |
| --- | --- |
| EHS / Sustainability | Monitoring, data analysis, and target setting |
| Production | Implementation of water reduction practices |
| Maintenance | Leak prevention and system efficiency |
| ETP Team | Water treatment and reuse management |
| Top Management | Approval and resource allocation |

5. PROCEDURE

5.1 Metering & Monitoring System

Installation of Water Meters:

- Main water meter for the facility

- Sub-meters for:

  - Dyeing

  - Printing

  - ETP

  - Utilities

  - Domestic usage

Data Recording:

- Daily (operational monitoring)

- Monthly (analysis and reporting)

Key Indicators:

- m³/day

- m³/kg production

✔ All data shall be recorded in a Water Monitoring Dashboard

5.2 Data Analysis

- Compare actual consumption against:

  - Baseline

  - Defined targets

- Identify:

  - High-consumption areas

  - Deviations and abnormal usage

✔ Monthly analysis reports shall be issued

5.3 Industrial Water Reduction (Process Optimization)

A. Dyeing Operations

- Utilize low liquor ratio machines (1:4 – 1:6)

- Reduce the number of rinsing cycles

- Optimize recipes to achieve right-first-time production

- Implement counter-current rinsing systems

✔ Target: 15–30% reduction

B. Printing Operations

- Minimize water use in screen cleaning

- Use high-pressure, low-volume cleaning systems

- Reuse initial rinse water where applicable

C. Washing & Finishing

- Optimize washing cycles and durations

- Implement spray washing systems

- Ensure optimal machine loading

D. Utilities

- Improve boiler efficiency

- Recover and reuse condensate water

5.4 Water Reuse & Recycling

Sources:

- Treated water from ETP

- Cooling water

- Rinse water

Applications:

- Initial washing processes

- Floor cleaning

- Irrigation (where applicable)

✔ Target:

- Water reuse ≥ 50–60%

5.5 Leak Detection & Management

Inspection:

- Weekly visual inspection

- Monthly detailed inspection

Actions:

- Immediate repair within 24 hours

- Record all incidents in maintenance logs

✔ KPI: Zero leakage

5.6 Domestic Water Reduction

Engineering Controls:

- Sensor-based faucets

- Dual-flush toilets

Operational Controls:

- Awareness signage

- Employee engagement programs

5.7 Training & Awareness

- Annual training for all employees

- Awareness posters in:

  - Production areas

  - Restrooms

  - Common areas

6. KEY PERFORMANCE INDICATORS (KPIs)

| KPI | Target |
| --- | --- |
| Water intensity | 5–10% annual reduction |
| Water reuse ratio | ≥ 50–60% |
| Leakage incidents | Zero |
| Data accuracy | 100% |

7. RECORDS

- Water consumption logs

- KPI and analysis reports

- Maintenance and leak records

- ETP reuse records