1.1   Purpose

The purpose of this risk management procedure is to provide a guidance in how to apply consistent and comprehensive risk management.  This procedure provides information on how to identify, analyse, evaluate and treat risks.  In addition, it identifies other key activities needed for an effective risk management a pproach.  The risk management process contained in this procedure aligns with the ISO 45001:2018 Occupational Health and Safety Management System requirements

1.2    Reference Documents

| EMS/OHS/MAU | OHS Manual |
| --- | --- |
| EMS/OHS/ |  |

  - 1.3 Responsibility

| OHS Team | The OHS team should encourage the employees to participate various programs organize by the management |
| --- | --- |
| MR | The MR design, establish and review the process of Risk Assessment Procedure Training of RA team, Conducting RA , Reviewing the RA & Identification of possible changes, RA results communication, Amendments/changes to RA |
| Department Heads | Implementation of Control measures, inform any changes to the existing operation for risk assessment. |

1.4 Process

    - Definitions

      - Hazard: Source, situation of act with potential harm in terms of human injury or ill health or a combination of these. It may include damage to material, equipment, property, environment, workplace or a combination thereof.

      - Risk: Combination of the probability of the occurrence of one or several hazardous situations or the exposure to one or more hazardous situations and the severity of the bodily injury or the damage to health that this event or this/these exposure(s) may cause.

      - Risk Assessment: Process of identifying one or more risks caused by one or more hazards while taking into account any existing means of control and determining if this or these risks are acceptable

      - Exposure: The number of times that workers are exposed to danger during the execution of the task, in terms of frequency, duration or number of people exposed.

      - Severity: The greatest potential consequence that may result from the dangerous situation.

      - Probability: The degree of possibility of occurrence of the hazardous event trigger that can lead to the consequence.

      - Acceptable Risk: Risk that has been brought back to a tolerable level (yellow or green) for the company according to its legal obligations and its internal health and safety procedures.

      - Residual Risk: Risk remaining after applying means of control.

      - Hazardous situations: Any situation in which a person is exposed to one or more hazards.

      - Insignificant: There is no injury only pain. Can obtained treatment through Site First Aid Centre.

      - Minor: There is an injury or pain treated from Site First Aid Centre and employee supposed to carry out his normal duties

      - Moderate: There is an injury or pain treated obtained treatment from hospital consulting medical officer and obtained, consume prescribe medicine or vaccinated and employee returned to his workstation after treatment and supposed to carry out his normal duties

      - Major: As a result of the accident employee admitted to hospital or he is away from work more than 1 working day.

      - Fatality or Loss: Means loss of life or loss of body part which is irreversible nature

      - Rear: Expected to occur at least annually

      - Unlikely: Not expected to occur for years

      - Possible: Expected to occur at least month

      - Likely: Expected to occur at least weekly

      - Certain: Expected to occur at least daily

1.4.2 General

- When a new activity or a modification of an existing activity, an unusual or non-routine task, a corrective or preventive action request, an incident report, an emergency report from a real event or a drill, a risk assessment must be completed or updated accordingly.

- The risk assessment is led by the HS Manager or Safety Representative (if applicable) and must be made in collaboration with trained site risk assessment team familiar with the completion of the task assessed.

- Each site completes a Risk assessment using the Risk Assement Template (EMS/OHS/F02). The names of participants, dates and activities covered during the assessment are written in the tab “attendance” of the record sheet

- Description of the Task, Equipment or Operation must be clearly identified and described to ensure that risk assessment is as representative as possible.

- For each hazard, the risk assessment team lists the following elements in the appropriate columns of the risk assessment form: hazards; who might be harm and consequence or possible harm.

- Using the criteria set out in the document Risk Matrix and guideline (EMS/OHS/ R03), the team identifies the potential severity and probability of occurrence, for each dangerous situation.

- In order to obtain the most realistic result, it is appropriate that members of the team ask the following questions and consider the answers in the risk assessment:

  - Has any incident ever occurred regarding the activity assessed?

  - If so, what were the real consequences of the incidents? How often did these incidents have occurred?

  - If not, what would be the most likely consequence to occur if an incident happens related to the activity assessed? How often is this consequence likely to occur?

  - Scenarios regarding identified consequences shall not be based on more than two conditional situations. Thus, if more than two conditions (if) are necessary to assess the risk, it must be considered unlikely to occur (unrealistic).

    - Assessment of Residual Risk

- Taking into account the means of control currently in place, the team assesses the residual risk by using the method described in Risk Matrix and guideline (EMS/OHS/R03)

- Dangers are then prioritized in terms of their risk level, which also determines the delay at which means of control should be implemented. Items in red must be prioritized.

- When a red or orange residual risk is raised link with a risk machine (e.g. available mobile part, etc.), a risk assessment machine guarding must be carried out according to Risk Matrix and guideline instruction.

- The HS Site Leader determines the means of control to implement in order to eliminate, control, mitigate or minimize hazards. To determine the means of control to put in place, the HS Site Leader considers the following elements:

- specific OHS objectives;

- legal and other applicable requirements;

- results obtained following application of the weighting factory resources of the company (human, financial, technical, etc.).

- He also takes into account the hierarchy of means of control and ensures that the means of control to implement are feasible:

  - elimination;

  - substitution;

  - engineering controls;

  - signage/warning and/or administrative controls;

  - Personal protective equipment.

1.5 Records

|  | Record Description | Retention Responsibility | Minimum Retention Time |
| --- | --- | --- | --- |
| EMS/OHS/R03 | Risk Matrix and guideline | MR |  |
| EMS/OHS/F02. | Risk Assement Template | MR |  |