### 1. Purpose

To establish standardized and safe steps for operating, handling, and maintaining firefighting equipment within the facility, ensuring rapid containment of initial fires and minimizing risks to lives and assets pursuant to ISO 45001:2018 standards.

### 2. Scope

This procedure applies to all fixed and portable firefighting equipment, fire hose cabinets, and networks across all facility departments, warehouses, and related utilities.

### 3. Responsibilities

- HSE Manager/Engineer: Supervises procedure implementation, coordinates periodic inspections, and trains emergency teams and personnel.

- Firefighting and Emergency Response Team: Responds immediately and handles firefighting equipment directly during emergencies.

- Maintenance Team: Monitors network pressure, generators, and fire pumps, resolving any faults promptly.

- All Employees: Immediately reports any fire incident or damage to firefighting equipment.

### 4. Operational Steps and Guidelines

#### First: Portable Fire Extinguishers

- Situation Assessment: Portable extinguishers are used only for incipient stage fires. If the fire expands, evacuate immediately.

- Standard Operating Method (PASS Technique):

  - P (Pull): Pull the safety pin.

  - A (Aim): Aim the nozzle at the base of the fire, not the flames.

  - S (Squeeze): Squeeze the handle to discharge the extinguishing agent.

  - S (Sweep): Sweep side to side horizontally until the fire is extinguished.

- Extinguisher Selection by Fire Class:

  - Dry Chemical Powder (ABC): For solids, flammable liquids, gases, and electrical fires.

  - Carbon Dioxide (CO2): For electrical equipment and control panels.

  - Foam: For flammable liquids (Prohibited for live electrical equipment).

#### Second: Fire Hose Cabinets (FHC)

- Access: Open the cabinet door and pull out the hose/reel.

- Deployment: Unroll the hose completely toward the fire location, ensuring no kinks impede water flow.

- Connection & Valve Operation:

  - Ensure the hose is securely connected to the internal valve and nozzle.

  - Open the water valve completely counterclockwise.

- Operation:

  - At least two persons must hold the hose to control water pressure.

  - Adjust the nozzle pattern to spray/fog (for cooling and heat shielding) or straight stream (for direct suppression) as required.

#### Third: Fixed Fire Suppression Systems and Pumps

- Automatic Sprinkler Systems: Activate automatically upon heat detection and bulb rupture. The main zone control valve must remain open and shall only be shut off by the HSE supervisor after verifying full extinguishment.

- Fire Pumps: Maintained permanently in Auto mode. Upon line pressure drop (due to hose cabinet or sprinkler activation), pumps start automatically to supply the network.

### 5. Inspection, Testing, and Maintenance

- Daily/Weekly Inspection:

  - Ensure clear access around all fire cabinets and extinguishers.

  - Inspect pressure gauges on powder extinguishers (pointer must be in the green zone).

  - Verify safety seals and inspection tags are intact.

  - Test emergency eyewash stations and operate fire pumps regularly.

- Monthly Inspection: Record field checks for each unit in the Firefighting Equipment Inspection Log.

- Annual Maintenance: Conduct certified annual servicing, recharging, hose pressure testing, and hydrostatic testing.

### 6. Training and Awareness

- Conduct practical hands-on training for employees on operating portable extinguishers and fire hose cabinets.

- Execute evacuation drills at least twice annually to evaluate emergency response effectiveness per ISO 45001 requirements, documenting findings and corrective action plans.

### 7. Associated Forms

|  | Record Description | Record Location/ Retention Responsibility | Minimum Retention Time |
| --- | --- | --- | --- |
|  | Fire Extinguisher / Hose Cabinet Inspection Tag | MR | Revise and maintain |
|  | Monthly Firefighting Equipment Inspection Register | MR | Revise and maintain |
|  | Emergency Evacuation Drill & Evaluation Report | MR | Revise and maintain |
|  | Fire Network and Pump Preventive Maintenance Log | MR | Revise and maintain |