### 1. Purpose

To establish operational controls and safety procedures for vehicle parking and traffic management within company premises, ensuring the prevention of pedestrian collisions, asset damage, and facilitating swift evacuation during emergencies.

### 2. Scope

This procedure applies to all vehicles (company vehicles, personal cars, trucks, and mobile equipment), drivers, contractors, and visitors across all company sites and facilities.

### 3. Responsibilities

- HSE Department: Conducts routine inspections, monitors compliance, and updates this procedure.

- Security Department: Controls site access, directs traffic to designated parking areas, and enforces parking regulations.

- All Drivers (Employees / Contractors / Visitors): Fully comply with the requirements outlined in this procedure.

### 4. Operational Controls

#### 4.1. First-Choice / Reverse Parking Policy

- All drivers must park using the Reverse-in, Drive-out method upon arrival.

- Objective: Enable immediate forward egress during emergency evacuations and maximize driver field of vision to prevent reversing accidents.

#### 4.2. Vehicle Immobilization & Securing Checklist

Before leaving the driver's seat, the following steps must be completed:

- Come to a complete stop inside the designated parking bay lines.

- Fully engage the parking brake (handbrake).

- Secure the transmission:

  - Automatic: Place in Park (P).

  - Manual: Engage 1st gear (or Reverse if parked on a decline).

- Turn off the engine and remove the key (Engine idling while unattended is strictly prohibited).

- Roll up all windows and fold side mirrors inward.

#### 4.3. Emergency Clear Zone Rules

Parking or stopping any vehicle is strictly prohibited in the following areas:

- Within 3 meters of fire hydrants, hose cabinets, or fire protection equipment.

- In front of emergency exit doors, evacuation routes, and assembly points.

- Blocking electrical sub-stations, transformers, or generator rooms.

- On designated pedestrian walkways (yellow paths) or forklift traffic lanes.

#### 4.4. Slopes and Heavy Vehicle Controls

- Curbing Wheels on Incline/Decline:

  - Downward Slope: Turn front wheels toward the curb/edge (Right).

  - Upward Slope: Turn front wheels away from the curb (Left).

- Wheel Chocks: Mandatory placement of at least two wheel chocks on rear wheels for all trucks, buses, and heavy equipment immediately upon parking.

### 5. Monitoring & Performance Evaluation

- Periodic inspections are conducted by the HSE team to ensure compliance.

- Unsafe parking incidents are reported as Near Misses for root-cause analysis.

- Non-compliance results in a Corrective Action Request (CAR) issued to the responsible department or contractor.

### 6. Documented Information / Forms

- Parking Area Inspection Checklist (Form-HSE-012-A)