1. PURPOSE

To establish a comprehensive chemical management procedure covering purchasing, handling, storage, and evaluation of chemicals used in the EROGLU GARMENT factory. This procedure ensures compliance with the ZDHC MRSL, Higg Facility Environmental Module (FEM), and ISO 14001/45001 standards for health, safety, Egyptian requirements and legalization and environmental performance.

2. SCOPE

This procedure applies to all departments and personnel involved in the lifecycle of chemicals, including: - Procurement - Chemical Warehouse - Production Areas (Dyeing, Printing, Washing) - ETP (Effluent Treatment Plant) - Maintenance and Cleaning

DEFINITIONS:

The Zero Discharge of Hazardous Chemicals (ZDHC):  program is a global industry initiative aimed at eliminating hazardous chemicals from the supply chain, particularly in the textile, apparel, leather, and footwear industries.

ZDHC MRSL: is a list of chemical substances banned from intentional use in suppliers that process textile materials and trim parts in textile, apparel, leather, and footwear.

Safety Data Sheets (SDS): is a standardized document that provides detailed information about the properties, hazards, safe handling, storage, and emergency measures related to a chemical substance or mixture.

Personal Protective Equipment (PPE): refers to specialized equipment or clothing worn by individuals to protect themselves from hazards that can cause injury, illness, or exposure to harmful substances in the workplace or environment.

The Globally Harmonized System (GHS): is an international standard for classifying and communicating chemical hazards. It ensures that information about the hazards of chemicals is consistently conveyed through labels and safety data sheets (SDS) worldwide.

The Higg Facility Environmental Module (Higg FEM): is a globally recognized sustainability assessment tool designed to measure and evaluate the environmental performance of manufacturing facilities, primarily in the apparel, footwear, textile, and consumer goods industries.

The Higg FEM KPIs: are measurable indicators that assess and track a facility’s environmental performance. These KPIs help facilities monitor progress, identify areas for improvement, and demonstrate sustainability performance to brands and stakeholders.

A Certificate of Analysis (COA): is an official document issued by a manufacturer, supplier, or certified laboratory that verifies the quality, composition, and compliance of a product, material, or chemical based on specified standards or specifications.

3. RESPONSIBILITIES

Procurement Team:

- Source chemicals only from ZDHC-approved suppliers.

- Ensure MRSL compliance with Sustainability Team.

EHS Officer:

- Oversee implementation of this procedure.

- conduct internal audits and training.

Warehouse Supervisor:

- Ensure safe receipt, labeling, and storage of chemicals.

- Maintain inventory.

Production Managers:

- Ensure safe use of chemicals and compliance with SDS.

Sustainability Team:

- Monitor Conformance to ZDHC and Higg FEM KPIs.

- Update documentation.

- Ensure MRSL compliance with Procurement Team.

4. CHEMICAL PURCHASING PROCEDURE

- Only purchase chemicals listed on the ZDHC Gateway – Verified MRSL Conformant List.

- Request the latest Safety Data Sheets (SDS) from the supplier before purchase.

- Evaluate supplier credentials (e.g., ZDHC InCheck, CleanChain reports).

- Maintain a centralized Chemical Approval Register.

Documentation Required: - SDS (in English and local language) - Certificate of Analysis (COA) - ZDHC conformance evidence - Internal chemical approval form

5. CHEMICAL HANDLING PROCEDURE

  - Only trained personnel can handle chemicals.

  - All handlers must wear appropriate PPE (gloves, goggles, aprons, etc.).

  - Follow the handling instructions provided in the SDS.

  - Use designated equipment for mixing, transferring, and diluting chemicals.

  - Emergency procedures (spill, exposure) must be posted and known to all handlers.

Training Topics: - GHS symbols and label reading - Spill response and first aid - Emergency contact and evacuation plan

6. CHEMICAL STORAGE PROCEDURE

- Store chemicals in designated, ventilated, and bunded areas.

- Use flameproof cabinets for flammable materials.

- Segregate incompatible chemicals (acids, bases, oxidizers, flammables).

- Ensure clear GHS-compliant labeling of all containers.

- Maintain Chemical Inventory List updated monthly.

Storage Requirements Based on Type:

| Chemical Type | Storage Method |
| --- | --- |
| Flammable | Fire-rated cabinet |
| Corrosive | Corrosion-proof shelves, away from metal |
| Reactive | Segregated by hazard class |
| Dyes | Cool, dry, labeled area |

7. CHEMICAL EVALUATION AND MONITORING

  - Conduct regular evaluation of all chemicals to ensure MRSL compliance.

  - Maintain records of:

  - Chemical inventory

  - Usage logs

  - Disposal records

  - SDS revisions

- Perform periodic chemical audits (quarterly or after change of supplier).

- Monitor environmental and occupational risks via risk assessment matrix.

Evaluation Tools: - ZDHC InCheck Report - Higg FEM Chemical Inventory Template - ISO 14001 Risk & Opportunity Register

8. CHEMICAL WASTE AND DISPOSAL

- All chemical waste must be categorized and labeled as per local and ISO 14001 regulations.

- Maintain waste disposal logs with authorized waste handlers.

- Segregate hazardous and non-hazardous waste.

- No chemical waste shall be discharged into drains without treatment (via ETP).

9. RECORDKEEPING

Maintain documentation for at least 5 years, including:

  - SDS and COAs

  - Chemical inventory and usage records

  - Training logs

  - Audit findings and corrective actions

  - Supplier certifications (ZDHC, ISO-compliant)

10. Review and Update

This procedure shall be reviewed:

- Annually

- After incidents involving chemicals

- When new chemicals or suppliers are introduced

- In response to updated ZDHC, ISO, or Higg Index requirements

Compliance References:

- ZDHC MRSL v3.1 and Chemical Management Framework

- EGYPTIAN REQIRMENTS AND LEGALIZATION

- Higg FEM 4.0 – Chemical Management Section

- ISO 14001:2015 (Environmental Management Systems)

- ISO 45001:2018 (Occupational Health & Safety Management)