1. PURPOSE

The purpose of this procedure is to establish safe and environmentally responsible requirements for the receipt, storage, segregation, transfer, handling, use and emergency control of chemicals used within the facility.

This procedure aims to:

- Prevent chemical spills, leaks and uncontrolled releases.

- Prevent incompatible chemical reactions.

- Protect workers from chemical exposure.

- Prevent contamination of soil, stormwater and wastewater systems.

- Ensure chemical traceability and inventory control.

- Ensure compliance with applicable legal, customer and chemical management requirements.

- Support implementation of the ZDHC Chemical Management System (CMS), ISO 14001 and ISO 45001 requirements.

2. SCOPE

This procedure applies to all chemicals received, stored, transferred, handled or used within the facility, including chemicals used in:

- Knitting operations.

- Dyeing operations.

- Washing and pretreatment.

- Desizing and scouring.

- Bleaching.

- Finishing operations.

- Printing operations.

- T-shirt and garment production.

- Maintenance activities.

- Boiler and utility operations.

- Water treatment and wastewater treatment.

- Cleaning and housekeeping activities.

- Laboratories and quality control activities.

The procedure applies to employees, chemical warehouse personnel, production workers, maintenance personnel, contractors and chemical suppliers entering the facility.

3. RESPONSIBILITIES

3.1 Sustainability / HSE / Chemical Management Department

The department shall:

- Establish and maintain the Chemical Inventory List (CIL).

- Verify availability of current Safety Data Sheets (SDS).

- Review chemical hazards and storage compatibility.

- Conduct chemical risk assessments.

- Verify chemical compliance with applicable MRSL and customer requirements.

- Conduct chemical storage and handling inspections.

- Provide chemical safety and spill response training.

- Investigate chemical spills and chemical-related incidents.

- Monitor corrective and preventive actions.

3.2 Chemical Warehouse Responsible Person

The warehouse responsible person shall:

- Receive chemicals only after authorization.

- Verify chemical name, supplier, quantity, batch/LOT number and container condition.

- Maintain chemical receipt and issuance records.

- Apply FIFO or FEFO, where applicable.

- Maintain chemical segregation according to compatibility.

- Ensure containers remain properly labelled.

- Immediately report leaks, damaged containers or abnormal conditions.

- Prevent unauthorized access to chemical storage areas.

3.3 Production Department

Production supervisors shall:

- Ensure chemicals are used only for approved processes.

- Ensure workers follow SDS and operating instructions.

- Ensure required PPE is used.

- Prevent excessive chemical quantities from being stored in production areas.

- Ensure chemical containers are closed after use.

- Report spills, leaks and unsafe chemical conditions immediately.

3.4 Employees and Chemical Handlers

Employees shall:

- Follow chemical handling instructions and training.

- Wear required PPE.

- Read and understand chemical labels and hazard pictograms.

- Never mix chemicals unless required by an approved process instruction.

- Never use unlabelled chemical containers.

- Immediately report chemical spills, exposure or damaged containers.

4. CHEMICAL APPROVAL BEFORE PURCHASE

No new chemicals shall be purchased or introduced into the facility without chemical approval.

Before purchase, the following documents and information shall be reviewed:

- Current Safety Data Sheet (SDS).

- Technical Data Sheet (TDS), where applicable.

- Chemical composition and hazard classification.

- ZDHC MRSL conformance information, where applicable.

- Customer or brand restricted substance requirements.

- Applicable certificates or chemical compliance declarations.

- Intended process and area of use.

Restricted or prohibited chemicals shall not be purchased or used.

The approved chemical shall be added to the Chemical Inventory List before routine use.

5. CHEMICAL RECEIVING

Upon chemical delivery, the warehouse responsible person shall inspect the shipment.

The following shall be verified:

- Chemical trade name.

- Manufacturer or supplier name.

- Quantity received.

- Batch or LOT number.

- Manufacturing and expiry date, where applicable.

- Container integrity.

- Chemical label.

- Hazard pictograms.

- Purchase order and delivery documents.

Damaged, leaking, swollen, corroded or improperly labelled containers shall not be accepted into the normal chemical storage area.

Such containers shall be isolated in a designated controlled area and reported immediately to the HSE/Chemical Management Department.

Chemical receipt records shall be maintained for traceability.

6. CHEMICAL LABELLING

All chemical containers shall be clearly labelled.

The label shall include, as applicable:

- Chemical or product name.

- Hazard pictogram.

- Hazard information.

- Supplier or manufacturer information.

- Batch/LOT number.

- Relevant handling precautions.

Labels shall be available in a language understood by workers. Arabic and English labelling should be provided where necessary.

Chemicals transferred into secondary containers shall be immediately labelled.

Food bottles, drinking water bottles or beverage containers shall never be used for chemical storage.

Unlabelled chemicals are strictly prohibited.

7. CHEMICAL STORAGE REQUIREMENTS

Chemical storage areas shall be:

- Dedicated to chemical storage.

- Secured against unauthorized access.

- Clearly identified with chemical hazard signs.

- Adequately ventilated.

- Protected from excessive heat and direct sunlight.

- Provided with suitable lighting.

- Constructed with chemical-resistant flooring.

- Free from uncontrolled floor drains.

- Maintained in a clean and organized condition.

- Provided with suitable fire protection equipment.

- Provided with appropriate spill kits.

- Provided with emergency eyewash and safety shower facilities where chemical exposure risks exist.

Chemicals shall not be stored directly on the floor.

Chemical containers shall be stored on suitable chemical-resistant pallets, racks or shelving.

Wooden pallets shall be avoided for liquid chemical storage where chemical absorption and contamination may occur.

Storage racks shall be stable and suitable for the chemical load.

Heavy chemical containers shall be stored at lower levels.

Containers shall remain closed when not in use.

8. SECONDARY CONTAINMENT

Liquid chemicals shall be stored within suitable secondary containment.

Secondary containment shall:

- Be chemically compatible with the stored chemical.

- Be resistant to corrosion.

- Be maintained without cracks or damage.

- Be kept free from rainwater, waste and unrelated materials.

Chemicals with incompatible properties shall not share the same secondary containment system.

Spilled chemicals collected in secondary containment shall not be returned to the original container unless technically assessed and authorized.

Contaminated spill material shall be managed according to the hazardous waste management procedure.

9. CHEMICAL SEGREGATION AND COMPATIBILITY

Chemicals shall be segregated according to their hazard and chemical compatibility.

The following chemical groups shall be stored separately where applicable:

- Acids.

- Alkalis/bases.

- Oxidizing agents.

- Reducing agents.

- Flammable chemicals.

- Reactive chemicals.

- Toxic chemicals.

- Corrosive chemicals.

Acids shall be separated from alkalis.

Oxidizing agents shall be separated from flammable, combustible and reducing materials.

Sodium hypochlorite shall be separated from acids and ammonia-containing chemicals.

Hydrogen peroxide and other oxidizing agents shall be stored away from incompatible organic and reducing materials.

Chemical compatibility shall be determined using the SDS, particularly Sections 7 and 10, and the approved Chemical Compatibility Matrix.

Where chemical compatibility is uncertain, the HSE/Chemical Management Department shall be consulted before storage.

10. CHEMICAL STORAGE IN PRODUCTION AREAS

Bulk or excessive chemical quantities shall not be stored in production areas.

Only the minimum quantity required for the production process or work shift shall be maintained at the point of use.

Chemicals in production areas shall:

- Be kept in designated locations.

- Have proper labels.

- Be stored within secondary containment where leakage is possible.

- Remain closed when not in use.

- Be protected from vehicle and equipment impact.

- Be separated from incompatible chemicals.

Unused chemicals shall be returned to the designated chemical storage area according to the applicable production and warehouse control system.

11. CHEMICAL HANDLING AND TRANSFER

Before handling chemicals, workers shall:

- Review the chemical label.

- Understand the applicable SDS instructions.

- Wear the required PPE.

- Inspect the container for leaks or damage.

- Verify that the receiving container is compatible and correctly labelled.

Chemical transfer shall be performed using suitable equipment such as:

- Chemical-resistant pumps.

- Closed transfer systems.

- Approved dispensing equipment.

Manual pouring shall be minimized, particularly for corrosive or hazardous chemicals.

Workers shall not use their mouths to siphon chemicals.

Chemicals shall not be mixed unless mixing is required by an approved production recipe, process instruction or technical procedure.

Transfer equipment used for incompatible chemicals shall be separated or adequately cleaned according to an approved cleaning procedure.

12. PERSONAL PROTECTIVE EQUIPMENT

Required PPE shall be determined according to the SDS and chemical risk assessment.

PPE may include:

- Chemical-resistant gloves.

- Safety goggles.

- Face shield.

- Chemical-resistant apron.

- Chemical-resistant safety footwear.

- Protective clothing.

- Suitable respiratory protection where required by risk assessment.

PPE shall be inspected before use.

Damaged or contaminated PPE shall be replaced.

Respiratory protection shall only be used according to an established respiratory protection program and appropriate filter selection.

13. CHEMICAL DISPENSING AND DOSING

Automatic or closed chemical dosing systems shall be used where technically practicable.

Chemical dosing equipment shall be:

- Properly identified.

- Maintained in good condition.

- Checked for leakage.

- Calibrated where measurement accuracy is required.

Chemical recipes and dosing instructions shall be authorized.

Unauthorized modification of chemical recipes is prohibited.

Chemical consumption shall be recorded to support inventory reconciliation and chemical traceability.

14. EMPTY CHEMICAL CONTAINERS

Empty chemical containers shall be treated according to their previous chemical contents and applicable waste requirements.

Empty containers shall:

- Be completely drained where safe and applicable.

- Remain labelled or otherwise traceable to their previous contents.

- Be stored in a designated area.

- Be protected from unauthorized reuse.

- Be transferred to an authorized waste or recycling route where applicable.

Chemical containers shall not be used for drinking water, food, domestic purposes or personal use.

IBC tanks, drums and chemical containers shall not be reused for another chemical unless compatibility, cleaning requirements and intended reuse have been technically assessed and formally approved.

15. CHEMICAL SPILL RESPONSE

In the event of a chemical spill:

- Stop work and identify the chemical, if this can be done safely.

- Warn personnel in the affected area.

- Isolate the spill area.

- Notify the supervisor and HSE/Emergency Response Team.

- Wear appropriate PPE.

- Stop the source of leakage only if safe to do so.

- Protect drains and wastewater systems.

- Use suitable absorbent or spill control material.

- Do not mix incompatible spill residues.

- Collect contaminated material in suitable labelled waste containers.

- Clean the affected area according to SDS requirements.

- Report and investigate the spill.

Chemicals shall not be washed into stormwater drains, soil or uncontrolled drainage systems.

Spill kits shall be inspected regularly and replenished after use.

16. EMERGENCY EXPOSURE

Eye Contact

Immediately flush the eyes using the emergency eyewash station for the period specified in the SDS. Obtain medical attention as required.

Skin Contact

Remove contaminated clothing and rinse the affected area with water using the emergency safety shower where necessary.

Inhalation

Move the affected person to fresh air without exposing rescuers to chemical hazards. Obtain medical assistance.

Ingestion

Do not induce vomiting unless specifically instructed by qualified medical personnel or the SDS.

The applicable SDS shall be made available to medical or emergency personnel.

17. FIRE AND CHEMICAL EMERGENCY

In case of fire involving chemicals:

- Activate the emergency alarm.

- Notify the Emergency Response Team.

- Evacuate according to the emergency plan.

- Identify the chemicals involved where possible.

- Provide relevant SDS information to emergency responders.

- Use only suitable firefighting media specified in the SDS.

Untrained employees shall not attempt to control a chemical fire.

18. CHEMICAL INVENTORY AND TRACEABILITY

The facility shall maintain an updated Chemical Inventory List.

The inventory shall include, as applicable:

- Chemical trade name.

- Manufacturer.

- Supplier.

- Chemical function.

- Area/process of use.

- Monthly or annual consumption.

- Storage location.

- Hazard classification.

- SDS availability.

- ZDHC MRSL conformance status, where applicable.

- Batch/LOT traceability information.

Chemical stock records shall be periodically reconciled with chemical consumption and purchasing records.

Expired, obsolete or unidentified chemicals shall be identified, isolated and appropriately managed.

19. INSPECTION

Chemical storage and handling areas shall be periodically inspected.

Inspection shall cover:

- Chemical labels.

- Container condition.

- Chemical compatibility.

- Secondary containment.

- Chemical leakage.

- Spill kit availability.

- Eyewash and safety shower accessibility.

- Ventilation.

- Fire protection.

- PPE availability.

- Chemical housekeeping.

- Expired chemicals.

- Unauthorized chemicals.

Identified nonconformities shall be recorded and corrective actions followed until closure.

20. TRAINING

Employees handling chemicals shall receive appropriate training before performing chemical handling activities.

Training shall include:

- Chemical hazard awareness.

- GHS hazard pictograms.

- SDS understanding.

- Chemical compatibility.

- Safe chemical storage.

- Chemical handling and transfer.

- Required PPE.

- Chemical spill response.

- Emergency eyewash and safety shower use.

- Chemical waste management.

Refresher training shall be provided periodically and when significant process or chemical changes occur.

21. RECORDS

The following records shall be maintained:

- Chemical Inventory List (CIL).

- Approved Chemical List.

- Safety Data Sheets.

- Chemical receipt records.

- Chemical issuance and consumption records.

- Chemical compatibility matrix.

- Chemical inspection checklists.

- Chemical risk assessments.

- Spill and incident reports.

- Corrective action records.

- Chemical training records.

- Waste transfer and disposal records.

Records shall be retained according to the facility document and record control requirements.

22. REFERENCES

- ZDHC Chemical Management System Framework.

- ZDHC CMS Technical Industry Guide.

- ZDHC Manufacturing Restricted Substances List (MRSL).

- Applicable ZDHC Wastewater Guidelines.

- ISO 14001 Environmental Management Systems.

- ISO 45001 Occupational Health and Safety Management Systems.

- Globally Harmonized System of Classification and Labelling of Chemicals (GHS).

- Applicable Egyptian environmental, occupational health and safety, and waste management legislation.

- Customer and brand chemical requirements.

23. RELATED DOCUMENTS

- Chemical Inventory List (CIL).

- Chemical Approval Procedure.

- Chemical Compatibility Matrix.

- Chemical Risk Assessment.

- Spill Response Procedure.

- Hazardous Waste Management Procedure.

- Emergency Preparedness and Response Procedure.

- PPE Procedure.

- Chemical Warehouse Inspection Checklist.

- Chemical Consumption Log.

Approved By:

Date: