PURPOSE

This instruction describes how the final AQL inspection of pressed and packed goods is carried out at Eroglu Knitting (EMS) before shipment, including pressing and measurement checks, AQL sampling and accept/reject decisions, metal detection verification, packing and labelling checks, and the control of failed lots, in line with AQL ISO 2859-1 and customer requirements (EMS.QA.TL.02.05).

SCOPE

This instruction applies to all orders that are at least 80% packed and ready for shipment, inspected by EMS final inspectors before any customer or third-party inspection. It covers the final inspection stage of the Pressing, Final, Packing and AQL department.

RESPONSIBILITIES

• Final Quality Inspector: Draws the sample, inspects it, classifies defects and fills in the final inspection report and the defect distribution form.

• Pressing Supervisor: Maintains the pressing temperature, steam and measurement standard and corrects pressing defects.

• Packing Supervisor: Prepares the cartons according to the packing list and corrects rejected lots.

• Metal Detector Operator: Performs the daily detector verification, passes all pieces and keeps the record.

• Quality Manager: Approves the result and decides on release, rework, re-inspection or concession.

• OHS Officer: Monitors steam, hot surface and ergonomic risks at the pressing and packing area.

DEFINITIONS

• AQL (Acceptable Quality Limit): The maximum defect level, in percent, accepted as satisfactory in the sampling plan.

• Critical, Major and Minor Defect: A defect that endangers user safety; a defect that impairs the use or sale of the product; a small defect that does not affect use.

• Accept Number (Ac) and Reject Number (Re): The highest number of defects allowed in the sample, and the number of defects that makes the lot fail.

• Defect Distribution: The classification of defects found at final inspection by type.

• Quarantine Box: The locked box in which pieces with detected metal are kept.

INSTRUCTIONS

1. BEFORE STARTING

1.1. Have the sealed sample, measurement chart, packing list, customer specification and customer AQL level ready (commonly General Inspection Level II, AQL 2.5 for major and 4.0 for minor defects; zero tolerance for critical defects). The values are examples; the limits are defined in the customer specification and company policy.

1.2. Confirm that at least 80% of the order is packed and that pressing and in-line inspection records are complete.

1.3. Find the sample size and accept/reject numbers from the ISO 2859-1 table according to the lot quantity and write them on the report.

1.4. Select cartons at random from the whole lot (at least the square root of the total number of cartons). The inspector, not production, chooses the cartons.

2. PRESSING AND MEASUREMENT CHECKS

2.1. Check that the pressing temperature, steam pressure and dwell time comply with the fabric care instruction and the pressing temperature record.

2.2. Check pressed pieces for shine, scorch, creases, water marks and shape.

2.3. Measure the pieces required by the customer against the measurement chart and record the results on the Quality Assurance Measurement Form (EMS.QA.FRM.05.02).

3. INSPECTION STEPS

3.1. Check the cartons: marks, dimensions, weight, condition and quantity against the packing list.

3.2. Check the packing: size and colour assortment, folding, polybags, warning text on polybags, hangtags, barcodes and labels against the approved artwork.

3.3. Inspect each sample piece for appearance, workmanship, colour and fabric against the sealed sample.

3.4. Check the metal detection record: the detector is verified daily with Fe, Non-Fe and SUS test pieces, and 100% of the lot has passed through the detector (EMS.QA.PRO.02.05).

3.5. Classify each defect as critical, major or minor and write it on the Finished Product Defect Distribution Form (EMS.QA.FRM.05.01).

3.6. Compare the defect totals with the accept/reject numbers and give the result: PASS or FAIL. Record it on the Quality Control Report (EMS.QA.FRM.05.03).

4. RULES / PROHIBITIONS

- Do not let production or packing staff choose the cartons to be inspected.

- Do not pass a lot with any critical defect (needle, sharp object, metal, safety or labelling defect).

- Do not ship a lot without a PASS report signed by the Quality Manager.

- Do not open sealed cartons for any purpose other than inspection, and re-seal them under the seal control procedure.

- Do not use an uncalibrated measuring tape or detector.

5. IN CASE OF FAILURE

5.1. A failed lot is labelled "HOLD", shipment is stopped, and the lot is 100% sorted and reworked.

5.2. The reworked lot is re-inspected with a new sample drawn at the same or a tighter inspection level. A second failure is escalated to the Quality Manager.

5.3. Pieces with detected metal are placed in the quarantine box, the cause is investigated, and the lot is re-passed through the detector.

5.4. A corrective and preventive action (CAPA) is opened for the failure, and the defect cause is fed back to the responsible department.

5.5. A lot is shipped with defects only with a written concession from the Quality Manager and, where required, the customer.

6. KEY PERFORMANCE INDICATORS

- AQL reject rate of final inspection: 2% or less.

- Days without daily detector verification: 0; metal findings reaching the customer: 0.

- Shipment rejections due to label, barcode or quantity errors: 0.

- Closing time of an open CAPA: 15 days or less.

7. RELATED DOCUMENTS AND RECORDS

- EMS.QA.TL.02.05 AQL Sampling and Accept/Reject Instruction (QMS)

- EMS.QA.FRM.05.01 Finished Product Defect Distribution Form

- EMS.QA.FRM.05.02 Quality Assurance Measurement Form

- EMS.QA.FRM.05.03 Quality Control Report and Packing List

- EMS.QA.PRO.02.09. Quality Inspection Procedure (Incoming, In-Line, Final AQL)

- EMS.QA.PRO.02.05. Metal Detector Procedure

- EMS.QA.PRO.02.04. Seal Management Procedure

- EMS.KS.PRO.22.02. Nonconforming Product Control Procedure

- EMS.KS.PRO.22.03. Corrective and Preventive Action (CAPA) for Quality Problems Procedure

- EMS.ÜR.PRO.05.10. Pressing, Final Control and Packing Procedure

8. FEEDBACK AND MONITORING

The Quality Manager reviews the final inspection pass rate, defect distribution and metal findings monthly and this instruction every year, and revises it when necessary.

الغرض

تصف هذه التعليمات كيفية تنفيذ الفحص النهائي AQL للبضائع المكوية والمعبأة في شركة Eroglu Knitting (EMS) قبل الشحن، بما في ذلك فحوصات الكي والقياس، وأخذ العينات وفق AQL وقرارات القبول والرفض، والتحقق من كشف المعادن، وفحص التعبئة والملصقات، ومراقبة الدفعات الراسبة، وفقًا لمعيار AQL ISO 2859-1 ومتطلبات العميل (EMS.QA.TL.02.05).

النطاق

تُطبق هذه التعليمات على جميع الطلبيات المعبأة بنسبة 80% على الأقل والجاهزة للشحن، والتي يفحصها مفتشو الفحص النهائي في EMS قبل أي فحص من العميل أو طرف ثالث. وتشمل مرحلة الفحص النهائي في قسم الكي والنهائي والتعبئة وAQL.

المسؤوليات

• مفتش الجودة النهائي: يسحب العينة ويفحصها ويصنف العيوب ويملأ تقرير الفحص النهائي ونموذج توزيع العيوب.

• مشرف الكي: يحافظ على معيار درجة حرارة الكي والبخار والقياس ويصحح عيوب الكي.

• مشرف التعبئة: يجهز الكراتين وفق قائمة التعبئة ويصحح الدفعات المرفوضة.

• مشغل جهاز كشف المعادن: ينفذ التحقق اليومي من الجهاز ويمرر جميع القطع ويحتفظ بالسجل.

• مدير الجودة: يعتمد النتيجة ويقرر الإفراج أو إعادة التشغيل أو إعادة الفحص أو الموافقة الاستثنائية.

• مسؤول السلامة والصحة المهنية: يراقب مخاطر البخار والأسطح الساخنة والإرجونوميك في منطقة الكي والتعبئة.

التعريفات

• AQL (حد الجودة المقبول): أقصى مستوى للعيوب، بالنسبة المئوية، يُعد مقبولًا في خطة أخذ العينات.

• العيب الحرج والرئيسي والثانوي: عيب يعرّض سلامة المستخدم للخطر؛ عيب يضعف استخدام المنتج أو بيعه؛ عيب صغير لا يؤثر على الاستخدام.

• رقم القبول (Ac) ورقم الرفض (Re): أكبر عدد عيوب مسموح به في العينة، وعدد العيوب الذي تُرفض به الدفعة.

• توزيع العيوب: تصنيف العيوب المكتشفة في الفحص النهائي حسب نوعها.

• صندوق الحجر: الصندوق المقفل الذي تُحفظ فيه القطع التي اكتُشف فيها معدن.

التعليمات

1. قبل البدء

1.1. جهّز العينة المختومة وجدول المقاسات وقائمة التعبئة ومواصفات العميل ومستوى AQL الخاص بالعميل (عادة مستوى الفحص العام II، وAQL 2.5 للعيوب الرئيسية و4.0 للعيوب الثانوية، وعدم السماح مطلقًا بالعيوب الحرجة). القيم أمثلة؛ وتُحدد الحدود في مواصفات العميل وسياسة الشركة.

1.2. تأكد من تعبئة 80% على الأقل من الطلبية ومن اكتمال سجلات الكي والفحص أثناء الإنتاج.

1.3. حدد حجم العينة وأرقام القبول والرفض من جدول ISO 2859-1 حسب كمية الدفعة واكتبها في التقرير.

1.4. اختر الكراتين عشوائيًا من الدفعة كاملة (على الأقل الجذر التربيعي لإجمالي عدد الكراتين). يختار المفتش الكراتين وليس الإنتاج.

2. فحوصات الكي والقياس

2.1. تأكد من مطابقة درجة حرارة الكي وضغط البخار ومدة التثبيت لتعليمات العناية بالقماش وسجل درجة حرارة الكي.

2.2. افحص القطع المكوية من حيث اللمعان والحروق والتجاعيد وبقع الماء والشكل.

2.3. قِس القطع التي يطلبها العميل مقارنة بجدول المقاسات وسجل النتائج في نموذج قياس ضمان الجودة (EMS.QA.FRM.05.02).

3. خطوات الفحص

3.1. افحص الكراتين: العلامات والأبعاد والوزن والحالة والكمية مقارنة بقائمة التعبئة.

3.2. افحص التعبئة: توزيع المقاسات والألوان والطي والأكياس البلاستيكية ونص التحذير عليها والبطاقات المعلقة والباركود والملصقات مقارنة بالتصميم المعتمد.

3.3. افحص كل قطعة في العينة من حيث المظهر وجودة التصنيع واللون والقماش مقارنة بالعينة المختومة.

3.4. تحقق من سجل كشف المعادن: يُتحقق من الجهاز يوميًا بقطع الاختبار Fe وNon-Fe وSUS، وتمر 100% من الدفعة عبر الجهاز (EMS.QA.PRO.02.05).

3.5. صنّف كل عيب إلى حرج أو رئيسي أو ثانوي وسجله في نموذج متابعة توزيع عيوب المنتج النهائي (EMS.QA.FRM.05.01).

3.6. قارن مجموع العيوب بأرقام القبول والرفض وأعطِ النتيجة: ناجح أو راسب. وسجلها في تقرير مراقبة الجودة (EMS.QA.FRM.05.03).

4. القواعد / المحظورات

- لا تسمح لعمال الإنتاج أو التعبئة باختيار الكراتين التي ستُفحص.

- لا تُجِز أي دفعة بها عيب حرج (إبرة أو جسم حاد أو معدن أو عيب في السلامة أو الملصقات).

- لا تشحن أي دفعة دون تقرير «ناجح» موقع من مدير الجودة.

- لا تفتح الكراتين المختومة لغير غرض الفحص، وأعد ختمها وفق إجراء مراقبة الأختام.

- لا تستخدم شريط قياس أو جهاز كشف غير معايَر.

5. في حالة الرسوب

5.1. توسم الدفعة الراسبة بملصق «معلّق»، ويُوقف الشحن، وتُفرز الدفعة وتُعاد معالجتها بنسبة 100%.

5.2. يُعاد فحص الدفعة المعالجة بعينة جديدة بنفس مستوى الفحص أو بمستوى أشد. ويُصعّد الرسوب الثاني إلى مدير الجودة.

5.3. توضع القطع التي اكتُشف فيها معدن في صندوق الحجر، ويُحقق في السبب، وتُمرر الدفعة من جديد عبر الجهاز.

5.4. يُفتح إجراء تصحيحي ووقائي (DOP) للرسوب، ويُبلغ القسم المسؤول بسبب العيب.

5.5. لا تُشحن دفعة بها عيوب إلا بموافقة استثنائية مكتوبة من مدير الجودة ومن العميل عند الحاجة.

6. مؤشرات الأداء الرئيسية

- نسبة رفض AQL في الفحص النهائي: 2% أو أقل.

- أيام دون تحقق يومي من الجهاز: 0؛ ملاحظات معدنية وصلت إلى العميل: 0.

- رفض الشحنات بسبب أخطاء الملصقات أو الباركود أو الكمية: 0.

- مدة إغلاق الإجراء التصحيحي المفتوح: 15 يومًا أو أقل.

7. الوثائق والسجلات ذات الصلة

- EMS.QA.TL.02.05 تعليمات أخذ عينات AQL والقبول والرفض (QMS)

- EMS.QA.FRM.05.01 نموذج متابعة توزيع عيوب المنتج النهائي

- EMS.QA.FRM.05.02 نموذج قياس ضمان الجودة

- EMS.QA.FRM.05.03 تقرير مراقبة الجودة وقائمة التعبئة

- EMS.QA.PRO.02.09. إجراء فحص الجودة (الوارد، أثناء الإنتاج، النهائي AQL)

- EMS.QA.PRO.02.05. إجراء جهاز كشف المعادن

- EMS.QA.PRO.02.04. إجراء إدارة الأختام

- EMS.KS.PRO.22.02. إجراء مراقبة المنتج غير المطابق

- EMS.KS.PRO.22.03. إجراء الإجراءات التصحيحية والوقائية (CAPA) لمشاكل الجودة

- EMS.ÜR.PRO.05.10. إجراء الكي والمراقبة النهائية والتعبئة

8. التغذية الراجعة والمتابعة

يراجع مدير الجودة نسبة نجاح الفحص النهائي وتوزيع العيوب وملاحظات المعادن شهريًا، وهذه التعليمات سنويًا، ويعدلها عند الحاجة.

ONAYLAYANLAR:

| Strateji ve Süreç Geliştirme Müdürü BAHAR YILDIRIM |  | İnsan Kaynakları Grup Müdürü ALİ KAAN GENÇ |  | Kalite Güvence Müdürü ABDULLAH BAYDOĞAN |  | EMS Genel Müdürü FATİH SALBARS |
| --- | --- | --- | --- | --- | --- | --- |

Hazırlayan: Kalite Güvence Müdürü  ..........................

Kontrol: Üretim Müdürü  ..........................

Kalite Müdürü: ..........................    Fabrika Müdürü: ..........................