PURPOSE

This instruction describes how embroidery is carried out on multi-head embroidery machines at EMS, so that embroidery matches the approved sample in design, colour and position, and the work is done safely.

SCOPE

It applies to all multi-head embroidery machines, the punching (design) computer, hooping stations and to all embroidery operators and supervisors.

RESPONSIBILITIES

• Embroidery Supervisor: Checks the design file, thread colours and position against the approved sample and approves the first piece.

• Embroidery Operators: Hoop, run and unload garments according to this instruction and check every piece.

• Mechanic: Adjusts tension, needles and hooks and repairs machine faults.

DEFINITIONS

• Design File: The digitised embroidery program giving stitch count, sequence and colour changes.

• Backing: The stabiliser material placed under the fabric to prevent puckering during embroidery.

INSTRUCTIONS

1. BEFORE STARTING

1.1. Check the embroidery order, approved sample, design file version, thread colour codes and backing type.

1.2. Check needles, thread tension, hooks and the needle guard; record the needle count on the needle control form at shift start.

1.3. Load the design, check its size and orientation on screen and set the colour sequence.

2. OPERATION

2.1. Hoop the panel with the right backing, keeping the fabric flat and the marked position centred.

2.2. Run the first piece, compare it with the approved sample and get supervisor approval before bulk production.

2.3. Watch the heads during running; stop the machine for thread breaks, missed stitches or puckering and correct them.

2.4. After embroidery, remove the backing and loose threads carefully, check every piece and keep panels in bundle and size order.

3. SAFETY RULES / PROHIBITIONS

- Do not put your hands near the needles or the moving pantograph while the machine is running.

- Do not remove the needle guards or work with damaged hoops.

- Do not change needles without stopping the machine; follow the needle exchange rule.

- Do not leave scissors or tweezers on the machine table; keep them tied at the workstation.

4. IN CASE OF PROBLEM / EMERGENCY

4.1. If a needle breaks, stop the machine, find all pieces and follow the broken needle replacement instruction.

4.2. For repeated thread breaks or machine faults, call the mechanic; do not adjust hooks or timing yourself.

4.3. In case of injury, press the emergency stop, call the first aider and inform the supervisor.

5. RELATED DOCUMENTS AND RECORDS

- Embroidery Order, Approved Sample and Design File

- Needle Control Form and Embroidery Production Records

- EMS.QA.TL.02.02. Broken Needle Replacement Instruction

- EMS.QA.PRO.02.07. Needle and Sharp Tools Control Procedure

6. FEEDBACK AND MONITORING

Embroidery defects, machine stoppages and needle breakages are monitored monthly by the Embroidery Supervisor. This instruction is reviewed every year or after a new machine type or customer complaint.

الغرض

توضح هذه التعليمات كيفية تنفيذ التطريز على ماكينات التطريز متعددة الرؤوس في شركة إيروغلو للتريكو (EMS)، بحيث يطابق التطريز العينة المعتمدة من حيث التصميم واللون والموضع، ويُنفذ العمل بأمان.

النطاق

تنطبق على جميع ماكينات التطريز متعددة الرؤوس وحاسوب تصميم التطريز ومحطات التطويق، وعلى جميع مشغلي التطريز والمشرفين.

المسؤوليات

• مشرف التطريز: يراجع ملف التصميم وألوان الخيوط والموضع مقابل العينة المعتمدة، ويعتمد القطعة الأولى.

• مشغلو التطريز: يطوّقون الملابس ويشغلون الماكينة ويفرغونها وفق هذه التعليمات، ويفحصون كل قطعة.

• الميكانيكي: يضبط شد الخيط والإبر والخطافات ويصلح أعطال الماكينة.

التعريفات

• ملف التصميم: برنامج التطريز الرقمي الذي يحدد عدد الغرز وتسلسلها وتغييرات الألوان.

• البطانة (الداعم): مادة التثبيت التي توضع تحت القماش لمنع التجعد أثناء التطريز.

التعليمات

1. قبل البدء

1.1. راجع أمر التطريز والعينة المعتمدة وإصدار ملف التصميم وأكواد ألوان الخيوط ونوع البطانة.

1.2. افحص الإبر وشد الخيط والخطافات وواقي الإبر، وسجّل عدد الإبر في نموذج التحكم في الإبر عند بداية الوردية.

1.3. حمّل التصميم وتحقق من مقاسه واتجاهه على الشاشة واضبط تسلسل الألوان.

2. التشغيل

2.1. طوّق القطعة مع البطانة الصحيحة مع إبقاء القماش مستويًا والموضع المحدد في المنتصف.

2.2. شغّل القطعة الأولى وقارنها بالعينة المعتمدة، واحصل على اعتماد المشرف قبل الإنتاج الكمي.

2.3. راقب الرؤوس أثناء التشغيل، وأوقف الماكينة عند انقطاع الخيط أو تخطي الغرز أو التجعد وصحّح ذلك.

2.4. بعد التطريز أزل البطانة والخيوط الزائدة بعناية، وافحص كل قطعة، وحافظ على القطع بترتيب الربطة والمقاس.

3. قواعد السلامة / المحظورات

- لا تقرّب يديك من الإبر أو من الإطار المتحرك (البانتوغراف) أثناء تشغيل الماكينة.

- لا تُزِل واقيات الإبر ولا تعمل بأطواق تالفة.

- لا تغيّر الإبر دون إيقاف الماكينة، واتبع قاعدة استبدال الإبر.

- لا تترك المقصات أو الملاقط على طاولة الماكينة؛ واحتفظ بها مربوطة في مكان العمل.

4. في حالة وجود مشكلة / طارئ

4.1. إذا انكسرت إبرة، أوقف الماكينة واعثر على جميع أجزائها واتبع تعليمات استبدال الإبر المكسورة.

4.2. عند تكرار انقطاع الخيط أو أعطال الماكينة، استدعِ الميكانيكي، ولا تضبط الخطافات أو التوقيت بنفسك.

4.3. عند حدوث إصابة، اضغط زر الإيقاف الطارئ واستدعِ المسعف وأبلغ المشرف.

5. الوثائق والسجلات ذات الصلة

- أمر التطريز والعينة المعتمدة وملف التصميم

- نموذج التحكم في الإبر وسجلات إنتاج التطريز

- EMS.QA.TL.02.02. تعليمات استبدال الإبر المكسورة

- EMS.QA.PRO.02.07. إجراء التحكم في الإبر والأدوات الحادة

6. التغذية الراجعة والمتابعة

يتابع مشرف التطريز شهريًا عيوب التطريز وتوقفات الماكينات وحالات كسر الإبر. وتُراجع هذه التعليمات سنويًا أو عند إدخال نوع جديد من الماكينات أو بعد شكوى عميل.

ONAYLAYANLAR:

| Strateji ve Süreç Geliştirme Müdürü BAHAR YILDIRIM |  | İnsan Kaynakları Grup Müdürü ALİ KAAN GENÇ |  | Kalite Güvence Müdürü ABDULLAH BAYDOGAN |  | EMS Genel Müdürü FATİH SALBARS |
| --- | --- | --- | --- | --- | --- | --- |