PURPOSE

This instruction describes safe lifting, carrying, pushing and pulling of loads at Eroglu Knitting (EMS), in order to prevent back injuries, strains and other musculoskeletal disorders.

SCOPE

It applies to all manual handling of yarn cones, fabric rolls, chemical containers, cartons, cut bundles and trolleys in all departments, warehouses and loading areas.

RESPONSIBILITIES

• Department Managers: Provide trolleys, lifting aids and enough workers so that heavy loads are not handled alone.

• Supervisors: Ensure that workers are trained and apply safe lifting techniques.

• Employees: Assess the load before lifting, use lifting aids and ask for help when needed.

• OHS Specialist: Assesses manual handling risks and recommends weight limits and improvements, with special attention to pregnant workers.

INSTRUCTIONS

1. BEFORE STARTING

1.1. Check the weight, size and stability of the load and plan the route; remove obstacles and check that the floor is not slippery.

1.2. Use a trolley, pallet truck, hoist or forklift wherever possible instead of carrying by hand.

1.3. If the load is too heavy or awkward, ask a colleague to help or split the load.

2. STEPS

2.1. Stand close to the load with feet apart and one foot slightly forward.

2.2. Bend your knees, keep your back straight and get a firm grip with both hands.

2.3. Lift smoothly using your leg muscles, keeping the load close to your body.

2.4. Turn by moving your feet; do not twist your back while holding the load.

2.5. Put the load down by bending your knees, then adjust its position.

2.6. Push trolleys rather than pull them, and do not overload them.

3. SAFETY RULES / PROHIBITIONS

- Do not lift loads above the weight limit posted in the area; heavy fabric rolls and drums are handled with lifting aids or by two persons.

- Do not lift or carry loads above shoulder height or block your view with the load.

- Do not assign heavy lifting to pregnant workers.

- Do not throw or drop cartons, cones or containers.

4. IN CASE OF PROBLEM

4.1. If you feel pain or a strain while handling, stop immediately and inform your supervisor and the clinic.

4.2. Report recurring discomfort or unsafe handling conditions so that the task can be redesigned.

5. RELATED DOCUMENTS AND RECORDS

- EMS.İS.PRO.18.01. Hazard Identification and Risk Assessment Procedure

- EMS.İK.PRO.01.03. Pregnant Workers Procedure

- EMS.İS.TL.18.01. Forklift Safe Operation Instruction

- Manual Handling Training Records and Ergonomic Risk Assessments

6. FEEDBACK AND MONITORING

The OHS Specialist monitors musculoskeletal complaints from the clinic records and workplace observations every quarter and proposes improvements to the OHS Committee. This instruction is reviewed every year.

الغرض

تصف هذه التعليمات الرفع والحمل والدفع والسحب الآمن للأحمال في شركة إيروغلو للتريكو (EMS)، وذلك لمنع إصابات الظهر والإجهاد العضلي والاضطرابات العضلية الهيكلية الأخرى.

النطاق

تنطبق على جميع أعمال المناولة اليدوية لبكرات الغزل ولفائف القماش وعبوات المواد الكيميائية والكراتين وحزم القطع والعربات في جميع الأقسام والمستودعات ومناطق التحميل.

المسؤوليات

• مديرو الأقسام: يوفرون العربات ووسائل الرفع والعدد الكافي من العمال حتى لا تُناول الأحمال الثقيلة بشكل فردي.

• المشرفون: يضمنون تدريب العمال وتطبيقهم لأساليب الرفع الآمن.

• الموظفون: يقيّمون الحمولة قبل رفعها، ويستخدمون وسائل الرفع، ويطلبون المساعدة عند الحاجة.

• أخصائي السلامة والصحة المهنية: يقيّم مخاطر المناولة اليدوية، ويوصي بحدود الأوزان والتحسينات، مع إيلاء اهتمام خاص للعاملات الحوامل.

التعليمات

1. قبل البدء

1.1. تحقق من وزن الحمولة وحجمها وثباتها وخطط للمسار؛ أزل العوائق وتأكد من أن الأرضية غير زلقة.

1.2. استخدم العربة أو عربة المنصات أو الرافعة أو الرافعة الشوكية كلما أمكن بدلًا من الحمل باليد.

1.3. إذا كانت الحمولة ثقيلة جدًا أو صعبة الإمساك، فاطلب المساعدة من زميل أو قسّم الحمولة.

2. الخطوات

2.1. قف قريبًا من الحمولة مع تباعد القدمين وتقديم إحداهما قليلًا.

2.2. اثنِ ركبتيك، وأبقِ ظهرك مستقيمًا، وأمسك الحمولة بإحكام بكلتا يديك.

2.3. ارفع بسلاسة باستخدام عضلات الساقين، مع إبقاء الحمولة قريبة من جسمك.

2.4. استدر بتحريك قدميك، ولا تلوِ ظهرك أثناء حمل الحمولة.

2.5. ضع الحمولة بثني ركبتيك، ثم عدّل وضعها.

2.6. ادفع العربات بدلًا من سحبها، ولا تحمّلها فوق طاقتها.

3. قواعد السلامة / المحظورات

- لا ترفع أحمالًا تتجاوز حد الوزن المعلن في المنطقة؛ وتُناول لفائف القماش الثقيلة والبراميل بوسائل الرفع أو بواسطة شخصين.

- لا ترفع أو تحمل أحمالًا فوق مستوى الكتف، ولا تحجب رؤيتك بالحمولة.

- لا تكلّف العاملات الحوامل برفع الأحمال الثقيلة.

- لا ترمِ أو تُسقط الكراتين أو البكرات أو العبوات.

4. في حالة المشكلة

4.1. إذا شعرت بألم أو إجهاد أثناء المناولة، فتوقف فورًا وأبلغ مشرفك والعيادة.

4.2. أبلغ عن الانزعاج المتكرر أو ظروف المناولة غير الآمنة حتى يُعاد تصميم المهمة.

5. الوثائق والسجلات ذات الصلة

- EMS.İS.PRO.18.01. إجراء تحديد المخاطر وتقييمها

- EMS.İK.PRO.01.03. إجراء العاملات الحوامل

- EMS.İS.TL.18.01. تعليمات التشغيل الآمن للرافعات الشوكية

- سجلات التدريب على المناولة اليدوية وتقييمات المخاطر المريحة (الأرغونومية)

6. التغذية الراجعة والمتابعة

يتابع أخصائي السلامة والصحة المهنية كل ربع سنة الشكاوى العضلية الهيكلية من سجلات العيادة وملاحظات مواقع العمل، ويقترح التحسينات على لجنة السلامة والصحة المهنية. وتُراجع هذه التعليمات سنويًا.

ONAYLAYANLAR:

| Strateji ve Süreç Geliştirme Müdürü BAHAR YILDIRIM |  | İnsan Kaynakları Grup Müdürü ALİ KAAN GENÇ |  | Kalite Güvence Müdürü ABDULLAH BAYDOGAN |  | EMS Genel Müdürü FATİH SALBARS |
| --- | --- | --- | --- | --- | --- | --- |