PURPOSE

This instruction defines how all chemical containers at Eroglu Knitting (EMS) are labelled, so that every person can identify the chemical and its hazards at a glance and no unlabelled chemical is ever used.

SCOPE

This instruction applies to original supplier containers, transfer (secondary) containers, day tanks, dosing tanks, spray bottles and waste chemical containers in all departments.

RESPONSIBILITIES

• Chemical Manager: Prepares the standard label templates and checks labelling during area inspections.

• Warehouse Supervisor: Checks that original containers are correctly labelled at receipt.

• Users (operators, technicians, cleaners): Label every container they fill and report damaged or missing labels.

INSTRUCTIONS

1. RECEIVING ORIGINAL CONTAINERS

1.1. Accept only containers with the supplier GHS label showing product name, pictograms, signal word, hazard and precautionary statements and supplier details.

1.2. Add the EMS chemical code and the receipt date; if the supplier label is not in Arabic, add an Arabic EMS label next to it.

2. LABELLING SECONDARY CONTAINERS

2.1. Before filling, label the container with the EMS secondary label showing:

- Chemical name and EMS chemical code,

- GHS pictograms and signal word (Danger / Warning),

- Main hazards and required PPE,

- Filling date and name of the person who filled it.

2.2. Use only clean containers that are compatible with the chemical; never use food or drink bottles.

2.3. Waste chemical containers are labelled as 'Hazardous Waste' with the content and date.

3. SAFETY RULES / PROHIBITIONS

- Do not use any chemical from an unlabelled or illegibly labelled container; take it to the Chemical Manager.

- Do not cover, remove or write over a label; replace damaged labels immediately.

- Do not reuse an empty container for a different chemical without cleaning it and replacing the label.

4. IN CASE OF PROBLEM

4.1. An unlabelled container is isolated, marked 'Unknown – Do Not Use' and reported to the Chemical Manager, who identifies or disposes of it safely.

5. RELATED DOCUMENTS AND RECORDS

- EMS Chemical Label Templates

- Chemical Area Inspection Checklist

- EMS.QA.PRO.02.02. Chemical Hazard Communication Procedure

- EMS.KM.PRO.20.02. Chemical Risk Assessment and Safety Data Sheet Management Procedure

- EMS.ÇV.TL.19.02. Hazardous Waste Temporary Storage Area Instruction

6. FEEDBACK AND MONITORING

Labelling is checked in monthly chemical area inspections; findings are followed up by the Chemical Manager. This instruction is reviewed every year or when GHS or customer requirements change.

الغرض

تحدد هذه التعليمات كيفية وسم جميع حاويات المواد الكيميائية في شركة إيروغلو للتريكو (EMS)، بحيث يتمكن كل شخص من التعرف على المادة الكيميائية ومخاطرها بنظرة واحدة، ولا تُستخدم أبدًا أي مادة كيميائية بدون ملصق.

النطاق

تنطبق هذه التعليمات على الحاويات الأصلية من المورد، وحاويات النقل (الثانوية)، والخزانات اليومية، وخزانات الجرعات، وزجاجات الرش، وحاويات النفايات الكيميائية في جميع الأقسام.

المسؤوليات

• مدير الكيماويات: يعد نماذج الملصقات القياسية ويتحقق من الوسم أثناء تفتيش المناطق.

• مشرف المستودع: يتحقق من وسم الحاويات الأصلية بشكل صحيح عند الاستلام.

• المستخدمون (المشغلون والفنيون وعمال النظافة): يضعون ملصقًا على كل حاوية يملؤونها ويبلغون عن الملصقات التالفة أو المفقودة.

التعليمات

1. استلام الحاويات الأصلية

1.1. لا تقبل إلا الحاويات التي تحمل ملصق GHS من المورد ويظهر عليه اسم المنتج والرموز التحذيرية وكلمة التنبيه وعبارات الخطر والاحتياطات وبيانات المورد.

1.2. أضف رمز EMS الكيميائي وتاريخ الاستلام؛ وإذا لم يكن ملصق المورد باللغة العربية فأضف بجانبه ملصق EMS باللغة العربية.

2. وسم الحاويات الثانوية

2.1. قبل التعبئة، ضع على الحاوية ملصق EMS الثانوي الذي يبين:

- اسم المادة الكيميائية ورمز EMS الكيميائي،

- رموز GHS التحذيرية وكلمة التنبيه (خطر / تحذير)،

- المخاطر الرئيسية ومعدات الوقاية المطلوبة،

- تاريخ التعبئة واسم الشخص الذي قام بها.

2.2. لا تستخدم إلا حاويات نظيفة ومتوافقة مع المادة الكيميائية؛ ولا تستخدم أبدًا زجاجات الطعام أو الشراب.

2.3. تُوسم حاويات النفايات الكيميائية بعبارة «نفايات خطرة» مع المحتوى والتاريخ.

3. قواعد السلامة / المحظورات

- لا تستخدم أي مادة كيميائية من حاوية بدون ملصق أو بملصق غير مقروء؛ وسلّمها إلى مدير الكيماويات.

- لا تغطِّ الملصق ولا تنزعه ولا تكتب فوقه؛ واستبدل الملصقات التالفة فورًا.

- لا تُعد استخدام حاوية فارغة لمادة كيميائية مختلفة دون تنظيفها واستبدال ملصقها.

4. عند حدوث مشكلة

4.1. تُعزل الحاوية غير الموسومة وتُعلّم بعبارة «مجهولة – لا تستخدم» ويُبلغ مدير الكيماويات الذي يتعرف عليها أو يتخلص منها بأمان.

5. الوثائق والسجلات ذات الصلة

- نماذج ملصقات المواد الكيميائية الخاصة بـ EMS

- قائمة تفتيش مناطق المواد الكيميائية

- EMS.QA.PRO.02.02. إجراء الإبلاغ عن المخاطر الكيميائية

- EMS.KM.PRO.20.02. إجراء تقييم المخاطر الكيميائية وإدارة صحائف بيانات السلامة

- EMS.ÇV.TL.19.02. تعليمات منطقة التخزين المؤقت للنفايات الخطرة

6. التغذية الراجعة والمتابعة

يُفحص الوسم خلال التفتيش الشهري لمناطق المواد الكيميائية، ويتابع مدير الكيماويات الملاحظات. وتُراجع هذه التعليمات سنويًا أو عند تغير متطلبات GHS أو العملاء.

ONAYLAYANLAR:

| Strateji ve Süreç Geliştirme Müdürü BAHAR YILDIRIM |  | İnsan Kaynakları Grup Müdürü ALİ KAAN GENÇ |  | Kalite Güvence Müdürü ABDULLAH BAYDOGAN |  | EMS Genel Müdürü FATİH SALBARS |
| --- | --- | --- | --- | --- | --- | --- |