1. PURPOSE

To establish a comprehensive, controlled and auditable system at Eroglu Knitting (EMS) for managing lint (fibrous dust) in order to:

• Prevent accumulation and airborne dispersion

• Eliminate fire and explosion risks

• Maintain product quality and prevent contamination

• Control both hazardous and non-hazardous lint streams

• Ensure compliance with global brand requirements and environmental standards

• Operate effectively without centralized extraction systems

2. SCOPE

This procedure applies to all lint-generating operations:

2.1 Production Areas

• Spinning (blow room, carding, drawing, ring/compact)

• Knitting

• Dyeing & Finishing

• Printing

• Cutting

• Sewing

2.2 Supporting Areas

• Fabric storage & warehouses

• Packing sections

• Maintenance workshops

• Utility areas (boilers, compressors)

3. COMPLIANCE FRAMEWORK

This procedure complies with:

• Higg Facility Environmental Module (waste, housekeeping, risk control)

• Inditex – GTW (Fire Safety & Housekeeping)

• Next plc Audit Protocol

• Kontoor Brands Standards

• Levi Strauss & Co. EHS Guidelines

• ISO 14001 Environmental Management System

• Local fire and occupational safety regulations

4. DEFINITIONS

4.1 Lint (Fluff)

Fine fibrous airborne or settled particles generated during textile processes.

4.2 Non-Hazardous Lint

• Clean cotton or polyester lint

• Generated from spinning, knitting and cutting

4.3 Hazardous Lint

Lint contaminated with:

• Dyes, chemicals or auxiliaries

• Printing inks or solvents

• Oils, grease or lubricants

5. RISK ASSESSMENT & CLASSIFICATION

5.1 Area-Based Risk Matrix

• Spinning: Risk level: Very High; Source: Fiber dust; Fire risk: Very High; Cleaning frequency: Every 2 hrs

• Knitting: Risk level: High; Source: Yarn lint; Fire risk: High; Cleaning frequency: Every 2–4 hrs

• Cutting: Risk level: High; Source: Fabric dust; Fire risk: High; Cleaning frequency: Every shift

• Sewing: Risk level: Medium; Source: Thread lint; Fire risk: Medium; Cleaning frequency: Daily

• Dyeing/Finishing: Risk level: Medium; Source: Contaminated lint; Fire risk: Medium; Cleaning frequency: Daily

• Warehouse: Risk level: Low; Source: Settled lint; Fire risk: Low; Cleaning frequency: Daily

5.2 Risk Factors

• Accumulation near heat sources

• Airborne dispersion

• Chemical contamination

• Poor housekeeping

6. ROLES & RESPONSIBILITIES

• EHS Manager: Develop the system, KPI monitoring, audit readiness, risk assessment

• Production Manager: Control lint at source, enforce compliance

• Maintenance: Prevent lint buildup inside machines, ensure sealing

• Housekeeping Supervisor: Implement cleaning schedules, verify execution

• Operators: Immediate cleaning at source, reporting

• Warehouse Team: Control storage and segregation

7. LINT CLASSIFICATION & SEGREGATION SYSTEM

7.1 Classification

• Non-Hazardous: Source: Spinning, knitting, cutting; Handling method: Reuse / recycle

• Hazardous: Source: Dyeing, printing, oily areas; Handling method: Controlled disposal

7.2 Container Standards

• Fire-resistant, closed containers

• Clearly labeled with: waste type, hazard level, source and date

• Color coding: Gray → Non-hazardous; Dark Red → Hazardous

8. CONTROL STRATEGY (WITHOUT EXTRACTION SYSTEM)

8.1 Engineering Controls

• Industrial vacuum cleaners (HEPA recommended)

• Machine enclosures and covers

• Local lint traps

• Anti-static measures

• Controlled compressed air systems

8.2 Administrative Controls (CRITICAL)

• Strict cleaning schedules

• Defined accountability per area

• Supervisory verification

• KPI-driven monitoring

9. OPERATIONAL CONTROL PROCEDURE

9.1 Cleaning Schedule (MANDATORY)

• Machines: Method: Vacuum; Frequency: Every 2–4 hrs; Responsible: Operator

• Floors: Method: Vacuum; Frequency: Every shift; Responsible: Housekeeping

• Elevated/Hidden Areas: Method: Deep cleaning; Frequency: Daily; Responsible: Housekeeping

• Electrical Panels: Method: Inspection & cleaning; Frequency: Daily; Responsible: Maintenance

9.2 Cleaning Methods

• Approved: Industrial vacuum cleaning (primary method)

• Restricted: Compressed air (only in controlled zones with supervision)

• Prohibited: Dry sweeping; air blowing without containment

9.3 Lint Collection Process

• Collect lint immediately after cleaning

• Transfer into labeled containers

• Ensure containers remain closed

• No temporary accumulation allowed

9.4 Storage Requirements

• Separate hazardous and non-hazardous lint

• Store away from heat sources and electrical systems

• Daily removal from production

9.5 Disposal & Recycling

• Non-Hazardous: Recycle (preferred); textile reuse

• Hazardous: Dispose via licensed contractor; maintain manifests and traceability

10. FIRE & EXPLOSION RISK CONTROL

• Zero lint near ignition sources

• Maintain clear zones around electrical panels, motors and boilers

• Daily inspection checklist

• Firefighting equipment accessible

• Emergency drills conducted

11. MONITORING & KPI SYSTEM

• Visible lint: Target: Zero; Method: Inspection

• Cleaning compliance: Target: 100%; Method: Logs

• Segregation compliance: Target: 100%; Method: Audit

• Fire incidents: Target: Zero; Method: Incident tracking

12. DOCUMENTATION (AUDIT REQUIREMENT)

12.1 Cleaning Log

• Frequency-based

• Signed by the responsible person

12.2 Inspection Reports

• Daily supervisor verification

12.3 Waste Records

• Quantity

• Type

• Disposal method

• Contractor

12.4 Training Records

• Attendance

• Topics

• Evaluation

13. INTERNAL AUDIT SYSTEM

The internal audit checklist includes:

• No visible lint

• Proper segregation

• Labeling compliance

• Closed containers

• Electrical safety

14. TRAINING PROGRAM

• Induction training

• Annual refresher

• Fire risk awareness

• Chemical contamination awareness

15. NON-CONFORMITY MANAGEMENT

• Immediate correction

• Root cause analysis (RCA)

• Corrective action plan (CAPA)

• Follow-up verification

16. CONTINUOUS IMPROVEMENT

Reduce lint generation at source:

• Improve fiber quality (spinning)

• Optimize machine settings

• Improve housekeeping efficiency

• Evaluate future engineering solutions

17. CRITICAL AUDIT POINTS

Auditors will focus on:

• Lint accumulation (especially spinning & knitting)

• Electrical panel cleanliness

• Hazardous lint segregation

• Cleaning records consistency

1. الغرض

إنشاء نظام شامل ومُحكم وقابل للتدقيق في إيروغلو للتريكو (EMS) لإدارة الوبر (الغبار الليفي) من أجل:

• منع التراكم والانتشار في الهواء

• القضاء على مخاطر الحريق والانفجار

• الحفاظ على جودة المنتج ومنع التلوث

• التحكم في تيارات الوبر الخطرة وغير الخطرة

• ضمان الامتثال لمتطلبات العلامات التجارية العالمية والمعايير البيئية

• العمل بفعالية دون أنظمة شفط مركزية

2. النطاق

يُطبَّق هذا الإجراء على جميع العمليات المولِّدة للوبر:

2.1 مناطق الإنتاج

• الغزل (غرفة التفتيح، التمشيط، السحب، الغزل الحلقي/المدمج)

• التريكو

• الصباغة والتجهيز

• الطباعة

• القص

• الخياطة

2.2 المناطق المساندة

• تخزين الأقمشة والمستودعات

• أقسام التعبئة

• ورش الصيانة

• مناطق المرافق (الغلايات، الضواغط)

3. إطار الامتثال

يمتثل هذا الإجراء لما يلي:

• وحدة Higg البيئية للمنشآت (النفايات، النظافة والترتيب، التحكم في المخاطر)

• Inditex – GTW (السلامة من الحرائق والنظافة والترتيب)

• بروتوكول تدقيق Next plc

• معايير Kontoor Brands

• إرشادات البيئة والصحة والسلامة لدى Levi Strauss & Co.

• نظام الإدارة البيئية ISO 14001

• اللوائح المحلية للسلامة من الحرائق والسلامة المهنية

4. التعاريف

4.1 الوبر (الزغب)

جسيمات ليفية دقيقة عالقة في الهواء أو مترسبة تتولد أثناء عمليات النسيج.

4.2 الوبر غير الخطر

• وبر قطني أو بوليستر نظيف

• يتولد من الغزل والتريكو والقص

4.3 الوبر الخطر

الوبر الملوث بما يلي:

• الأصباغ أو المواد الكيميائية أو المواد المساعدة

• أحبار الطباعة أو المذيبات

• الزيوت أو الشحوم أو مواد التزييت

5. تقييم المخاطر والتصنيف

5.1 مصفوفة المخاطر حسب المنطقة

• الغزل: مستوى الخطر: عالٍ جدًا؛ المصدر: غبار الألياف؛ خطر الحريق: عالٍ جدًا؛ تكرار التنظيف: كل ساعتين

• التريكو: مستوى الخطر: عالٍ؛ المصدر: وبر الخيوط؛ خطر الحريق: عالٍ؛ تكرار التنظيف: كل 2–4 ساعات

• القص: مستوى الخطر: عالٍ؛ المصدر: غبار القماش؛ خطر الحريق: عالٍ؛ تكرار التنظيف: كل وردية

• الخياطة: مستوى الخطر: متوسط؛ المصدر: وبر الخيوط؛ خطر الحريق: متوسط؛ تكرار التنظيف: يوميًا

• الصباغة/التجهيز: مستوى الخطر: متوسط؛ المصدر: وبر ملوث؛ خطر الحريق: متوسط؛ تكرار التنظيف: يوميًا

• المستودع: مستوى الخطر: منخفض؛ المصدر: وبر مترسب؛ خطر الحريق: منخفض؛ تكرار التنظيف: يوميًا

5.2 عوامل الخطر

• التراكم بالقرب من مصادر الحرارة

• الانتشار في الهواء

• التلوث الكيميائي

• سوء النظافة والترتيب

6. الأدوار والمسؤوليات

• مدير البيئة والصحة والسلامة: تطوير النظام، ومراقبة مؤشرات الأداء الرئيسية، والجاهزية للتدقيق، وتقييم المخاطر

• مدير الإنتاج: التحكم في الوبر من المصدر، وفرض الامتثال

• الصيانة: منع تراكم الوبر داخل الآلات، وضمان الإحكام

• مشرف النظافة: تطبيق جداول التنظيف، والتحقق من تنفيذها

• المشغلون: التنظيف الفوري عند المصدر، والإبلاغ

• فريق المستودع: التحكم في التخزين والفصل

7. نظام تصنيف الوبر وفصله

7.1 التصنيف

• غير خطر: المصدر: الغزل، التريكو، القص؛ طريقة المناولة: إعادة الاستخدام / إعادة التدوير

• خطر: المصدر: الصباغة، الطباعة، المناطق الملوثة بالزيوت؛ طريقة المناولة: تخلص مُحكم

7.2 معايير الحاويات

• حاويات مغلقة مقاومة للحريق

• موسومة بوضوح بما يلي: نوع النفايات، مستوى الخطورة، المصدر، التاريخ

• الترميز اللوني: الرمادي ← غير خطر؛ الأحمر الداكن ← خطر

8. استراتيجية التحكم (بدون نظام شفط)

8.1 الضوابط الهندسية

• مكانس كهربائية صناعية (يُوصى بفلاتر HEPA)

• حواجز وأغطية للآلات

• مصائد وبر موضعية

• تدابير مضادة للكهرباء الساكنة

• أنظمة هواء مضغوط مُحكمة

8.2 الضوابط الإدارية (حرجة)

• جداول تنظيف صارمة

• مساءلة محددة لكل منطقة

• تحقق إشرافي

• مراقبة قائمة على مؤشرات الأداء الرئيسية

9. إجراء التحكم التشغيلي

9.1 جدول التنظيف (إلزامي)

• الآلات: الطريقة: الشفط بالمكنسة؛ التكرار: كل 2–4 ساعات؛ المسؤول: المشغل

• الأرضيات: الطريقة: الشفط بالمكنسة؛ التكرار: كل وردية؛ المسؤول: فريق النظافة

• المناطق المرتفعة/المخفية: الطريقة: تنظيف عميق؛ التكرار: يوميًا؛ المسؤول: فريق النظافة

• اللوحات الكهربائية: الطريقة: فحص وتنظيف؛ التكرار: يوميًا؛ المسؤول: الصيانة

9.2 طرق التنظيف

• معتمدة: التنظيف بالمكانس الكهربائية الصناعية (الطريقة الأساسية)

• مقيدة: الهواء المضغوط (فقط في المناطق المُحكمة وتحت الإشراف)

• محظورة: الكنس الجاف؛ النفخ بالهواء دون احتواء

9.3 عملية جمع الوبر

• جمع الوبر فور الانتهاء من التنظيف

• نقله إلى حاويات موسومة

• ضمان بقاء الحاويات مغلقة

• لا يُسمح بأي تراكم مؤقت

9.4 متطلبات التخزين

• فصل الوبر الخطر عن غير الخطر

• التخزين بعيدًا عن مصادر الحرارة والأنظمة الكهربائية

• الإزالة اليومية من مناطق الإنتاج

9.5 التخلص وإعادة التدوير

• غير الخطر: إعادة التدوير (مفضلة)؛ إعادة الاستخدام في النسيج

• الخطر: التخلص عبر مقاول مرخص؛ والاحتفاظ بمستندات الشحن وضمان التتبع

10. التحكم في مخاطر الحريق والانفجار

• انعدام الوبر بالقرب من مصادر الاشتعال

• الحفاظ على مناطق خالية حول اللوحات الكهربائية والمحركات والغلايات

• قائمة فحص يومية

• سهولة الوصول إلى معدات مكافحة الحريق

• إجراء تدريبات الطوارئ

11. نظام المراقبة ومؤشرات الأداء الرئيسية

• الوبر المرئي: الهدف: صفر؛ الطريقة: التفتيش

• الامتثال للتنظيف: الهدف: 100%؛ الطريقة: السجلات

• الامتثال للفصل: الهدف: 100%؛ الطريقة: التدقيق

• حوادث الحريق: الهدف: صفر؛ الطريقة: تتبع الحوادث

12. التوثيق (متطلب التدقيق)

12.1 سجل التنظيف

• وفقًا لتكرار التنظيف

• موقَّع من الشخص المسؤول

12.2 تقارير التفتيش

• تحقق يومي من المشرف

12.3 سجلات النفايات

• الكمية

• النوع

• طريقة التخلص

• المقاول

12.4 سجلات التدريب

• الحضور

• الموضوعات

• التقييم

13. نظام التدقيق الداخلي

تتضمن قائمة التدقيق الداخلي ما يلي:

• عدم وجود وبر مرئي

• الفصل السليم

• الامتثال لمتطلبات الوسم

• الحاويات المغلقة

• السلامة الكهربائية

14. برنامج التدريب

• التدريب التعريفي

• التدريب التنشيطي السنوي

• التوعية بمخاطر الحريق

• التوعية بالتلوث الكيميائي

15. إدارة عدم المطابقة

• التصحيح الفوري

• تحليل السبب الجذري (RCA)

• خطة الإجراءات التصحيحية (CAPA)

• التحقق من المتابعة

16. التحسين المستمر

تقليل توليد الوبر من المصدر من خلال:

• تحسين جودة الألياف (الغزل)

• تحسين إعدادات الآلات

• رفع كفاءة النظافة والترتيب

• تقييم الحلول الهندسية المستقبلية

17. نقاط التدقيق الحرجة

سيركز المدققون على ما يلي:

• تراكم الوبر (خاصة في الغزل والتريكو)

• نظافة اللوحات الكهربائية

• فصل الوبر الخطر

• اتساق سجلات التنظيف

ONAYLAYANLAR:

| Strateji ve Süreç Geliştirme Müdürü BAHAR YILDIRIM |  | İnsan Kaynakları Grup Müdürü ALİ KAAN GENÇ |  | Kalite Güvence Müdürü ABDULLAH BAYDOĞAN |  | EMS Genel Müdürü FATİH SALBARS |
| --- | --- | --- | --- | --- | --- | --- |