1. PURPOSE

To establish a comprehensive chemical management procedure covering the purchasing, handling, storage and evaluation of chemicals used in the Eroglu Knitting (EMS) factory. This procedure ensures compliance with the ZDHC MRSL, the Higg Facility Environmental Module (FEM), ISO 14001/45001 standards, and Egyptian requirements and legislation for health, safety and environmental performance.

2. SCOPE

This procedure applies to all departments and personnel involved in the lifecycle of chemicals, including:

• Procurement

• Chemical Warehouse

• Production Areas (Dyeing, Printing, Washing)

• ETP (Effluent Treatment Plant)

• Maintenance and Cleaning

3. DEFINITIONS

• Zero Discharge of Hazardous Chemicals (ZDHC): A global industry initiative aimed at eliminating hazardous chemicals from the supply chain, particularly in the textile, apparel, leather and footwear industries.

• ZDHC MRSL: A list of chemical substances banned from intentional use at suppliers that process textile materials and trim parts in textile, apparel, leather and footwear.

• Safety Data Sheet (SDS): A standardised document that provides detailed information about the properties, hazards, safe handling, storage and emergency measures related to a chemical substance or mixture.

• Personal Protective Equipment (PPE): Specialised equipment or clothing worn by individuals to protect themselves from hazards that can cause injury, illness or exposure to harmful substances in the workplace or environment.

• Globally Harmonized System (GHS): An international standard for classifying and communicating chemical hazards. It ensures that information about the hazards of chemicals is consistently conveyed through labels and safety data sheets (SDS) worldwide.

• Higg Facility Environmental Module (Higg FEM): A globally recognised sustainability assessment tool designed to measure and evaluate the environmental performance of manufacturing facilities, primarily in the apparel, footwear, textile and consumer goods industries.

• Higg FEM KPIs: Measurable indicators that assess and track a facility's environmental performance. These KPIs help facilities monitor progress, identify areas for improvement and demonstrate sustainability performance to brands and stakeholders.

• Certificate of Analysis (COA): An official document issued by a manufacturer, supplier or certified laboratory that verifies the quality, composition and compliance of a product, material or chemical against specified standards or specifications.

4. RESPONSIBILITIES

4.1 Procurement Team

• Source chemicals only from ZDHC-approved suppliers.

• Ensure MRSL compliance together with the Sustainability Team.

4.2 EHS Officer

• Oversee the implementation of this procedure.

• Conduct internal audits and training.

4.3 Warehouse Supervisor

• Ensure the safe receipt, labelling and storage of chemicals.

• Maintain the inventory.

4.4 Production Managers

• Ensure the safe use of chemicals and compliance with the SDS.

4.5 Sustainability Team

• Monitor conformance to ZDHC and Higg FEM KPIs.

• Update documentation.

• Ensure MRSL compliance together with the Procurement Team.

5. CHEMICAL PURCHASING PROCEDURE

• Only purchase chemicals listed on the ZDHC Gateway – Verified MRSL Conformant List.

• Request the latest Safety Data Sheets (SDS) from the supplier before purchase.

• Evaluate supplier credentials (e.g. ZDHC InCheck, CleanChain reports).

• Maintain a centralised Chemical Approval Register.

Documentation required:

• SDS (in English and the local language)

• Certificate of Analysis (COA)

• ZDHC conformance evidence

• Internal chemical approval form

6. CHEMICAL HANDLING PROCEDURE

• Only trained personnel may handle chemicals.

• All handlers must wear appropriate PPE (gloves, goggles, aprons, etc.).

• Follow the handling instructions provided in the SDS.

• Use designated equipment for mixing, transferring and diluting chemicals.

• Emergency procedures (spill, exposure) must be posted and known to all handlers.

Training topics:

• GHS symbols and label reading

• Spill response and first aid

• Emergency contacts and evacuation plan

7. CHEMICAL STORAGE PROCEDURE

• Store chemicals in designated, ventilated and bunded areas.

• Use flameproof cabinets for flammable materials.

• Segregate incompatible chemicals (acids, bases, oxidisers, flammables).

• Ensure clear GHS-compliant labelling of all containers.

• Maintain a Chemical Inventory List updated monthly.

7.1 Storage Requirements Based on Type

• Flammable: Fire-rated cabinet

• Corrosive: Corrosion-proof shelves, away from metal

• Reactive: Segregated by hazard class

• Dyes: Cool, dry, labelled area

8. CHEMICAL EVALUATION AND MONITORING

• Conduct regular evaluation of all chemicals to ensure MRSL compliance.

• Maintain records of: chemical inventory, usage logs, disposal records and SDS revisions.

• Perform periodic chemical audits (quarterly or after a change of supplier).

• Monitor environmental and occupational risks via the risk assessment matrix.

Evaluation tools:

• ZDHC InCheck Report

• Higg FEM Chemical Inventory Template

• ISO 14001 Risk & Opportunity Register

9. CHEMICAL WASTE AND DISPOSAL

• All chemical waste must be categorised and labelled as per local and ISO 14001 regulations.

• Maintain waste disposal logs with authorised waste handlers.

• Segregate hazardous and non-hazardous waste.

• No chemical waste shall be discharged into drains without treatment (via the ETP).

10. RECORDKEEPING

Maintain documentation for at least 5 years, including:

• SDS and COAs

• Chemical inventory and usage records

• Training logs

• Audit findings and corrective actions

• Supplier certifications (ZDHC, ISO-compliant)

11. REVIEW AND UPDATE

This procedure shall be reviewed:

• Annually

• After incidents involving chemicals

• When new chemicals or suppliers are introduced

• In response to updated ZDHC, ISO or Higg Index requirements

12. COMPLIANCE REFERENCES

• ZDHC MRSL v3.1 and Chemical Management Framework

• Egyptian requirements and legislation

• Higg FEM 4.0 – Chemical Management Section

• ISO 14001:2015 (Environmental Management Systems)

• ISO 45001:2018 (Occupational Health & Safety Management)

1. الغرض

وضع إجراء شامل لإدارة المواد الكيميائية يغطي شراء المواد الكيميائية المستخدمة في مصنع إيروغلو للتريكو (EMS) والتعامل معها وتخزينها وتقييمها. ويضمن هذا الإجراء الامتثال لقائمة المواد المقيدة في التصنيع (ZDHC MRSL)، ووحدة Higg البيئية للمنشآت (FEM)، ومعياري ISO 14001/45001، والمتطلبات والتشريعات المصرية فيما يخص الصحة والسلامة والأداء البيئي.

2. النطاق

ينطبق هذا الإجراء على جميع الإدارات والعاملين المشاركين في دورة حياة المواد الكيميائية، بما في ذلك:

• المشتريات

• مستودع المواد الكيميائية

• مناطق الإنتاج (الصباغة، الطباعة، الغسيل)

• محطة معالجة مياه الصرف (ETP)

• الصيانة والتنظيف

3. التعريفات

• برنامج التصريف الصفري للمواد الكيميائية الخطرة (ZDHC): مبادرة صناعية عالمية تهدف إلى التخلص من المواد الكيميائية الخطرة في سلسلة التوريد، ولا سيما في صناعات النسيج والملابس والجلود والأحذية.

• قائمة ZDHC MRSL: قائمة بالمواد الكيميائية المحظور استخدامها عمدًا لدى الموردين الذين يعالجون المواد النسيجية وأجزاء الإكسسوارات في النسيج والملابس والجلود والأحذية.

• صحيفة بيانات السلامة (SDS): وثيقة موحدة تقدم معلومات تفصيلية عن خصائص المادة الكيميائية أو الخليط ومخاطرها وطرق التعامل الآمن معها وتخزينها وتدابير الطوارئ المتعلقة بها.

• معدات الوقاية الشخصية (PPE): معدات أو ملابس متخصصة يرتديها الأفراد لحماية أنفسهم من المخاطر التي قد تسبب الإصابة أو المرض أو التعرض لمواد ضارة في مكان العمل أو البيئة.

• النظام المنسق عالميًا (GHS): معيار دولي لتصنيف المخاطر الكيميائية والإبلاغ عنها، يضمن نقل المعلومات المتعلقة بمخاطر المواد الكيميائية بشكل متسق من خلال الملصقات وصحائف بيانات السلامة (SDS) في جميع أنحاء العالم.

• وحدة Higg البيئية للمنشآت (Higg FEM): أداة معترف بها عالميًا لتقييم الاستدامة، مصممة لقياس الأداء البيئي لمنشآت التصنيع وتقييمه، وبصورة رئيسية في صناعات الملابس والأحذية والنسيج والسلع الاستهلاكية.

• مؤشرات الأداء الرئيسية لـ Higg FEM: مؤشرات قابلة للقياس تقيّم الأداء البيئي للمنشأة وتتابعه، وتساعد المنشآت على رصد التقدم وتحديد مجالات التحسين وإثبات أداء الاستدامة للعلامات التجارية وأصحاب المصلحة.

• شهادة التحليل (COA): وثيقة رسمية تصدر عن جهة مصنّعة أو مورد أو مختبر معتمد، تثبت جودة المنتج أو المادة أو المادة الكيميائية وتركيبها ومطابقتها للمعايير أو المواصفات المحددة.

4. المسؤوليات

4.1 فريق المشتريات

• شراء المواد الكيميائية من الموردين المعتمدين لدى ZDHC فقط.

• ضمان الامتثال لقائمة MRSL بالتعاون مع فريق الاستدامة.

4.2 مسؤول البيئة والصحة والسلامة (EHS)

• الإشراف على تطبيق هذا الإجراء.

• إجراء عمليات التدقيق الداخلي والتدريب.

4.3 مشرف المستودع

• ضمان الاستلام الآمن للمواد الكيميائية ووضع الملصقات عليها وتخزينها.

• الاحتفاظ بسجل المخزون.

4.4 مديرو الإنتاج

• ضمان الاستخدام الآمن للمواد الكيميائية والامتثال لصحائف بيانات السلامة.

4.5 فريق الاستدامة

• متابعة المطابقة لمتطلبات ZDHC ومؤشرات الأداء الرئيسية لـ Higg FEM.

• تحديث الوثائق.

• ضمان الامتثال لقائمة MRSL بالتعاون مع فريق المشتريات.

5. إجراء شراء المواد الكيميائية

• شراء المواد الكيميائية المدرجة فقط في قائمة المطابقة المعتمدة لـ MRSL على بوابة ZDHC.

• طلب أحدث صحائف بيانات السلامة (SDS) من المورد قبل الشراء.

• تقييم مؤهلات المورد (مثل تقارير ZDHC InCheck وCleanChain).

• الاحتفاظ بسجل مركزي لاعتماد المواد الكيميائية.

الوثائق المطلوبة:

• صحيفة بيانات السلامة (باللغة الإنجليزية واللغة المحلية)

• شهادة التحليل (COA)

• دليل المطابقة لمتطلبات ZDHC

• نموذج الاعتماد الداخلي للمادة الكيميائية

6. إجراء التعامل مع المواد الكيميائية

• لا يجوز التعامل مع المواد الكيميائية إلا للعاملين المدربين.

• يجب على جميع القائمين على التعامل ارتداء معدات الوقاية الشخصية المناسبة (القفازات، النظارات الواقية، المرايل، وغيرها).

• اتباع تعليمات التعامل الواردة في صحيفة بيانات السلامة.

• استخدام المعدات المخصصة لخلط المواد الكيميائية ونقلها وتخفيفها.

• يجب تعليق إجراءات الطوارئ (الانسكاب، التعرض) وأن يكون جميع القائمين على التعامل على علم بها.

موضوعات التدريب:

• رموز GHS وقراءة الملصقات

• الاستجابة للانسكاب والإسعافات الأولية

• جهات الاتصال في حالات الطوارئ وخطة الإخلاء

7. إجراء تخزين المواد الكيميائية

• تخزين المواد الكيميائية في مناطق مخصصة جيدة التهوية ومزودة بحواجز احتواء.

• استخدام خزائن مقاومة للحريق للمواد القابلة للاشتعال.

• فصل المواد الكيميائية غير المتوافقة (الأحماض، القواعد، المواد المؤكسدة، المواد القابلة للاشتعال).

• ضمان وضع ملصقات واضحة ومطابقة لنظام GHS على جميع الحاويات.

• الاحتفاظ بقائمة مخزون المواد الكيميائية وتحديثها شهريًا.

7.1 متطلبات التخزين حسب النوع

• المواد القابلة للاشتعال: خزانة مقاومة للحريق

• المواد الأكّالة: رفوف مقاومة للتآكل، بعيدًا عن المعادن

• المواد التفاعلية: تُفصل حسب فئة الخطر

• الأصباغ: منطقة باردة وجافة وموسومة بالملصقات

8. تقييم المواد الكيميائية ومراقبتها

• إجراء تقييم منتظم لجميع المواد الكيميائية لضمان الامتثال لقائمة MRSL.

• الاحتفاظ بسجلات: مخزون المواد الكيميائية، وسجلات الاستخدام، وسجلات التخلص، ومراجعات صحائف بيانات السلامة.

• إجراء عمليات تدقيق دورية للمواد الكيميائية (كل ثلاثة أشهر أو بعد تغيير المورد).

• مراقبة المخاطر البيئية والمهنية من خلال مصفوفة تقييم المخاطر.

أدوات التقييم:

• تقرير ZDHC InCheck

• نموذج مخزون المواد الكيميائية الخاص بـ Higg FEM

• سجل المخاطر والفرص وفق ISO 14001

9. النفايات الكيميائية والتخلص منها

• يجب تصنيف جميع النفايات الكيميائية ووضع الملصقات عليها وفقًا للوائح المحلية ومتطلبات ISO 14001.

• الاحتفاظ بسجلات التخلص من النفايات مع الجهات المرخصة للتعامل مع النفايات.

• فصل النفايات الخطرة عن غير الخطرة.

• يُحظر تصريف أي نفايات كيميائية في المصارف دون معالجة (عبر محطة معالجة مياه الصرف).

10. حفظ السجلات

الاحتفاظ بالوثائق لمدة 5 سنوات على الأقل، بما في ذلك:

• صحائف بيانات السلامة وشهادات التحليل

• سجلات مخزون المواد الكيميائية واستخدامها

• سجلات التدريب

• نتائج التدقيق والإجراءات التصحيحية

• شهادات الموردين (ZDHC، المطابقة لمعايير ISO)

11. المراجعة والتحديث

يجب مراجعة هذا الإجراء:

• سنويًا

• بعد الحوادث المتعلقة بالمواد الكيميائية

• عند إدخال مواد كيميائية أو موردين جدد

• استجابةً لتحديث متطلبات ZDHC أو ISO أو مؤشر Higg

12. المراجع التنظيمية

• قائمة ZDHC MRSL الإصدار 3.1 وإطار إدارة المواد الكيميائية

• المتطلبات والتشريعات المصرية

• Higg FEM 4.0 – قسم إدارة المواد الكيميائية

• ISO 14001:2015 (أنظمة الإدارة البيئية)

• ISO 45001:2018 (إدارة الصحة والسلامة المهنية)

ONAYLAYANLAR:

| Strateji ve Süreç Geliştirme Müdürü BAHAR YILDIRIM |  | İnsan Kaynakları Grup Müdürü ALİ KAAN GENÇ |  | Kalite Güvence Müdürü ABDULLAH BAYDOĞAN |  | EMS Genel Müdürü FATİH SALBARS |
| --- | --- | --- | --- | --- | --- | --- |