PURPOSE

This standard operating procedure defines the step-by-step method for applying 5S (Sort, Set in Order, Shine, Standardise, Sustain) and daily housekeeping in all areas of Eroglu Knitting (EMS), in order to provide a safe, clean and organised workplace, prevent product contamination and quality defects, and reduce fire risk and lost time.

SCOPE

This SOP applies to all production departments (knitting, dyeing and finishing, cutting, printing, embroidery, sewing, ironing and packing), warehouses, laboratory, maintenance workshop, utilities, offices, canteen, toilets and outdoor areas of the EMS facility.

RESPONSIBILITIES

• Factory Manager: Approves the 5S programme, appoints the 5S Coordinator and provides the resources (cleaning materials, labels, shelves, bins).

• 5S Coordinator (Quality Assurance): Prepares the 5S area map and zone owner list, organises 5S training, carries out monthly audits and publishes the scores.

• Department Managers and Zone Owners: Ensure daily 5S in their zone, close audit findings on time and keep the 5S board up to date.

• Line Supervisors / Team Leaders: Lead the 5-minute end-of-shift 5S and fill in the Daily 5S Checklist.

• Operators and All Employees: Keep their workstation clean and organised according to the area standard and report abnormalities.

• Housekeeping Team: Clean common areas, aisles, toilets and the canteen according to the Cleaning Schedule.

DEFINITIONS

• 5S: A workplace organisation method: Sort (Seiri), Set in Order (Seiton), Shine (Seiso), Standardise (Seiketsu) and Sustain (Shitsuke).

• Red Tag: A tag attached to an item whose need is unclear; the item is moved to the red tag area for a decision.

• Zone Owner: The person responsible for 5S in a defined area shown on the 5S area map.

• Visual Standard: Floor markings, shadow boards, labels and photos showing the correct state of an area.

MAIN AND SUB PROCESSES

1. PREPARATION

1.1. The 5S Coordinator divides the facility into zones, prepares the 5S area map and assigns a zone owner to each zone; the map is displayed on the department 5S board.

1.2. All employees receive 5S training during orientation and refresher training once a year; zone owners receive additional audit training.

1.3. Standard floor marking colours are applied: yellow for aisles and walkways, white for work in progress, red for nonconforming or quarantined product, and keep-clear zones in front of fire equipment, electrical panels and emergency exits.

2. SORT (SEIRI)

2.1. At the start of the programme and every six months, each zone owner checks all items in the zone together with the team.

2.2. Items not needed for current work (old samples, broken tools, empty cones, unused trims, personal items) receive a red tag and are moved to the red tag area.

2.3. Within 7 days the Department Manager decides for each red-tagged item: return to use, transfer to the warehouse, sell or recycle, or dispose of according to the waste procedure. The decision is recorded in the Red Tag Register.

3. SET IN ORDER (SEITON)

3.1. Every needed item is given a fixed, labelled place close to the point of use; tools are kept on shadow boards and scissors are tied to the workstation.

3.2. Bundles, cut panels and WIP trolleys are kept only inside the marked WIP areas and never on aisles or on the floor.

3.3. Fire extinguishers, hydrants, electrical panels, eye-wash stations and emergency exits are kept clear at all times (at least 1 metre).

4. SHINE (SEISO)

4.1. Every operator applies the 5-minute end-of-shift cleaning of the machine, table, chair and floor around the workstation; lint is removed with a brush or vacuum cleaner, never by blowing compressed air towards people.

4.2. The housekeeping team cleans aisles, toilets, the canteen and drinking water points according to the Cleaning Schedule and signs the cleaning record.

4.3. Oil leaks, loose cables, damaged guards or other abnormalities noticed during cleaning are reported to the supervisor and to maintenance.

4.4. Lint, fabric waste, needles and chemical containers are collected separately according to EMS.QA.PRO.02.01., EMS.QA.PRO.02.07. and EMS.ÇV.TL.19.01.

5. STANDARDISE (SEIKETSU)

5.1. For each area a 5S Standard Sheet with photos of the correct state is prepared and displayed in English and Arabic.

5.2. The supervisor fills in the Daily 5S Checklist at the end of every shift.

6. SUSTAIN (SHITSUKE) AND AUDITS

6.1. The 5S Coordinator audits each zone every month using the 5S Audit Form (score 0–100%); results are displayed on the 5S boards.

6.2. Findings are given to the zone owner with a deadline (critical safety findings within 24 hours, others within 7 days) and are verified at the next audit.

6.3. The best-scoring zone of the month is announced and recognised.

6.4. The following indicators are monitored every month:

- Average 5S audit score per department (target ≥ 85%),

- Number of open findings past their due date,

- Number of blocked fire equipment, exits or aisles found (target: zero).

7. RELATED DOCUMENTS AND RECORDS

- 5S Area Map and Zone Owner List

- Daily 5S Checklist, Cleaning Schedule and Cleaning Records

- 5S Audit Form and Red Tag Register

- EMS.QA.PRO.02.01. Lint Waste Procedure

- EMS.QA.PRO.02.07. Needle and Sharp Tools Control Procedure

- EMS.ÇV.TL.19.01. Waste Segregation Instruction

- EMS.SS.PRO.10.01. Fire Prevention and Firefighting Equipment Control Procedure

- EMS.SO.POL.23.01. Standard Operating Procedures Policy

8. FEEDBACK AND MONITORING

5S results are reviewed every month in the production meeting and every year in the management review. This SOP is reviewed every year by the 5S Coordinator and revised when necessary.

الغرض

يحدد إجراء التشغيل القياسي هذا الطريقة خطوة بخطوة لتطبيق نظام 5S (الفرز، والترتيب، والتنظيف، والتوحيد القياسي، والاستدامة) وأعمال النظافة اليومية في جميع مناطق شركة إيروغلو للتريكو (EMS)، وذلك لتوفير مكان عمل آمن ونظيف ومنظم، ومنع تلوث المنتج وعيوب الجودة، وتقليل خطر الحريق والوقت الضائع.

النطاق

ينطبق هذا الإجراء على جميع أقسام الإنتاج (التريكو، والصباغة والتجهيز، والقص، والطباعة، والتطريز، والخياطة، والكي والتعبئة)، والمخازن، والمختبر، وورشة الصيانة، والمرافق، والمكاتب، والمطعم، ودورات المياه، والمناطق الخارجية في منشأة EMS.

المسؤوليات

• مدير المصنع: يعتمد برنامج 5S، ويعيّن منسق 5S، ويوفر الموارد اللازمة (مواد التنظيف والملصقات والرفوف والحاويات).

• منسق 5S (ضمان الجودة): يعد خريطة مناطق 5S وقائمة مسؤولي المناطق، وينظم تدريب 5S، وينفذ التدقيقات الشهرية وينشر النتائج.

• مديرو الأقسام ومسؤولو المناطق: يضمنون تطبيق 5S يوميًا في مناطقهم، ويغلقون ملاحظات التدقيق في موعدها، ويحدّثون لوحة 5S باستمرار.

• مشرفو الخطوط / قادة الفرق: يقودون نشاط 5S لمدة 5 دقائق في نهاية الوردية، ويملؤون قائمة فحص 5S اليومية.

• المشغلون وجميع الموظفين: يحافظون على نظافة محطة عملهم وتنظيمها وفق معيار المنطقة، ويبلغون عن أي خلل.

• فريق النظافة: ينظف المناطق المشتركة والممرات ودورات المياه والمطعم وفق جدول النظافة.

التعريفات

• 5S: أسلوب لتنظيم مكان العمل يتكون من: الفرز (Seiri)، والترتيب (Seiton)، والتنظيف (Seiso)، والتوحيد القياسي (Seiketsu)، والاستدامة (Shitsuke).

• البطاقة الحمراء: بطاقة تُثبت على أي غرض غير مؤكد الحاجة إليه، ويُنقل الغرض إلى منطقة البطاقات الحمراء لاتخاذ قرار بشأنه.

• مسؤول المنطقة: الشخص المسؤول عن 5S في منطقة محددة موضحة على خريطة مناطق 5S.

• المعيار المرئي: علامات الأرضيات ولوحات الأدوات المظللة والملصقات والصور التي توضح الحالة الصحيحة للمنطقة.

العمليات الرئيسية والفرعية

1. التحضير

1.1. يقسم منسق 5S المنشأة إلى مناطق، ويعد خريطة مناطق 5S، ويعيّن مسؤولًا لكل منطقة؛ وتُعرض الخريطة على لوحة 5S في القسم.

1.2. يتلقى جميع الموظفين تدريب 5S أثناء التوجيه وتدريبًا تنشيطيًا مرة كل عام، ويتلقى مسؤولو المناطق تدريبًا إضافيًا على التدقيق.

1.3. تُطبق ألوان قياسية لعلامات الأرضيات: الأصفر للممرات ومسارات المشاة، والأبيض للإنتاج تحت التشغيل، والأحمر للمنتج غير المطابق أو المحجوز، مع مناطق يجب إبقاؤها خالية أمام معدات الحريق واللوحات الكهربائية ومخارج الطوارئ.

2. الفرز (SEIRI)

2.1. في بداية البرنامج وكل ستة أشهر، يفحص كل مسؤول منطقة جميع الأغراض الموجودة في منطقته مع فريقه.

2.2. توضع بطاقة حمراء على الأغراض غير اللازمة للعمل الحالي (العينات القديمة والأدوات المكسورة والمخاريط الفارغة والإكسسوارات غير المستخدمة والأغراض الشخصية) وتُنقل إلى منطقة البطاقات الحمراء.

2.3. خلال 7 أيام يقرر مدير القسم بشأن كل غرض عليه بطاقة حمراء: إعادته للاستخدام، أو نقله إلى المخزن، أو بيعه أو إعادة تدويره، أو التخلص منه وفق إجراء النفايات. ويُسجل القرار في سجل البطاقات الحمراء.

3. الترتيب (SEITON)

3.1. يُخصص لكل غرض لازم مكان ثابت عليه ملصق بالقرب من مكان استخدامه؛ وتُحفظ الأدوات على لوحات الأدوات المظللة، وتُربط المقصات بمحطة العمل.

3.2. تُحفظ الحزم والقطع المقصوصة وعربات الإنتاج تحت التشغيل داخل المناطق المحددة لها فقط، ولا توضع أبدًا في الممرات أو على الأرض.

3.3. تبقى طفايات الحريق وحنفيات الحريق واللوحات الكهربائية ومحطات غسل العيون ومخارج الطوارئ خالية في جميع الأوقات (مسافة متر واحد على الأقل).

4. التنظيف (SEISO)

4.1. يطبق كل مشغل التنظيف لمدة 5 دقائق في نهاية الوردية للآلة والطاولة والكرسي والأرضية حول محطة العمل؛ ويُزال الوبر بالفرشاة أو المكنسة الكهربائية، ولا يُنفخ الهواء المضغوط أبدًا باتجاه الأشخاص.

4.2. ينظف فريق النظافة الممرات ودورات المياه والمطعم ونقاط مياه الشرب وفق جدول النظافة، ويوقع على سجل النظافة.

4.3. يُبلغ المشرف والصيانة عن تسربات الزيت أو الكابلات المرتخية أو الواقيات التالفة أو أي خلل آخر يُلاحظ أثناء التنظيف.

4.4. يُجمع الوبر ونفايات القماش والإبر والعبوات الكيميائية بشكل منفصل وفق EMS.QA.PRO.02.01. وEMS.QA.PRO.02.07. وEMS.ÇV.TL.19.01.

5. التوحيد القياسي (SEIKETSU)

5.1. تُعد لكل منطقة ورقة معيار 5S تتضمن صورًا للحالة الصحيحة، وتُعرض باللغتين الإنجليزية والعربية.

5.2. يملأ المشرف قائمة فحص 5S اليومية في نهاية كل وردية.

6. الاستدامة (SHITSUKE) والتدقيقات

6.1. يدقق منسق 5S كل منطقة شهريًا باستخدام نموذج تدقيق 5S (درجة من 0 إلى 100%)، وتُعرض النتائج على لوحات 5S.

6.2. تُسلم الملاحظات إلى مسؤول المنطقة مع موعد نهائي (ملاحظات السلامة الحرجة خلال 24 ساعة، والباقي خلال 7 أيام)، ويُتحقق منها في التدقيق التالي.

6.3. يُعلن عن المنطقة الحاصلة على أعلى درجة في الشهر ويتم تكريمها.

6.4. تُتابع المؤشرات التالية شهريًا:

- متوسط درجة تدقيق 5S لكل قسم (الهدف ≥ 85%)،

- عدد الملاحظات المفتوحة التي تجاوزت موعدها النهائي،

- عدد حالات إعاقة معدات الحريق أو المخارج أو الممرات المكتشفة (الهدف: صفر).

7. الوثائق والسجلات ذات الصلة

- خريطة مناطق 5S وقائمة مسؤولي المناطق

- قائمة فحص 5S اليومية وجدول النظافة وسجلات النظافة

- نموذج تدقيق 5S وسجل البطاقات الحمراء

- EMS.QA.PRO.02.01. إجراء نفايات الوبر

- EMS.QA.PRO.02.07. إجراء التحكم في الإبر والأدوات الحادة

- EMS.ÇV.TL.19.01. تعليمات فرز النفايات

- EMS.SS.PRO.10.01. إجراء الوقاية من الحريق والتحكم في معدات مكافحة الحريق

- EMS.SO.POL.23.01. سياسة إجراءات التشغيل القياسية

8. التغذية الراجعة والمتابعة

تُراجع نتائج 5S شهريًا في اجتماع الإنتاج وسنويًا في مراجعة الإدارة. ويُراجع هذا الإجراء سنويًا من قبل منسق 5S ويُعدل عند الحاجة.

ONAYLAYANLAR:

| Strateji ve Süreç Geliştirme Müdürü BAHAR YILDIRIM |  | İnsan Kaynakları Grup Müdürü ALİ KAAN GENÇ |  | Kalite Güvence Müdürü ABDULLAH BAYDOGAN |  | EMS Genel Müdürü FATİH SALBARS |
| --- | --- | --- | --- | --- | --- | --- |