1. FACILITY PROFILE

This plan applies to the following areas of Eroglu Knitting (EMS):

• Knitting Department: low water, high energy consumption.

• Dyeing Department: very high water and energy consumption (critical area).

• Printing Department: moderate water consumption, chemical intensive.

• Garment (T-shirt) Manufacturing: low water, medium energy consumption.

• Utilities: boilers, cooling, ETP.

2. KEY ENVIRONMENTAL RISKS (AUDIT FOCUS)

2.1 High-Risk Areas

• Dyeing: highest water and energy consumption.

• Printing: chemical discharge risk (ZDHC critical).

• ETP: compliance and reuse.

• Boilers: energy efficiency.

3. WATER MANAGEMENT PLAN (PROCESS-BASED)

3.1 Knitting Department

• Characteristics: Low water consumption.

• Action: Optimize humidification systems.

• Action: Prevent unnecessary use of cleaning water.

• Impact: Minor, but important for Higg scoring.

3.2 Dyeing Department (Critical Area)

Main issues:

• High liquor ratio

• High reprocessing rate

• High wastewater load

Actions:

• Reduce the liquor ratio (target: ≤ 1:6 if possible).

• Use low-liquor-ratio dyeing machines.

• Optimize batch size.

• Improve Right-First-Time (RFT ≥ 90%).

• Use automatic dosing systems.

Water KPIs:

• m³ per kg of fabric: to be reduced

• Re-dyeing rate: to be reduced

3.3 Printing Department

Main issues:

• Screen washing

• Chemical residues

Actions:

• Use high-pressure, low-volume cleaning.

• Reuse wash water (if applicable).

• Control chemical usage (ZDHC MRSL compliance).

3.4 Garment (T-shirt) Section

Actions:

• Optimize washing processes.

• Reduce cleaning water.

• Improve housekeeping.

3.5 Utilities and ETP (Core of Savings)

• Before mid-2026 (no ETP yet): Monitoring and control only; limited savings.

• After the ETP (mid-2026 onwards): Treated water reuse applications: cooling towers, floor cleaning, dyeing (if quality allows).

Reuse rate targets:

• 2026: 25%

• 2027: 50%

• 2028: 60%

3.6 Water Balance (Mandatory for Higg Level 3)

The water balance shall cover:

• Input sources

• Process consumption

• Losses

• Reuse streams

4. ENERGY MANAGEMENT PLAN (PROCESS-BASED)

4.1 Knitting

• Optimize machine efficiency.

• Use energy-efficient motors.

4.2 Dyeing (Highest Energy Use)

Actions:

• Optimize steam consumption.

• Insulate dyeing machines.

• Reduce process time.

• Recover heat from hot effluent.

4.3 Printing

• Optimize drying ovens.

• Control curing temperatures.

4.4 Garment Section

• Use efficient sewing machines.

• Use LED lighting.

4.5 Boilers and Utilities (Critical)

Actions:

• Boiler efficiency ≥ 85%.

• Steam trap maintenance.

• Condensate recovery system.

• Insulation of pipelines.

5. SOLAR ENERGY INTEGRATION PLAN

• Objective: By 2027, following the completion of construction works, develop and finalize a plan for integrating solar energy into the facility's operations.

Priority areas for implementation:

• Knitting machines

• Lighting systems

• Air compressors

• Administration buildings

• Note: The dyeing process will continue to rely primarily on thermal energy (steam) due to its high heat demand.

Implementation timeline:

• 2026: Propose the concept and conduct an initial feasibility assessment.

• 2027: Perform a detailed feasibility study and system design.

• 2028: Install the first phase, targeting 10–15% of the facility's electricity demand.

• 2029: Expand the system to supply approximately 30% of the facility's electricity demand.

6. ZDHC COMPLIANCE (VERY IMPORTANT)

Requirements:

• Wastewater testing (monthly)

• Upload of results to the ZDHC Gateway

• Sludge disposal records

• Chemical inventory (MRSL compliant)

7. KPI SYSTEM (AUDIT-CRITICAL)

• Water consumption (m³/kg): Target: reduction; Department: Dyeing

• Reuse rate: Target: 50–60%; Department: ETP

• Energy (kWh/kg): Target: reduction; Department: All

• Boiler efficiency: Target: ≥ 85%; Department: Utilities

• RFT %: Target: ≥ 90%; Department: Dyeing

• Wastewater compliance: Target: 100%; Department: ETP

8. MONITORING SYSTEM

• Daily tracking (mandatory)

• Monthly analysis

• Department-wise reporting

• Linking consumption with production

9. DOCUMENTATION (FOR AUDIT READINESS)

Documentation must include:

• Water balance diagram

• Energy consumption records

• KPI reports

• ZDHC test reports

• Maintenance logs

• Training records

10. TRAINING PROGRAM

• Operators (dyeing and ETP focus)

• Maintenance team

• Awareness for all employees

11. INTERNAL AUDIT SYSTEM

• Quarterly audits

• Gap analysis against: Higg FEM, GRS / GOTS, OEKO-TEX and ZDHC

12. TIMELINE

• Until mid-2026: Monitoring and system setup

• Mid-2026 to 2027: ETP and major water savings

• 2026 to 2029: Optimization and solar expansion

13. EXPECTED RESULTS

• Dyeing: Largest water and energy reduction.

• ETP: Main driver for compliance and reuse.

• Solar: Reduces electricity cost (not steam).

• Overall: Strong audit performance.

13.1 Key Insight

• 80% of water savings will come from: dyeing optimization and ETP reuse.

• 80% of energy savings will come from: boilers and dyeing processes.

1. لمحة عن المنشأة

تنطبق هذه الخطة على المناطق التالية في إيروغلو للتريكو (EMS):

• قسم التريكو: استهلاك منخفض للمياه ومرتفع للطاقة.

• قسم الصباغة: استهلاك مرتفع جدًا للمياه والطاقة (منطقة حرجة).

• قسم الطباعة: استهلاك متوسط للمياه، واستخدام مكثف للمواد الكيميائية.

• تصنيع الملابس (التي شيرت): استهلاك منخفض للمياه ومتوسط للطاقة.

• المرافق: الغلايات، التبريد، محطة معالجة مياه الصرف (ETP).

2. المخاطر البيئية الرئيسية (محور التدقيق)

2.1 المناطق عالية الخطورة

• الصباغة: أعلى استهلاك للمياه والطاقة.

• الطباعة: خطر تصريف المواد الكيميائية (حرج وفق ZDHC).

• محطة المعالجة (ETP): الامتثال وإعادة الاستخدام.

• الغلايات: كفاءة الطاقة.

3. خطة إدارة المياه (حسب العمليات)

3.1 قسم التريكو

• الخصائص: استهلاك منخفض للمياه.

• إجراء: تحسين أنظمة الترطيب.

• إجراء: منع الاستخدام غير الضروري لمياه التنظيف.

• الأثر: محدود، لكنه مهم لتقييم Higg.

3.2 قسم الصباغة (منطقة حرجة)

المشكلات الرئيسية:

• ارتفاع نسبة الحمام (نسبة السائل إلى القماش)

• ارتفاع معدل إعادة المعالجة

• ارتفاع حمل مياه الصرف

الإجراءات:

• خفض نسبة الحمام (الهدف: ≤ 1:6 إن أمكن).

• استخدام ماكينات صباغة منخفضة نسبة الحمام.

• تحسين حجم الدفعة.

• تحسين نسبة الصواب من المرة الأولى (RFT ≥ 90%).

• استخدام أنظمة الجرعات الأوتوماتيكية.

مؤشرات أداء المياه:

• م³ لكل كجم من القماش: يجب خفضه

• معدل إعادة الصباغة: يجب خفضه

3.3 قسم الطباعة

المشكلات الرئيسية:

• غسيل الشاشات

• المخلفات الكيميائية المتبقية

الإجراءات:

• استخدام التنظيف بالضغط العالي والحجم المنخفض من المياه.

• إعادة استخدام مياه الغسيل (حيثما ينطبق).

• التحكم في استخدام المواد الكيميائية (الامتثال لقائمة ZDHC MRSL).

3.4 قسم الملابس (التي شيرت)

الإجراءات:

• تحسين عمليات الغسيل.

• تقليل مياه التنظيف.

• تحسين النظافة والترتيب.

3.5 المرافق ومحطة المعالجة (جوهر التوفير)

• قبل منتصف 2026 (قبل وجود محطة المعالجة): المراقبة والتحكم فقط؛ توفير محدود.

• بعد تشغيل محطة المعالجة (من منتصف 2026 فصاعدًا): تطبيقات إعادة استخدام المياه المعالجة: أبراج التبريد، تنظيف الأرضيات، الصباغة (إذا سمحت الجودة).

أهداف معدل إعادة الاستخدام:

• 2026: 25%

• 2027: 50%

• 2028: 60%

3.6 الميزان المائي (إلزامي للمستوى 3 في Higg)

يجب أن يغطي الميزان المائي:

• مصادر المياه الواردة

• استهلاك العمليات

• الفواقد

• تيارات إعادة الاستخدام

4. خطة إدارة الطاقة (حسب العمليات)

4.1 التريكو

• تحسين كفاءة الماكينات.

• استخدام محركات موفرة للطاقة.

4.2 الصباغة (أعلى استخدام للطاقة)

الإجراءات:

• تحسين استهلاك البخار.

• عزل ماكينات الصباغة.

• تقليل زمن العملية.

• استرداد الحرارة من مياه الصرف الساخنة.

4.3 الطباعة

• تحسين أفران التجفيف.

• التحكم في درجات حرارة التثبيت.

4.4 قسم الملابس

• استخدام ماكينات خياطة عالية الكفاءة.

• استخدام إضاءة LED.

4.5 الغلايات والمرافق (حرجة)

الإجراءات:

• كفاءة الغلايات ≥ 85%.

• صيانة مصائد البخار.

• نظام استرداد المتكثفات.

• عزل خطوط الأنابيب.

5. خطة دمج الطاقة الشمسية

• الهدف: بحلول عام 2027، وبعد الانتهاء من أعمال الإنشاء، إعداد خطة لدمج الطاقة الشمسية في عمليات المنشأة واستكمالها.

مجالات التطبيق ذات الأولوية:

• ماكينات التريكو

• أنظمة الإضاءة

• ضواغط الهواء

• المباني الإدارية

• ملاحظة: ستظل عملية الصباغة تعتمد بشكل أساسي على الطاقة الحرارية (البخار) نظرًا لارتفاع احتياجها للحرارة.

الجدول الزمني للتنفيذ:

• 2026: طرح الفكرة وإجراء تقييم أولي للجدوى.

• 2027: إجراء دراسة جدوى تفصيلية وتصميم النظام.

• 2028: تركيب المرحلة الأولى بهدف تغطية 10–15% من الطلب على الكهرباء في المنشأة.

• 2029: توسيع النظام لتغطية نحو 30% من الطلب على الكهرباء في المنشأة.

6. الامتثال لمتطلبات ZDHC (مهم جدًا)

المتطلبات:

• اختبار مياه الصرف (شهريًا)

• رفع النتائج إلى بوابة ZDHC Gateway

• سجلات التخلص من الحمأة

• جرد المواد الكيميائية (متوافق مع MRSL)

7. نظام مؤشرات الأداء الرئيسية (حرج للتدقيق)

• استهلاك المياه (م³/كجم): الهدف: الخفض؛ القسم: الصباغة

• معدل إعادة الاستخدام: الهدف: 50–60%؛ القسم: محطة المعالجة (ETP)

• الطاقة (كيلوواط ساعة/كجم): الهدف: الخفض؛ القسم: جميع الأقسام

• كفاءة الغلايات: الهدف: ≥ 85%؛ القسم: المرافق

• نسبة RFT: الهدف: ≥ 90%؛ القسم: الصباغة

• امتثال مياه الصرف: الهدف: 100%؛ القسم: محطة المعالجة (ETP)

8. نظام المراقبة

• المتابعة اليومية (إلزامية)

• التحليل الشهري

• التقارير على مستوى الأقسام

• ربط الاستهلاك بالإنتاج

9. التوثيق (للجاهزية للتدقيق)

يجب أن تشمل الوثائق:

• مخطط الميزان المائي

• سجلات استهلاك الطاقة

• تقارير مؤشرات الأداء الرئيسية

• تقارير اختبارات ZDHC

• سجلات الصيانة

• سجلات التدريب

10. برنامج التدريب

• المشغلون (مع التركيز على الصباغة ومحطة المعالجة)

• فريق الصيانة

• التوعية لجميع الموظفين

11. نظام التدقيق الداخلي

• عمليات تدقيق ربع سنوية

• تحليل الفجوات مقارنةً بـ: Higg FEM وGRS / GOTS وOEKO-TEX وZDHC

12. الجدول الزمني

• حتى منتصف 2026: المراقبة وإعداد الأنظمة

• من منتصف 2026 إلى 2027: محطة المعالجة وتحقيق وفورات كبيرة في المياه

• من 2026 إلى 2029: التحسين والتوسع في الطاقة الشمسية

13. النتائج المتوقعة

• الصباغة: أكبر خفض في المياه والطاقة.

• محطة المعالجة (ETP): المحرك الرئيسي للامتثال وإعادة الاستخدام.

• الطاقة الشمسية: تخفض تكلفة الكهرباء (وليس البخار).

• بشكل عام: أداء قوي في عمليات التدقيق.

13.1 ملاحظة رئيسية

• 80% من وفورات المياه ستتحقق من خلال: تحسين عمليات الصباغة وإعادة الاستخدام عبر محطة المعالجة.

• 80% من وفورات الطاقة ستتحقق من خلال: الغلايات وعمليات الصباغة.

ONAYLAYANLAR:

| Strateji ve Süreç Geliştirme Müdürü BAHAR YILDIRIM |  | İnsan Kaynakları Grup Müdürü ALİ KAAN GENÇ |  | Kalite Güvence Müdürü ABDULLAH BAYDOĞAN |  | EMS Genel Müdürü FATİH SALBARS |
| --- | --- | --- | --- | --- | --- | --- |