1. PURPOSE

To establish a rigorous, standardized and auditable procedure for reading utility meters that ensures:

• 100% accurate and repeatable readings.

• Full traceability between recorded data and physical meters.

• Elimination of human error, estimation and manipulation.

• Safe reading practices (especially for gas and electrical systems).

• Compliance with international standards and brand audits.

2. SCOPE

This procedure applies strictly to meter reading activities only, for all utility meters in the Eroglu Knitting (EMS) facility:

2.1 Water

• Main incoming water meter.

• Dyeing machines.

• Boilers.

• Cooling systems.

• ETP (inlet and outlet).

• Domestic water.

2.2 Electricity

• Main incomer.

• Submeters (spinning, knitting, dyeing, printing, garment).

• Compressors.

• HVAC systems.

• Lighting panels.

2.3 Natural Gas

• Main gas meter.

• Boiler gas consumption meters.

• Submeters (if available).

3. COMPLIANCE FRAMEWORK

This procedure ensures compliance with:

• Higg Facility Environmental Module (accurate tracking and verification).

• ISO 14001 Clause 9.1 (Monitoring and Measurement).

• Global brand requirements.

• GOTS and GRS.

4. DEFINITIONS

• Meter Reading: Direct recording of the displayed meter value without calculation.

• Cumulative Meter: Meter with a continuously increasing value.

• Submeter: Meter assigned to a specific process.

5. RESPONSIBILITIES

• Operator: Perform readings, ensure accuracy, record immediately.

• Supervisor: Verify readings, detect abnormalities.

• Maintenance: Ensure meters are functional, calibrated and accessible.

• Sustainability Department: Audit compliance and record integrity.

6. REQUIRED TOOLS, EQUIPMENT AND PPE (MANDATORY)

6.1 Recording and Documentation Tools

• Approved Meter Reading Log Sheet (hard copy or tablet).

• Permanent pen (no pencil allowed).

• Calculator (for quick validation if needed).

• Mobile device (photo evidence, recommended for audit).

6.2 Inspection Tools

• Flashlight (for dark areas).

• Cleaning cloth (for display clarity).

6.3 Personal Protective Equipment (PPE)

• General areas: Safety shoes; gloves.

• Electrical areas: Insulated gloves; arc-rated PPE (if required).

• Gas areas (critical): Portable gas detector; anti-static clothing; safety helmet.

6.4 Access Equipment

• Approved ladder.

• Access permit (if restricted area).

7. PRE-READING CONTROL CHECK (CRITICAL STEP)

Before reading any meter, the operator must confirm:

7.1 Identification

• Correct meter ID.

• Correct location.

• Correct resource (water, electricity or gas).

• Unit of measurement.

7.2 Physical Condition

• Display is readable.

• No cracks or damage.

• No signs of tampering.

7.3 Safety Verification

• Water: No leakage.

• Electricity: No exposed wires; no alarm indicators.

• Gas: No gas odor; no leakage sound; area ventilated.

Warning: If unsafe, STOP and report.

8. STANDARD READING CONDITIONS

• Reading time must be fixed daily (e.g. 08:00 AM).

• Allowed variation: ±15 minutes.

• Same sequence of meters daily.

• Same operator preferred.

9. STEP-BY-STEP METER READING PROCEDURE

Step 1: Safe Access

• Ensure a safe approach.

• Wear the required PPE.

• Use a ladder if needed.

Step 2: Clean and Prepare

• Clean the meter display if dirty.

• Ensure visibility.

Step 3: Visual Inspection

Check:

• Display clarity.

• No damage.

• No leakage (water or gas).

• No abnormal vibration or noise.

Step 4: Read Meter Value

• Digital meter: Record the full value (including decimals if required).

• Analog meter: Record all digits from left to right; ignore the red/rotating dial unless required.

Step 5: Immediate Recording (Mandatory)

• Record the reading immediately on-site.

• No memorization allowed.

Step 6: Recording Format

• Date.

• Time.

• Meter ID.

• Resource.

• Area.

• Previous reading.

• Current reading.

• Unit.

• Operator.

• Remarks.

Step 7: On-Site Validation

The operator must check:

• Current reading ≥ previous reading.

• Logical increase.

• No abnormal jump.

If abnormal, note it in the remarks.

10. RESOURCE-SPECIFIC INSTRUCTIONS

10.1 Water

• Unit: m³.

• Check for nearby leakage.

• Ensure a continuous increase.

10.2 Electricity

• Unit: kWh.

• Check display stability.

• No error codes.

10.3 Natural Gas (High Risk)

• Additional safety checks: No gas smell; no leakage sound; stable pressure.

• Reading rules: Unit m³ or Nm³; record the exact value (no rounding).

11. ERROR PREVENTION CONTROLS

Operators must avoid:

• Skipping meters.

• Estimating values.

• Recording from memory.

• Wrong unit entry.

• Digit misplacement.

12. ABNORMAL READING PROTOCOL

12.1 Trigger Conditions

• Sudden increase (>10%).

• No change.

• Unexpected decrease.

12.2 Required Actions

• Re-check the reading.

• Confirm the meter ID.

• Inform the supervisor immediately.

• Record in the remarks.

• Do NOT modify data.

13. RECORD CONTROL AND INTEGRITY

• No overwriting allowed.

• Corrections must be crossed out once and signed by the operator and the supervisor.

• Records must be clear, complete and traceable.

14. DOCUMENTATION REQUIREMENTS

Maintain:

• Daily reading sheets.

• Signed verification records.

• Meter register.

• Photo evidence (optional but recommended).

15. INTERNAL AUDIT REQUIREMENTS

Auditors will verify:

• Daily readings without gaps.

• Same reading time.

• Consistency.

• Inclusion of gas readings.

• Traceability to physical meters.

16. TRAINING REQUIREMENTS

All personnel must be trained on:

• Meter types and units.

• Reading accuracy.

• Safety (gas and electrical).

• Error prevention.

17. NON-CONFORMITY MANAGEMENT

• Missing reading: Immediate correction.

• Wrong data: Retraining.

• Unsafe reading: Stop work immediately.

1. الغرض

وضع إجراء صارم وموحد وقابل للتدقيق لقراءة عدادات المرافق يضمن ما يلي:

• قراءات دقيقة بنسبة 100% وقابلة للتكرار.

• إمكانية تتبع كاملة بين البيانات المسجلة والعدادات الفعلية.

• القضاء على الخطأ البشري والتقدير والتلاعب.

• ممارسات قراءة آمنة (خاصة لأنظمة الغاز والكهرباء).

• الامتثال للمعايير الدولية وتدقيقات العلامات التجارية.

2. النطاق

ينطبق هذا الإجراء حصريًا على أنشطة قراءة العدادات فقط، لجميع عدادات المرافق في منشأة إيروغلو للتريكو (EMS):

2.1 المياه

• عداد المياه الرئيسي الوارد.

• ماكينات الصباغة.

• الغلايات.

• أنظمة التبريد.

• محطة معالجة الصرف الصناعي ETP (المدخل والمخرج).

• المياه المنزلية (الاستخدام الصحي).

2.2 الكهرباء

• المغذي الرئيسي الوارد.

• العدادات الفرعية (الغزل، التريكو، الصباغة، الطباعة، الملابس الجاهزة).

• الضواغط.

• أنظمة التدفئة والتهوية وتكييف الهواء (HVAC).

• لوحات الإضاءة.

2.3 الغاز الطبيعي

• عداد الغاز الرئيسي.

• عدادات استهلاك الغاز للغلايات.

• العدادات الفرعية (إن وجدت).

3. إطار الامتثال

يضمن هذا الإجراء الامتثال لما يلي:

• وحدة Higg البيئية للمنشآت (FEM) (التتبع والتحقق الدقيق).

• البند 9.1 من ISO 14001 (الرصد والقياس).

• متطلبات العلامات التجارية العالمية.

• GOTS وGRS.

4. التعاريف

• قراءة العداد: التسجيل المباشر للقيمة المعروضة على العداد دون إجراء أي حساب.

• العداد التراكمي: عداد تتزايد قيمته باستمرار.

• العداد الفرعي: عداد مخصص لعملية محددة.

5. المسؤوليات

• المشغل: إجراء القراءات وضمان دقتها وتسجيلها فورًا.

• المشرف: التحقق من القراءات واكتشاف الحالات غير الطبيعية.

• الصيانة: ضمان أن العدادات تعمل ومعايرة ويمكن الوصول إليها.

• إدارة الاستدامة: تدقيق الامتثال وسلامة السجلات.

6. الأدوات والمعدات ومعدات الوقاية الشخصية المطلوبة (إلزامي)

6.1 أدوات التسجيل والتوثيق

• سجل قراءة العدادات المعتمد (نسخة ورقية أو جهاز لوحي).

• قلم حبر ثابت (يُمنع استخدام القلم الرصاص).

• آلة حاسبة (للتحقق السريع عند الحاجة).

• جهاز محمول (لأدلة مصورة، يوصى به لأغراض التدقيق).

6.2 أدوات الفحص

• مصباح يدوي (للمناطق المظلمة).

• قطعة قماش للتنظيف (لوضوح الشاشة).

6.3 معدات الوقاية الشخصية

• المناطق العامة: أحذية السلامة؛ القفازات.

• المناطق الكهربائية: قفازات عازلة؛ معدات وقاية مقاومة للقوس الكهربائي (عند الحاجة).

• مناطق الغاز (حرجة): كاشف غاز محمول؛ ملابس مضادة للكهرباء الساكنة؛ خوذة السلامة.

6.4 معدات الوصول

• سلم معتمد.

• تصريح دخول (إذا كانت المنطقة مقيدة).

7. فحص الضبط قبل القراءة (خطوة حرجة)

قبل قراءة أي عداد، يجب على المشغل التأكد مما يلي:

7.1 التعريف

• رقم تعريف العداد الصحيح.

• الموقع الصحيح.

• المورد الصحيح (المياه أو الكهرباء أو الغاز).

• وحدة القياس.

7.2 الحالة المادية

• الشاشة قابلة للقراءة.

• عدم وجود شقوق أو تلف.

• عدم وجود علامات عبث.

7.3 التحقق من السلامة

• المياه: عدم وجود تسرب.

• الكهرباء: عدم وجود أسلاك مكشوفة؛ عدم وجود مؤشرات إنذار.

• الغاز: عدم وجود رائحة غاز؛ عدم وجود صوت تسرب؛ المنطقة جيدة التهوية.

تحذير: إذا كانت الظروف غير آمنة، توقف وأبلغ فورًا.

8. شروط القراءة القياسية

• يجب أن يكون وقت القراءة ثابتًا يوميًا (مثلًا الساعة 08:00 صباحًا).

• التفاوت المسموح به: ±15 دقيقة.

• نفس تسلسل العدادات يوميًا.

• يُفضَّل أن يكون المشغل نفسه.

9. إجراء قراءة العدادات خطوة بخطوة

الخطوة 1: الوصول الآمن

• ضمان الاقتراب الآمن.

• ارتداء معدات الوقاية الشخصية المطلوبة.

• استخدام سلم عند الحاجة.

الخطوة 2: التنظيف والتحضير

• تنظيف شاشة العداد إذا كانت متسخة.

• ضمان وضوح الرؤية.

الخطوة 3: الفحص البصري

افحص ما يلي:

• وضوح الشاشة.

• عدم وجود تلف.

• عدم وجود تسرب (مياه أو غاز).

• عدم وجود اهتزاز أو ضوضاء غير طبيعية.

الخطوة 4: قراءة قيمة العداد

• العداد الرقمي: سجّل القيمة كاملة (بما في ذلك الكسور العشرية عند الحاجة).

• العداد التناظري: سجّل جميع الأرقام من اليسار إلى اليمين؛ تجاهل القرص الأحمر/الدوار ما لم يُطلب ذلك.

الخطوة 5: التسجيل الفوري (إلزامي)

• سجّل القراءة فورًا في الموقع.

• لا يُسمح بالاعتماد على الذاكرة.

الخطوة 6: نموذج التسجيل

• التاريخ.

• الوقت.

• رقم تعريف العداد.

• المورد.

• المنطقة.

• القراءة السابقة.

• القراءة الحالية.

• الوحدة.

• المشغل.

• الملاحظات.

الخطوة 7: التحقق في الموقع

يجب على المشغل التحقق مما يلي:

• القراءة الحالية ≥ القراءة السابقة.

• زيادة منطقية.

• عدم وجود قفزة غير طبيعية.

في حال وجود حالة غير طبيعية، تُدوَّن في خانة الملاحظات.

10. تعليمات خاصة بكل مورد

10.1 المياه

• الوحدة: م³.

• التحقق من عدم وجود تسرب قريب.

• التأكد من الزيادة المستمرة.

10.2 الكهرباء

• الوحدة: كيلوواط ساعة.

• التحقق من ثبات الشاشة.

• عدم وجود رموز خطأ.

10.3 الغاز الطبيعي (خطورة عالية)

• فحوصات سلامة إضافية: عدم وجود رائحة غاز؛ عدم وجود صوت تسرب؛ ضغط مستقر.

• قواعد القراءة: الوحدة م³ أو م³ قياسي (Nm³)؛ تسجيل القيمة الدقيقة (دون تقريب).

11. ضوابط منع الأخطاء

يجب على المشغلين تجنب ما يلي:

• تخطي العدادات.

• تقدير القيم.

• التسجيل من الذاكرة.

• إدخال وحدة خاطئة.

• الخطأ في ترتيب الأرقام.

12. بروتوكول القراءات غير الطبيعية

12.1 الحالات المستوجبة

• زيادة مفاجئة (>10%).

• عدم وجود تغيير.

• انخفاض غير متوقع.

12.2 الإجراءات المطلوبة

• إعادة التحقق من القراءة.

• تأكيد رقم تعريف العداد.

• إبلاغ المشرف فورًا.

• التدوين في خانة الملاحظات.

• عدم تعديل البيانات مطلقًا.

13. ضبط السجلات وسلامتها

• لا يُسمح بالكتابة فوق القيم.

• يجب أن تُشطب التصحيحات بخط واحد وأن يوقع عليها المشغل والمشرف.

• يجب أن تكون السجلات واضحة وكاملة وقابلة للتتبع.

14. متطلبات التوثيق

يُحتفظ بما يلي:

• سجلات القراءة اليومية.

• سجلات التحقق الموقعة.

• سجل العدادات.

• الأدلة المصورة (اختيارية ولكن يوصى بها).

15. متطلبات التدقيق الداخلي

يتحقق المدققون مما يلي:

• القراءات اليومية دون انقطاع.

• ثبات وقت القراءة.

• الاتساق.

• إدراج قراءات الغاز.

• إمكانية التتبع إلى العدادات الفعلية.

16. متطلبات التدريب

يجب تدريب جميع الموظفين على:

• أنواع العدادات ووحداتها.

• دقة القراءة.

• السلامة (الغاز والكهرباء).

• منع الأخطاء.

17. إدارة حالات عدم المطابقة

• قراءة مفقودة: تصحيح فوري.

• بيانات خاطئة: إعادة التدريب.

• قراءة غير آمنة: إيقاف العمل فورًا.

ONAYLAYANLAR:

| Strateji ve Süreç Geliştirme Müdürü BAHAR YILDIRIM |  | İnsan Kaynakları Grup Müdürü ALİ KAAN GENÇ |  | Kalite Güvence Müdürü ABDULLAH BAYDOĞAN |  | EMS Genel Müdürü FATİH SALBARS |
| --- | --- | --- | --- | --- | --- | --- |