1. PURPOSE

The purpose of this procedure is to establish safe and environmentally responsible requirements for the receipt, storage, segregation, transfer, handling, use and emergency control of chemicals used within the facility.

This procedure aims to:

• Prevent chemical spills, leaks and uncontrolled releases.

• Prevent incompatible chemical reactions.

• Protect workers from chemical exposure.

• Prevent contamination of soil, stormwater and wastewater systems.

• Ensure chemical traceability and inventory control.

• Ensure compliance with applicable legal, customer and chemical management requirements.

• Support implementation of the ZDHC Chemical Management System (CMS), ISO 14001 and ISO 45001 requirements.

2. SCOPE

This procedure applies to all chemicals received, stored, transferred, handled or used within the facility, including chemicals used in:

• Knitting operations.

• Dyeing operations.

• Washing and pretreatment.

• Desizing and scouring.

• Bleaching.

• Finishing operations.

• Printing operations.

• T-shirt and garment production.

• Maintenance activities.

• Boiler and utility operations.

• Water treatment and wastewater treatment.

• Cleaning and housekeeping activities.

• Laboratories and quality control activities.

The procedure applies to employees, chemical warehouse personnel, production workers, maintenance personnel, contractors and chemical suppliers entering the facility.

3. RESPONSIBILITIES

3.1 Sustainability / HSE / Chemical Management Department

The department shall:

• Establish and maintain the Chemical Inventory List (CIL).

• Verify availability of current Safety Data Sheets (SDS).

• Review chemical hazards and storage compatibility.

• Conduct chemical risk assessments.

• Verify chemical compliance with applicable MRSL and customer requirements.

• Conduct chemical storage and handling inspections.

• Provide chemical safety and spill response training.

• Investigate chemical spills and chemical-related incidents.

• Monitor corrective and preventive actions.

3.2 Chemical Warehouse Responsible Person

The warehouse responsible person shall:

• Receive chemicals only after authorization.

• Verify chemical name, supplier, quantity, batch/LOT number and container condition.

• Maintain chemical receipt and issuance records.

• Apply FIFO or FEFO, where applicable.

• Maintain chemical segregation according to compatibility.

• Ensure containers remain properly labelled.

• Immediately report leaks, damaged containers or abnormal conditions.

• Prevent unauthorized access to chemical storage areas.

3.3 Production Department

Production supervisors shall:

• Ensure chemicals are used only for approved processes.

• Ensure workers follow SDS and operating instructions.

• Ensure required PPE is used.

• Prevent excessive chemical quantities from being stored in production areas.

• Ensure chemical containers are closed after use.

• Report spills, leaks and unsafe chemical conditions immediately.

3.4 Employees and Chemical Handlers

Employees shall:

• Follow chemical handling instructions and training.

• Wear required PPE.

• Read and understand chemical labels and hazard pictograms.

• Never mix chemicals unless required by an approved process instruction.

• Never use unlabelled chemical containers.

• Immediately report chemical spills, exposure or damaged containers.

4. CHEMICAL APPROVAL BEFORE PURCHASE

No new chemicals shall be purchased or introduced into the facility without chemical approval.

Before purchase, the following documents and information shall be reviewed:

• Current Safety Data Sheet (SDS).

• Technical Data Sheet (TDS), where applicable.

• Chemical composition and hazard classification.

• ZDHC MRSL conformance information, where applicable.

• Customer or brand restricted substance requirements.

• Applicable certificates or chemical compliance declarations.

• Intended process and area of use.

Restricted or prohibited chemicals shall not be purchased or used.

The approved chemical shall be added to the Chemical Inventory List before routine use.

5. CHEMICAL RECEIVING

Upon chemical delivery, the warehouse responsible person shall inspect the shipment.

The following shall be verified:

• Chemical trade name.

• Manufacturer or supplier name.

• Quantity received.

• Batch or LOT number.

• Manufacturing and expiry date, where applicable.

• Container integrity.

• Chemical label.

• Hazard pictograms.

• Purchase order and delivery documents.

Damaged, leaking, swollen, corroded or improperly labelled containers shall not be accepted into the normal chemical storage area.

Such containers shall be isolated in a designated controlled area and reported immediately to the HSE/Chemical Management Department.

Chemical receipt records shall be maintained for traceability.

6. CHEMICAL LABELLING

All chemical containers shall be clearly labelled.

The label shall include, as applicable:

• Chemical or product name.

• Hazard pictogram.

• Hazard information.

• Supplier or manufacturer information.

• Batch/LOT number.

• Relevant handling precautions.

Labels shall be available in a language understood by workers. Arabic and English labelling should be provided where necessary.

Chemicals transferred into secondary containers shall be immediately labelled.

Food bottles, drinking water bottles or beverage containers shall never be used for chemical storage.

Unlabelled chemicals are strictly prohibited.

7. CHEMICAL STORAGE REQUIREMENTS

Chemical storage areas shall be:

• Dedicated to chemical storage.

• Secured against unauthorized access.

• Clearly identified with chemical hazard signs.

• Adequately ventilated.

• Protected from excessive heat and direct sunlight.

• Provided with suitable lighting.

• Constructed with chemical-resistant flooring.

• Free from uncontrolled floor drains.

• Maintained in a clean and organized condition.

• Provided with suitable fire protection equipment.

• Provided with appropriate spill kits.

• Provided with emergency eyewash and safety shower facilities where chemical exposure risks exist.

Chemicals shall not be stored directly on the floor.

Chemical containers shall be stored on suitable chemical-resistant pallets, racks or shelving.

Wooden pallets shall be avoided for liquid chemical storage where chemical absorption and contamination may occur.

Storage racks shall be stable and suitable for the chemical load.

Heavy chemical containers shall be stored at lower levels.

Containers shall remain closed when not in use.

8. SECONDARY CONTAINMENT

Liquid chemicals shall be stored within suitable secondary containment.

Secondary containment shall:

• Be chemically compatible with the stored chemical.

• Be resistant to corrosion.

• Be maintained without cracks or damage.

• Be kept free from rainwater, waste and unrelated materials.

Chemicals with incompatible properties shall not share the same secondary containment system.

Spilled chemicals collected in secondary containment shall not be returned to the original container unless technically assessed and authorized.

Contaminated spill material shall be managed according to the hazardous waste management procedure.

9. CHEMICAL SEGREGATION AND COMPATIBILITY

Chemicals shall be segregated according to their hazard and chemical compatibility.

The following chemical groups shall be stored separately where applicable:

• Acids.

• Alkalis/bases.

• Oxidizing agents.

• Reducing agents.

• Flammable chemicals.

• Reactive chemicals.

• Toxic chemicals.

• Corrosive chemicals.

Acids shall be separated from alkalis.

Oxidizing agents shall be separated from flammable, combustible and reducing materials.

Sodium hypochlorite shall be separated from acids and ammonia-containing chemicals.

Hydrogen peroxide and other oxidizing agents shall be stored away from incompatible organic and reducing materials.

Chemical compatibility shall be determined using the SDS, particularly Sections 7 and 10, and the approved Chemical Compatibility Matrix.

Where chemical compatibility is uncertain, the HSE/Chemical Management Department shall be consulted before storage.

10. CHEMICAL STORAGE IN PRODUCTION AREAS

Bulk or excessive chemical quantities shall not be stored in production areas.

Only the minimum quantity required for the production process or work shift shall be maintained at the point of use.

Chemicals in production areas shall:

• Be kept in designated locations.

• Have proper labels.

• Be stored within secondary containment where leakage is possible.

• Remain closed when not in use.

• Be protected from vehicle and equipment impact.

• Be separated from incompatible chemicals.

Unused chemicals shall be returned to the designated chemical storage area according to the applicable production and warehouse control system.

11. CHEMICAL HANDLING AND TRANSFER

Before handling chemicals, workers shall:

• Review the chemical label.

• Understand the applicable SDS instructions.

• Wear the required PPE.

• Inspect the container for leaks or damage.

• Verify that the receiving container is compatible and correctly labelled.

Chemical transfer shall be performed using suitable equipment such as:

• Chemical-resistant pumps.

• Closed transfer systems.

• Approved dispensing equipment.

Manual pouring shall be minimized, particularly for corrosive or hazardous chemicals.

Workers shall not use their mouths to siphon chemicals.

Chemicals shall not be mixed unless mixing is required by an approved production recipe, process instruction or technical procedure.

Transfer equipment used for incompatible chemicals shall be separated or adequately cleaned according to an approved cleaning procedure.

12. PERSONAL PROTECTIVE EQUIPMENT

Required PPE shall be determined according to the SDS and chemical risk assessment.

PPE may include:

• Chemical-resistant gloves.

• Safety goggles.

• Face shield.

• Chemical-resistant apron.

• Chemical-resistant safety footwear.

• Protective clothing.

• Suitable respiratory protection where required by risk assessment.

PPE shall be inspected before use.

Damaged or contaminated PPE shall be replaced.

Respiratory protection shall only be used according to an established respiratory protection program and appropriate filter selection.

13. CHEMICAL DISPENSING AND DOSING

Automatic or closed chemical dosing systems shall be used where technically practicable.

Chemical dosing equipment shall be:

• Properly identified.

• Maintained in good condition.

• Checked for leakage.

• Calibrated where measurement accuracy is required.

Chemical recipes and dosing instructions shall be authorized.

Unauthorized modification of chemical recipes is prohibited.

Chemical consumption shall be recorded to support inventory reconciliation and chemical traceability.

14. EMPTY CHEMICAL CONTAINERS

Empty chemical containers shall be treated according to their previous chemical contents and applicable waste requirements.

Empty containers shall:

• Be completely drained where safe and applicable.

• Remain labelled or otherwise traceable to their previous contents.

• Be stored in a designated area.

• Be protected from unauthorized reuse.

• Be transferred to an authorized waste or recycling route where applicable.

Chemical containers shall not be used for drinking water, food, domestic purposes or personal use.

IBC tanks, drums and chemical containers shall not be reused for another chemical unless compatibility, cleaning requirements and intended reuse have been technically assessed and formally approved.

15. CHEMICAL SPILL RESPONSE

In the event of a chemical spill:

• Stop work and identify the chemical, if this can be done safely.

• Warn personnel in the affected area.

• Isolate the spill area.

• Notify the supervisor and HSE/Emergency Response Team.

• Wear appropriate PPE.

• Stop the source of leakage only if safe to do so.

• Protect drains and wastewater systems.

• Use suitable absorbent or spill control material.

• Do not mix incompatible spill residues.

• Collect contaminated material in suitable labelled waste containers.

• Clean the affected area according to SDS requirements.

• Report and investigate the spill.

Chemicals shall not be washed into stormwater drains, soil or uncontrolled drainage systems.

Spill kits shall be inspected regularly and replenished after use.

16. EMERGENCY EXPOSURE

• Eye Contact: Immediately flush the eyes using the emergency eyewash station for the period specified in the SDS. Obtain medical attention as required.

• Skin Contact: Remove contaminated clothing and rinse the affected area with water using the emergency safety shower where necessary.

• Inhalation: Move the affected person to fresh air without exposing rescuers to chemical hazards. Obtain medical assistance.

• Ingestion: Do not induce vomiting unless specifically instructed by qualified medical personnel or the SDS.

The applicable SDS shall be made available to medical or emergency personnel.

17. FIRE AND CHEMICAL EMERGENCY

In case of fire involving chemicals:

• Activate the emergency alarm.

• Notify the Emergency Response Team.

• Evacuate according to the emergency plan.

• Identify the chemicals involved where possible.

• Provide relevant SDS information to emergency responders.

• Use only suitable firefighting media specified in the SDS.

Untrained employees shall not attempt to control a chemical fire.

18. CHEMICAL INVENTORY AND TRACEABILITY

The facility shall maintain an updated Chemical Inventory List.

The inventory shall include, as applicable:

• Chemical trade name.

• Manufacturer.

• Supplier.

• Chemical function.

• Area/process of use.

• Monthly or annual consumption.

• Storage location.

• Hazard classification.

• SDS availability.

• ZDHC MRSL conformance status, where applicable.

• Batch/LOT traceability information.

Chemical stock records shall be periodically reconciled with chemical consumption and purchasing records.

Expired, obsolete or unidentified chemicals shall be identified, isolated and appropriately managed.

19. INSPECTION

Chemical storage and handling areas shall be periodically inspected.

Inspection shall cover:

• Chemical labels.

• Container condition.

• Chemical compatibility.

• Secondary containment.

• Chemical leakage.

• Spill kit availability.

• Eyewash and safety shower accessibility.

• Ventilation.

• Fire protection.

• PPE availability.

• Chemical housekeeping.

• Expired chemicals.

• Unauthorized chemicals.

Identified nonconformities shall be recorded and corrective actions followed until closure.

20. TRAINING

Employees handling chemicals shall receive appropriate training before performing chemical handling activities.

Training shall include:

• Chemical hazard awareness.

• GHS hazard pictograms.

• SDS understanding.

• Chemical compatibility.

• Safe chemical storage.

• Chemical handling and transfer.

• Required PPE.

• Chemical spill response.

• Emergency eyewash and safety shower use.

• Chemical waste management.

Refresher training shall be provided periodically and when significant process or chemical changes occur.

21. RECORDS

The following records shall be maintained:

• Chemical Inventory List (CIL).

• Approved Chemical List.

• Safety Data Sheets.

• Chemical receipt records.

• Chemical issuance and consumption records.

• Chemical compatibility matrix.

• Chemical inspection checklists.

• Chemical risk assessments.

• Spill and incident reports.

• Corrective action records.

• Chemical training records.

• Waste transfer and disposal records.

Records shall be retained according to the facility document and record control requirements.

22. REFERENCES

• ZDHC Chemical Management System Framework.

• ZDHC CMS Technical Industry Guide.

• ZDHC Manufacturing Restricted Substances List (MRSL).

• Applicable ZDHC Wastewater Guidelines.

• ISO 14001 Environmental Management Systems.

• ISO 45001 Occupational Health and Safety Management Systems.

• Globally Harmonized System of Classification and Labelling of Chemicals (GHS).

• Applicable Egyptian environmental, occupational health and safety, and waste management legislation.

• Customer and brand chemical requirements.

23. RELATED DOCUMENTS

• Chemical Inventory List (CIL).

• Chemical Approval Procedure.

• Chemical Compatibility Matrix.

• Chemical Risk Assessment.

• Spill Response Procedure.

• Hazardous Waste Management Procedure.

• Emergency Preparedness and Response Procedure.

• PPE Procedure.

• Chemical Warehouse Inspection Checklist.

• Chemical Consumption Log.

1. الغرض

يهدف هذا الإجراء إلى وضع متطلبات آمنة ومسؤولة بيئيًا لاستلام المواد الكيميائية المستخدمة داخل المنشأة وتخزينها وفصلها ونقلها ومناولتها واستخدامها والتحكم فيها في حالات الطوارئ.

ويهدف هذا الإجراء إلى ما يلي:

• منع انسكابات المواد الكيميائية وتسرّباتها وانبعاثاتها غير المنضبطة.

• منع التفاعلات الكيميائية بين المواد غير المتوافقة.

• حماية العاملين من التعرّض للمواد الكيميائية.

• منع تلوث التربة وشبكات مياه الأمطار ومياه الصرف.

• ضمان تتبّع المواد الكيميائية وضبط المخزون.

• ضمان الامتثال للمتطلبات القانونية ومتطلبات العملاء ومتطلبات إدارة المواد الكيميائية المعمول بها.

• دعم تطبيق متطلبات نظام إدارة المواد الكيميائية (CMS) الخاص بـ ZDHC ومتطلبات ISO 14001 وISO 45001.

2. النطاق

ينطبق هذا الإجراء على جميع المواد الكيميائية التي يتم استلامها أو تخزينها أو نقلها أو مناولتها أو استخدامها داخل المنشأة، بما في ذلك المواد الكيميائية المستخدمة في:

• عمليات التريكو.

• عمليات الصباغة.

• الغسيل والمعالجة المسبقة.

• إزالة البوش والتنظيف (الغلي).

• التبييض.

• عمليات التجهيز النهائي.

• عمليات الطباعة.

• إنتاج التيشيرتات والملابس الجاهزة.

• أنشطة الصيانة.

• عمليات الغلايات والمرافق.

• معالجة المياه ومعالجة مياه الصرف.

• أنشطة التنظيف والنظافة العامة.

• المختبرات وأنشطة مراقبة الجودة.

ويسري هذا الإجراء على الموظفين، وأفراد مستودع المواد الكيميائية، وعمال الإنتاج، وأفراد الصيانة، والمقاولين، وموردي المواد الكيميائية الذين يدخلون المنشأة.

3. المسؤوليات

3.1 قسم الاستدامة / الصحة والسلامة والبيئة / إدارة المواد الكيميائية

يتعيّن على القسم ما يلي:

• إعداد قائمة جرد المواد الكيميائية (CIL) والمحافظة عليها.

• التحقق من توفر صحائف بيانات السلامة (SDS) السارية.

• مراجعة مخاطر المواد الكيميائية وتوافقها في التخزين.

• إجراء تقييمات مخاطر المواد الكيميائية.

• التحقق من امتثال المواد الكيميائية لقائمة MRSL المعمول بها ولمتطلبات العملاء.

• إجراء عمليات تفتيش على تخزين المواد الكيميائية ومناولتها.

• تقديم التدريب على السلامة الكيميائية والاستجابة للانسكابات.

• التحقيق في انسكابات المواد الكيميائية والحوادث المتعلقة بها.

• متابعة الإجراءات التصحيحية والوقائية.

3.2 المسؤول عن مستودع المواد الكيميائية

يتعيّن على المسؤول عن المستودع ما يلي:

• عدم استلام المواد الكيميائية إلا بعد الحصول على الموافقة.

• التحقق من اسم المادة الكيميائية والمورد والكمية ورقم الدفعة/التشغيلة (LOT) وحالة العبوة.

• الاحتفاظ بسجلات استلام المواد الكيميائية وصرفها.

• تطبيق مبدأ الوارد أولًا يُصرف أولًا (FIFO) أو ما تنتهي صلاحيته أولًا يُصرف أولًا (FEFO)، حسب الاقتضاء.

• الحفاظ على فصل المواد الكيميائية وفقًا لتوافقها.

• التأكد من بقاء العبوات موسومة بشكل صحيح.

• الإبلاغ فورًا عن التسربات أو العبوات التالفة أو الظروف غير الطبيعية.

• منع الدخول غير المصرّح به إلى مناطق تخزين المواد الكيميائية.

3.3 قسم الإنتاج

يتعيّن على مشرفي الإنتاج ما يلي:

• التأكد من استخدام المواد الكيميائية في العمليات المعتمدة فقط.

• التأكد من التزام العمال بصحائف بيانات السلامة وتعليمات التشغيل.

• التأكد من استخدام معدات الوقاية الشخصية المطلوبة.

• منع تخزين كميات زائدة من المواد الكيميائية في مناطق الإنتاج.

• التأكد من إغلاق عبوات المواد الكيميائية بعد الاستخدام.

• الإبلاغ فورًا عن الانسكابات والتسربات والظروف الكيميائية غير الآمنة.

3.4 الموظفون والقائمون على مناولة المواد الكيميائية

يتعيّن على الموظفين ما يلي:

• اتباع تعليمات مناولة المواد الكيميائية والتدريب المقدَّم.

• ارتداء معدات الوقاية الشخصية المطلوبة.

• قراءة بطاقات المواد الكيميائية ورموز المخاطر التوضيحية وفهمها.

• عدم خلط المواد الكيميائية مطلقًا ما لم تتطلب ذلك تعليمات عملية معتمدة.

• عدم استخدام عبوات مواد كيميائية غير موسومة مطلقًا.

• الإبلاغ فورًا عن انسكابات المواد الكيميائية أو التعرّض لها أو العبوات التالفة.

4. اعتماد المواد الكيميائية قبل الشراء

لا يجوز شراء أي مواد كيميائية جديدة أو إدخالها إلى المنشأة دون اعتمادها.

قبل الشراء، تجب مراجعة المستندات والمعلومات التالية:

• صحيفة بيانات السلامة (SDS) السارية.

• صحيفة البيانات الفنية (TDS)، حسب الاقتضاء.

• التركيب الكيميائي وتصنيف المخاطر.

• معلومات المطابقة لقائمة ZDHC MRSL، حسب الاقتضاء.

• متطلبات العملاء أو العلامات التجارية الخاصة بالمواد المقيّدة.

• الشهادات أو إقرارات الامتثال الكيميائي المعمول بها.

• العملية المقصودة ومنطقة الاستخدام.

لا يجوز شراء المواد الكيميائية المقيّدة أو المحظورة أو استخدامها.

تُضاف المادة الكيميائية المعتمدة إلى قائمة جرد المواد الكيميائية قبل استخدامها بشكل روتيني.

5. استلام المواد الكيميائية

عند توريد المواد الكيميائية، يقوم المسؤول عن المستودع بفحص الشحنة.

ويجب التحقق مما يلي:

• الاسم التجاري للمادة الكيميائية.

• اسم الشركة المصنّعة أو المورد.

• الكمية المستلمة.

• رقم الدفعة أو التشغيلة (LOT).

• تاريخ التصنيع وتاريخ انتهاء الصلاحية، حسب الاقتضاء.

• سلامة العبوة.

• بطاقة المادة الكيميائية.

• رموز المخاطر التوضيحية.

• أمر الشراء ومستندات التسليم.

لا تُقبل العبوات التالفة أو المسرِّبة أو المنتفخة أو المتآكلة أو غير الموسومة بشكل صحيح في منطقة التخزين العادية للمواد الكيميائية.

وتُعزل هذه العبوات في منطقة مخصصة خاضعة للرقابة، ويُبلَّغ عنها فورًا قسم الصحة والسلامة والبيئة/إدارة المواد الكيميائية.

يُحتفظ بسجلات استلام المواد الكيميائية لأغراض التتبّع.

6. وسم المواد الكيميائية

يجب وسم جميع عبوات المواد الكيميائية بوضوح.

ويجب أن تتضمن البطاقة، حسب الاقتضاء، ما يلي:

• اسم المادة الكيميائية أو المنتج.

• رمز الخطر التوضيحي.

• معلومات المخاطر.

• بيانات المورد أو الشركة المصنّعة.

• رقم الدفعة/التشغيلة (LOT).

• احتياطات المناولة ذات الصلة.

يجب أن تكون البطاقات بلغة يفهمها العمال، وينبغي توفير الوسم باللغتين العربية والإنجليزية عند الضرورة.

تُوسم المواد الكيميائية المنقولة إلى عبوات ثانوية فورًا.

لا يجوز مطلقًا استخدام زجاجات الأطعمة أو زجاجات مياه الشرب أو عبوات المشروبات لتخزين المواد الكيميائية.

يُحظر تمامًا وجود مواد كيميائية غير موسومة.

7. متطلبات تخزين المواد الكيميائية

يجب أن تكون مناطق تخزين المواد الكيميائية:

• مخصصة لتخزين المواد الكيميائية.

• مؤمَّنة ضد الدخول غير المصرّح به.

• محددة بوضوح بلافتات المخاطر الكيميائية.

• جيدة التهوية.

• محمية من الحرارة المفرطة وأشعة الشمس المباشرة.

• مزودة بإضاءة مناسبة.

• مجهزة بأرضيات مقاومة للمواد الكيميائية.

• خالية من مصارف أرضية غير خاضعة للتحكم.

• محفوظة في حالة نظيفة ومنظمة.

• مزودة بمعدات مناسبة للحماية من الحريق.

• مزودة بأطقم مناسبة للتعامل مع الانسكابات.

• مزودة بمحطات غسيل العين في حالات الطوارئ ودُش السلامة حيثما توجد مخاطر التعرّض للمواد الكيميائية.

لا يجوز تخزين المواد الكيميائية على الأرض مباشرة.

تُخزَّن عبوات المواد الكيميائية على منصات أو حوامل أو أرفف مناسبة ومقاومة للمواد الكيميائية.

يُتجنَّب استخدام المنصات الخشبية لتخزين المواد الكيميائية السائلة حيث يمكن أن يحدث امتصاص للمواد الكيميائية وتلوث.

يجب أن تكون حوامل التخزين ثابتة ومناسبة لحمولة المواد الكيميائية.

تُخزَّن عبوات المواد الكيميائية الثقيلة في المستويات السفلية.

تبقى العبوات مغلقة عند عدم الاستخدام.

8. الاحتواء الثانوي

تُخزَّن المواد الكيميائية السائلة داخل نظام احتواء ثانوي مناسب.

ويجب أن يكون الاحتواء الثانوي:

• متوافقًا كيميائيًا مع المادة الكيميائية المخزنة.

• مقاومًا للتآكل.

• خاليًا من الشقوق أو التلف.

• خاليًا من مياه الأمطار والنفايات والمواد غير ذات الصلة.

لا يجوز أن تشترك المواد الكيميائية ذات الخصائص غير المتوافقة في نظام الاحتواء الثانوي نفسه.

لا تُعاد المواد الكيميائية المنسكبة والمجمّعة في الاحتواء الثانوي إلى العبوة الأصلية ما لم يتم تقييم ذلك فنيًا والموافقة عليه.

تُدار مواد الانسكاب الملوثة وفقًا لإجراء إدارة النفايات الخطرة.

9. فصل المواد الكيميائية وتوافقها

تُفصل المواد الكيميائية وفقًا لمخاطرها وتوافقها الكيميائي.

وتُخزَّن مجموعات المواد الكيميائية التالية بشكل منفصل، حسب الاقتضاء:

• الأحماض.

• القلويات/القواعد.

• العوامل المؤكسدة.

• العوامل المختزلة.

• المواد الكيميائية القابلة للاشتعال.

• المواد الكيميائية التفاعلية.

• المواد الكيميائية السامة.

• المواد الكيميائية الأكّالة.

تُفصل الأحماض عن القلويات.

تُفصل العوامل المؤكسدة عن المواد القابلة للاشتعال والقابلة للاحتراق والمواد المختزلة.

يُفصل هيبوكلوريت الصوديوم عن الأحماض والمواد الكيميائية المحتوية على الأمونيا.

يُخزَّن بيروكسيد الهيدروجين والعوامل المؤكسدة الأخرى بعيدًا عن المواد العضوية والمختزلة غير المتوافقة.

يُحدَّد التوافق الكيميائي باستخدام صحيفة بيانات السلامة، ولا سيما القسمين 7 و10، ومصفوفة التوافق الكيميائي المعتمدة.

وفي حال عدم التأكد من التوافق الكيميائي، يُستشار قسم الصحة والسلامة والبيئة/إدارة المواد الكيميائية قبل التخزين.

10. تخزين المواد الكيميائية في مناطق الإنتاج

لا يجوز تخزين كميات سائبة أو زائدة من المواد الكيميائية في مناطق الإنتاج.

ولا يُحتفظ في نقطة الاستخدام إلا بالحد الأدنى من الكمية اللازمة لعملية الإنتاج أو لوردية العمل.

ويجب أن تكون المواد الكيميائية في مناطق الإنتاج:

• محفوظة في أماكن مخصصة.

• موسومة ببطاقات صحيحة.

• مخزنة داخل احتواء ثانوي حيثما يكون التسرّب ممكنًا.

• مغلقة عند عدم الاستخدام.

• محمية من اصطدام المركبات والمعدات.

• مفصولة عن المواد الكيميائية غير المتوافقة.

تُعاد المواد الكيميائية غير المستخدمة إلى منطقة تخزين المواد الكيميائية المخصصة وفقًا لنظام الرقابة المعمول به في الإنتاج والمستودع.

11. مناولة المواد الكيميائية ونقلها

قبل مناولة المواد الكيميائية، يتعيّن على العمال ما يلي:

• مراجعة بطاقة المادة الكيميائية.

• فهم تعليمات صحيفة بيانات السلامة ذات الصلة.

• ارتداء معدات الوقاية الشخصية المطلوبة.

• فحص العبوة للتأكد من خلوها من التسرّب أو التلف.

• التحقق من أن العبوة المستقبِلة متوافقة وموسومة بشكل صحيح.

يتم نقل المواد الكيميائية باستخدام معدات مناسبة مثل:

• المضخات المقاومة للمواد الكيميائية.

• أنظمة النقل المغلقة.

• معدات الصرف المعتمدة.

يُقلَّل الصب اليدوي إلى أدنى حد، ولا سيما للمواد الكيميائية الأكّالة أو الخطرة.

لا يجوز للعمال استخدام الفم لسحب المواد الكيميائية بالشفط.

لا يجوز خلط المواد الكيميائية ما لم يكن الخلط مطلوبًا بموجب وصفة إنتاج معتمدة أو تعليمات عملية أو إجراء فني.

تُفصل معدات النقل المستخدمة لمواد كيميائية غير متوافقة أو تُنظَّف تنظيفًا كافيًا وفقًا لإجراء تنظيف معتمد.

12. معدات الوقاية الشخصية

تُحدَّد معدات الوقاية الشخصية المطلوبة وفقًا لصحيفة بيانات السلامة وتقييم المخاطر الكيميائية.

وقد تشمل معدات الوقاية الشخصية ما يلي:

• قفازات مقاومة للمواد الكيميائية.

• نظارات السلامة الواقية.

• واقي الوجه.

• مريلة مقاومة للمواد الكيميائية.

• أحذية سلامة مقاومة للمواد الكيميائية.

• ملابس واقية.

• وسائل حماية تنفسية مناسبة حيثما يتطلب تقييم المخاطر ذلك.

تُفحص معدات الوقاية الشخصية قبل الاستخدام.

وتُستبدل معدات الوقاية الشخصية التالفة أو الملوثة.

لا تُستخدم وسائل الحماية التنفسية إلا وفقًا لبرنامج معتمد للحماية التنفسية مع اختيار المرشِّحات (الفلاتر) المناسبة.

13. صرف المواد الكيميائية وجرعاتها

تُستخدم أنظمة جرعات المواد الكيميائية الآلية أو المغلقة حيثما كان ذلك ممكنًا من الناحية الفنية.

ويجب أن تكون معدات جرعات المواد الكيميائية:

• محددة بشكل صحيح.

• محفوظة في حالة جيدة.

• مفحوصة للتأكد من عدم وجود تسرّب.

• معايَرة حيثما تكون دقة القياس مطلوبة.

يجب اعتماد الوصفات الكيميائية وتعليمات الجرعات.

ويُحظر تعديل الوصفات الكيميائية دون تصريح.

يُسجَّل استهلاك المواد الكيميائية لدعم مطابقة المخزون وتتبّع المواد الكيميائية.

14. عبوات المواد الكيميائية الفارغة

تُعامَل عبوات المواد الكيميائية الفارغة وفقًا لمحتوياتها الكيميائية السابقة ومتطلبات النفايات المعمول بها.

ويجب أن تكون العبوات الفارغة:

• مفرَّغة تمامًا حيثما كان ذلك آمنًا وممكنًا.

• موسومة أو قابلة للتتبّع بطريقة أخرى إلى محتوياتها السابقة.

• مخزنة في منطقة مخصصة.

• محمية من إعادة الاستخدام غير المصرّح به.

• منقولة عبر مسار معتمد للتخلص من النفايات أو لإعادة التدوير، حسب الاقتضاء.

لا يجوز استخدام عبوات المواد الكيميائية لمياه الشرب أو الطعام أو الأغراض المنزلية أو الاستخدام الشخصي.

لا يجوز إعادة استخدام خزانات IBC والبراميل وعبوات المواد الكيميائية لمادة كيميائية أخرى ما لم يتم تقييم التوافق ومتطلبات التنظيف والاستخدام المقصود فنيًا واعتمادها رسميًا.

15. الاستجابة لانسكاب المواد الكيميائية

في حالة انسكاب مادة كيميائية:

• أوقف العمل وحدِّد المادة الكيميائية إذا أمكن ذلك بأمان.

• نبِّه الأفراد في المنطقة المتأثرة.

• اعزل منطقة الانسكاب.

• أبلغ المشرف وفريق الصحة والسلامة والبيئة/فريق الاستجابة للطوارئ.

• ارتدِ معدات الوقاية الشخصية المناسبة.

• أوقف مصدر التسرّب فقط إذا كان ذلك آمنًا.

• احمِ المصارف وشبكات مياه الصرف.

• استخدم مواد ماصة أو مواد مناسبة للسيطرة على الانسكاب.

• لا تخلط بقايا الانسكابات غير المتوافقة.

• اجمع المواد الملوثة في حاويات نفايات مناسبة وموسومة.

• نظِّف المنطقة المتأثرة وفقًا لمتطلبات صحيفة بيانات السلامة.

• أبلغ عن الانسكاب وحقِّق فيه.

لا يجوز غسل المواد الكيميائية إلى مصارف مياه الأمطار أو التربة أو شبكات الصرف غير الخاضعة للتحكم.

تُفحص أطقم الانسكاب بانتظام ويُعاد تزويدها بعد الاستخدام.

16. التعرّض في حالات الطوارئ

• ملامسة العين: اغسل العينين فورًا باستخدام محطة غسيل العين للطوارئ للمدة المحددة في صحيفة بيانات السلامة، واحصل على الرعاية الطبية حسب الحاجة.

• ملامسة الجلد: انزع الملابس الملوثة واشطف المنطقة المصابة بالماء باستخدام دُش السلامة للطوارئ عند الضرورة.

• الاستنشاق: انقل الشخص المصاب إلى الهواء الطلق دون تعريض المنقذين للمخاطر الكيميائية، واحصل على المساعدة الطبية.

• الابتلاع: لا تحفّز القيء ما لم يُطلب ذلك صراحةً من الطاقم الطبي المؤهل أو وفقًا لصحيفة بيانات السلامة.

تُتاح صحيفة بيانات السلامة ذات الصلة للطاقم الطبي أو لأفراد الطوارئ.

17. الحريق والطوارئ الكيميائية

في حالة نشوب حريق يتضمن مواد كيميائية:

• فعِّل إنذار الطوارئ.

• أبلغ فريق الاستجابة للطوارئ.

• قم بالإخلاء وفقًا لخطة الطوارئ.

• حدِّد المواد الكيميائية المعنية حيثما أمكن.

• قدِّم معلومات صحيفة بيانات السلامة ذات الصلة إلى المستجيبين للطوارئ.

• استخدم فقط وسائل الإطفاء المناسبة المحددة في صحيفة بيانات السلامة.

لا يجوز للموظفين غير المدربين محاولة السيطرة على حريق كيميائي.

18. جرد المواد الكيميائية وتتبّعها

تحتفظ المنشأة بقائمة محدَّثة لجرد المواد الكيميائية.

ويجب أن تتضمن القائمة، حسب الاقتضاء، ما يلي:

• الاسم التجاري للمادة الكيميائية.

• الشركة المصنّعة.

• المورد.

• وظيفة المادة الكيميائية.

• منطقة/عملية الاستخدام.

• الاستهلاك الشهري أو السنوي.

• موقع التخزين.

• تصنيف المخاطر.

• توفر صحيفة بيانات السلامة.

• حالة المطابقة لقائمة ZDHC MRSL، حسب الاقتضاء.

• معلومات تتبّع الدفعة/التشغيلة (LOT).

تُطابَق سجلات مخزون المواد الكيميائية دوريًا مع سجلات الاستهلاك والمشتريات.

تُحدَّد المواد الكيميائية منتهية الصلاحية أو المتقادمة أو مجهولة الهوية وتُعزل وتُدار بالشكل المناسب.

19. التفتيش

يتم التفتيش دوريًا على مناطق تخزين المواد الكيميائية ومناولتها.

ويشمل التفتيش ما يلي:

• بطاقات المواد الكيميائية.

• حالة العبوات.

• التوافق الكيميائي.

• الاحتواء الثانوي.

• تسرّب المواد الكيميائية.

• توفر أطقم الانسكاب.

• سهولة الوصول إلى محطات غسيل العين ودُش السلامة.

• التهوية.

• الحماية من الحريق.

• توفر معدات الوقاية الشخصية.

• النظافة والترتيب في مناطق المواد الكيميائية.

• المواد الكيميائية منتهية الصلاحية.

• المواد الكيميائية غير المصرّح بها.

تُسجَّل حالات عدم المطابقة المكتشفة وتُتابع الإجراءات التصحيحية حتى إغلاقها.

20. التدريب

يتلقى الموظفون الذين يتعاملون مع المواد الكيميائية تدريبًا مناسبًا قبل أداء أنشطة مناولة المواد الكيميائية.

ويشمل التدريب ما يلي:

• التوعية بالمخاطر الكيميائية.

• رموز المخاطر التوضيحية وفق النظام المنسق عالميًا (GHS).

• فهم صحائف بيانات السلامة.

• التوافق الكيميائي.

• التخزين الآمن للمواد الكيميائية.

• مناولة المواد الكيميائية ونقلها.

• معدات الوقاية الشخصية المطلوبة.

• الاستجابة لانسكاب المواد الكيميائية.

• استخدام محطات غسيل العين ودُش السلامة في حالات الطوارئ.

• إدارة النفايات الكيميائية.

يُقدَّم تدريب تنشيطي بشكل دوري وعند حدوث تغييرات جوهرية في العمليات أو المواد الكيميائية.

21. السجلات

يُحتفظ بالسجلات التالية:

• قائمة جرد المواد الكيميائية (CIL).

• قائمة المواد الكيميائية المعتمدة.

• صحائف بيانات السلامة.

• سجلات استلام المواد الكيميائية.

• سجلات صرف المواد الكيميائية واستهلاكها.

• مصفوفة التوافق الكيميائي.

• قوائم فحص المواد الكيميائية.

• تقييمات مخاطر المواد الكيميائية.

• تقارير الانسكابات والحوادث.

• سجلات الإجراءات التصحيحية.

• سجلات التدريب على المواد الكيميائية.

• سجلات نقل النفايات والتخلص منها.

يُحتفظ بالسجلات وفقًا لمتطلبات المنشأة الخاصة بضبط الوثائق والسجلات.

22. المراجع

• إطار نظام إدارة المواد الكيميائية الخاص بـ ZDHC.

• الدليل الفني الصناعي لنظام إدارة المواد الكيميائية (CMS) الخاص بـ ZDHC.

• قائمة المواد المقيّدة في التصنيع (MRSL) الخاصة بـ ZDHC.

• إرشادات ZDHC لمياه الصرف المعمول بها.

• ISO 14001 أنظمة الإدارة البيئية.

• ISO 45001 أنظمة إدارة الصحة والسلامة المهنية.

• النظام المنسق عالميًا لتصنيف المواد الكيميائية ووسمها (GHS).

• التشريعات المصرية المعمول بها في مجالات البيئة والصحة والسلامة المهنية وإدارة النفايات.

• متطلبات العملاء والعلامات التجارية الخاصة بالمواد الكيميائية.

23. الوثائق ذات الصلة

• قائمة جرد المواد الكيميائية (CIL).

• إجراء اعتماد المواد الكيميائية.

• مصفوفة التوافق الكيميائي.

• تقييم مخاطر المواد الكيميائية.

• إجراء الاستجابة للانسكابات.

• إجراء إدارة النفايات الخطرة.

• إجراء الاستعداد والاستجابة للطوارئ.

• إجراء معدات الوقاية الشخصية.

• قائمة فحص مستودع المواد الكيميائية.

• سجل استهلاك المواد الكيميائية.

ONAYLAYANLAR:

| Strateji ve Süreç Geliştirme Müdürü BAHAR YILDIRIM |  | İnsan Kaynakları Grup Müdürü ALİ KAAN GENÇ |  | Kalite Güvence Müdürü ABDULLAH BAYDOĞAN |  | EMS Genel Müdürü FATİH SALBARS |
| --- | --- | --- | --- | --- | --- | --- |